3GPP TSG-RAN4 Meeting #106	R4-2301470

# Athens, Greece, 27th Feb 2023 – 3rd March 2023

Source:			Ericsson

Title:			5G terrestrial broadcast – Bands discussion

Agenda item:		10.3.2

Document for:		Approval

# Introduction

In the scope of Release 18, a new WI on New bands and BW allocation for 5G terrestrial broadcast [1] has been approved by RAN.

One objective is to specify band(s), within the portion of UHF spectrum allocated to broadcast, that support the channelization available to broadcasting operators.

This contribution is based on our RAN4#105 contribution [3], further discussing this aspect.

# Discussion

## Background

In last RAN4#105 meeting, RAN4 discussed further on the band(s) definition for 5G broadcast with an agreed way Forward ([2]) mentioning the following:

## Single band 470-698/702 MHz consideration

In our last RAN4#105 contribution ([3]), we expressed our concerns specifying a band covering the full range 470-698/702 MHz. With ~40% relative bandwidth, filter design with good performance is challenging.

One filter we found has ~1.5dB typical insertion loss in 600-710 MHz and ~3dB typical (4dB max.) in 710-770 MHz. Also, it has ~33dB typical attenuation but for the 880-915 MHz frequency range. For lower frequency range, its attenuation is much lower, as it could be seen in Figure 1.

Figure 1: TDK DEA160710LT filter: insertion Loss and attenuation

This filter doesn’t have a good enough roll off to give a relevant protection from the adjacent bands. If we define a full band 470-694-/698/702 MHz band, the UE blocking requirement would have to be considerably relaxed, not given much protection from the adjacent bands. All broadcast channels in the upper part might suffer from interference from the 700MHz bands (n12, n28, …) in regions where they are deployed.

Also, in regions where n71 (US) or n105 (APT) are operated, those bands would be inside this large broadcast band and the broadcast channels in the 500MHz would not have any blocking protection (out of band).

Such large band specified based on this type of filter might hardly be deployed in many areas.

Moreover, whatever band(s) RAN4 would define, the requirements shall at least be compliant with existing standards (i.e. EN 303 340 [4]) and most likely anticipate the ECC Recommendation on “Receiver resilience to transmission on adjacent frequency ranges” ([5]), which is supposed to be released by October 2023 (mature drafts are available on ECC website)

Observation: RAN4 should not specify broadcast band(s) with too relaxed requirements, not compliant with EN 303 340 and ECC Recommendation on “Receiver resilience to transmission on adjacent frequency ranges”.

## Alternative proposal

In last RAN4#105 meeting, we proposed ([3]) to define 3 bands covering the 470-702 MHz frequency range, with some overlapping between those 3 bands to support any broadcast channel whatever the channel bandwidth is (6, 7 or 8 MHz). Those 3 bands are: 470-542 MHz, 540-606 MHz and 602-702 MHz.

Nevertheless, it must be noted that no such filter exists today. Without any simulation results from filter providers, it might be difficult for RAN4 to specify the corresponding UE requirements.

We understand this might require some effort as 3 types of filters would need to be simulated. If so, in case RAN4 can’t get any such simulation results, to minimize the simulations effort, our alternative proposal would be to specify the following 4 sub-bands (as shown on Figure 2):

- Sub-band 1: 470-617 MHz sub-band using a similar low pass filter than above but downshifted. It will have steeper roll off characteristics and would be able to reject n105 UL blockers.

- Sub-band 2: 612-652 MHz sub-band using the n105 UL filter.

- Sub-band 3: 646-686 MHz assuming re-using the n105 UL filter.

- Sub-band 4: 658-698 MHz (or 662-702 MHz) assuming re-using the n105 filter as well.

Figure 2: Sub-bands proposal

Thanks to the overlapping of those sub-bands, the 3 considered channel bandwidth (6, 7 and 8 MHz) could be handled properly, all broadcast channels could be fully supported by at least one of those sub-bands.

RAN4 would only need some filter simulations to finalize the requirements of the first sub-band 1 (470-617 MHz) while, for the other sub-bands 2, 3 and 4, requirements could be set based on the ones already specified for band n105.

The sub-bands would not suffer of coexistence with the 700MHz bands.

Proposal: If the initial proposal of 3 sub-bands (470-542 MHz, 540-606 MHz and 602-702 MHz) is not acceptable due to the effort and lack of simulation results, specify 4 sub-bands for 5G broadcast service: 470-617 MHz, 612-652 MHz, 646-686MHz and 658-698MHz. The requirements for the 3 last sub-bands (612-652 MHz, 646-686MHz and 658-698MHz) considering a n105 UL filter.

# Conclusion

In this contribution, we further discuss the bands plan for 5G terrestrial broadcast and made the following observation and proposal:

Observation RAN4 should not specify broadcast band(s) with too relaxed requirements, not compliant with EN 303 340 and ECC Recommendation on “Receiver resilience to transmission on adjacent frequency ranges”.

Proposal: If the initial proposal of 3 sub-bands (470-542 MHz, 540-606 MHz and 602-702 MHz) is not acceptable due to the effort and lack of simulation results, specify 4 sub-bands for 5G broadcast service: 470-617 MHz, 612-652 MHz, 646-686MHz and 658-698MHz. The requirements for the 3 last sub-bands (612-652 MHz, 646-686MHz and 658-698MHz) considering a n105 UL filter.

# References

- RP-222224, Revised WID on New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2, Qualcomm

- R4-2220501, WF on UE RF for 5G Broadcast, Qualcomm

- R4-2218493, 5G terrestrial broadcast – Bands discussion, Ericsson

- EN 303 340, Digital Terrestrial TV Broadcast Receivers; Harmonised Standard for access to radio spectrum, v1.2.1

- ECC Recommendation on Receiver resilience to transmission on adjacent frequency ranges, SE21_24 work item, planned to be released Oct. 2023.