# 3GPP TSG-RAN WG4 Meeting #106	R4-2301531

Athens, Greece, February 27 – March 3, 2023

Source: 	vivo

Title: 	Further discussion on feasibility aspects and RF requirements impact for CA_n5-n8

Agenda Item:	9.2.2.1

Document for:	Approval

Introduction

In the last meeting, we further discussed the feasibility aspects and RF requirements impacts for handheld UE supporting CA_n5-n8. Two-antenna architecture was agreed as baseline and three-antenna architecture can also be analysed [1] [2]. For full band RF filters, the overlapping issue of n5 DL and n8 UL needs further study in this meeting. For dedicated RF filters, whether to define new bands needs further confirmation. In this contribution, remaining issues for CA_n5-n8 are further discussed.

Discussion

Although frequency band usage restriction for CA_n5-n8 was clarified, full n5 and n8 RF filters were agreed as a baseline. The issue n5 DL overlapping with n8 UL still needs to be addressed. The WF of this was captured as follows:

Companies to propose which of the methods is used to manage the overlap of n5 DL and n8 UL

- Option 1: Restricting UL support to n5 UL only

- Option 2: 2UL but Non-concurrent operation between n5 DL and n8 UL

  - NOTE: Check if this kind of configuration is allowed according to current RAN2 specifications

Option 1 is straightforward by restricting to n5 UL only. However, the key intention of this SI is to combine low bands by CA to achieve the high data rate and good coverage at the same time. The objective is to investigate simultaneous transmission on two UL bands and simultaneous reception on two or three bands for the band combination of 700, 800 and 900MHz spectrum for smart phone form factor. In the band combinations request, uplink configuration CA_n5-n8 was brought up by operators. Option 1 restricts the scope of the objective to the transmission on only band n5. We suggest to check with operators if Option 1 is OK for them.

Summary of band combinations considered in the SI

| Configuration | Uplink configuration | Supported operators |
| --- | --- | --- |
| CA_n8-n20-n28 | CA_n8-n20, CA_n8-n28, CA_n20-n28 | Vodafone, Telecom Italia, Orange, Deutsche Telekom |
| CA_n5-n8 | CA_n5-n8 | China Telecom, Spark NZ, China Unicom |
| CA_n5-n28 | CA_n5-n28 | Spark NZ |

Proposal 1: Check with the operators if uplink configuration restricting UL support to n5 UL only is OK for CA_n5-n8.

For Option 2, two UL band transmissions can be achieved by adding a RF switch between n5DL and n8 UL. It needs further checking if this kind of configuration is allowed according to current RAN2 specifications. The CA configurations can be obtained for Option 2:

| CA Band combination | Uplink configuration | Downlink configuration |
| --- | --- | --- |
| CA_n5-n8 | CA_n5-n8 | Restrict to n8 DL |
|  | Restrict to n5 UL | CA_n5-n8 |

When UL with two bands transmission, the DL should be restricted to n8; When DL with two bands reception, the UL should be restricted to n5. From our understanding, gNB scheduling can enable 2UL but non-concurrent operation between n5 DL and n8 UL from implementation point of view. As for whether this kind of configuration is allowed according to current RAN2 specification, we think current CA configuration can not enable Option 2 after we checked TS 38.331. Current band combination configuration contains general band list information, ca-parametersNR… and we think these information elements can not support the configuration of Option 2. An LS can be sent to RAN2 colleagues to confirm this understanding.

Observation 1: From implementation point of view, gNB scheduling can enable 2UL but non-concurrent operation between n5 DL and n8 UL.

Besides full band RF filters, dedicated RF filters can also be analyzed. Whether new bands would be required needs further confirmation in RAN4.

Way forward:

Study the conditions under which requirements could be specified using dedicated RF filters

- Option 1: New bands would be required

- Option 2: New bands would not be required

- Option 3: Other

From our perspective, full/dedicated band filter architectures are from implementation point view, and it doesn’t necessarily mean new bands would be required. Actually, we prefer not to define new bands even dedicated RF filters. If we define two new band based on the subset of n5 and n8, new RF requirements need to be defined for these two new bands. Also, RF requirements need to be defined for this CA band combination of these two new bands. It required too much specification work for introducing new bands. Thus, Option 2 is preferred.

Proposal 2: New bands would not be required for CA_n5-n8 using dedicated RF filters.

Conclusion

This contribution further discusses the methods managing the overlap of n5 DL and n8 UL with full band RF filters, and whether to define new band for dedicated RF filters. The following observation and proposals are made:

Proposal 1: Check with the operators if uplink configuration restricting UL support to n5 UL only is OK for CA_n5-n8.

Observation 1: The CA configuration of 2UL but non-concurrent operation between n5 DL and n8 UL is allowed from specification point of view.

Proposal 2: New bands would not be required for CA_n5-n8 using dedicated RF filters.

- References

[1] R4-2220507, WF on UE architecture and other aspects for CA_n5-n8, Qualcomm, RAN4#105.

[2] R4-2220504, TP for TR 38.872 on CA_n5-n8 architectures, Apple, RAN4#105.