3GPP TSG-RAN WG4 Meeting # 106	R4-2301543

# Athens, Greece, Feb. 27th – Mar. 3rd, 2023

Source: 	vivo

Title: 	Discussion on enhancements of 3Tx for band combinations with two bands

Agenda Item:	8.29.3

Document for:	Discussion

Introduction

Currently, the UE 3Tx for inter-band UL CA and EN-DC feature had been introduced in RAN#98-e in [1]. the WID

4Rx feature for low frequency band is only defined for FWA form factor for NR. A WI for Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC has been approved in RAN#98-e in [1] in which the following objectives are introduced:

Enhancements of 3Tx for band combinations with two bands

  - Two bands with inter-band UL CA or EN-DC are considered with below limitations:

    - In each band only 1CC included. The Tx capability considered is 1Tx in one band, and 2Tx in the other band

    - The following power capabilities will be considered

      - CA power class or EN-DC power class is PC2

        - PC3 FDD band 1Tx + PC2 TDD band 2Tx (UL MIMO and TxD)

        - PC3 FDD band 1Tx + PC3 TDD band 2Tx (UL MIMO)

        - PC3 TDD band 1Tx + PC2 TDD band 2Tx (UL MIMO)

      - CA power class or EN-DC power class is PC1.5

        - PC3 FDD band 1Tx + PC1.5 TDD band 2Tx (UL MIMO and TxD)

  - Targeting UE type:

    - Specify UE RF requirements for FWA.

    - Study the applicable requirements for handheld UE but no normative work in Rel-18

  - Specify requirements for 3Tx, e.g. clarify the applicable requirements for the band which support UL MIMO in inter-band UL CA or inter-band EN-DC

    - Note 1: Increase UE power high limit feature is not included

  - Frequency band combinations considered in this WI is in table 4.1-2 with the contact information inside.

In this paper, some general guidelines and requirements analysis are provided for UE with 3Tx for inter-band UL CA and EN-DC.

Discussion

Current requirements

There was a view in that current inter-band UL CA and ENDC requirements are all applicable to UE with 1Tx+2Tx inter-band UL CA as in the WF [2]:

Issue 4: Whether the “1Tx+2Tx” can be consider as a UE architecture in this WI to support inter-band UL CA/DC

- Some components think that simultaneous turn on 3Tx for 2 bands inter-band CA/DC (1 CC/band) can be covered by the current specification, while other companies think that this is a new feature.

- Whether supporting 3Tx for 2 bands inter-band CA/DC (1 CC/band) need to be captured in Rel-18 UE FR1 WI or a new WI is up to RAN decision.

However, it is found that many requirements clearly not applicable to 1Tx + 2Tx case. E.g. for the simultaneous uplink in 2 bands case Inter-band UL CA MPR requirements in clause 6.2A.2.3 of 38.101-1, there is following requirements:

“For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.2 apply for each uplink component carrier. ”

It should be noted that the requirements in clause 6.2.2 are defined specifically for one antenna connector case, and the 2Tx requirements were defined in following clauses:

6.2D.2	UE maximum output power reduction for UL MIMO

6.2G.2	UE maximum output power reduction for Tx Diversity

So, it is quite clear that only 1Tx is considered in the current requirements for this inter-band UL CA MPR requirements case, and there are also many similar cases in other requirements. More comprehensive summary tables were attached in the Annex for reference

Observation 1: Many current RAN4 requirements for inter-band UL CA and EN-DC explicitly requires 1Tx antenna connector per carrier.

This principle can be further confirmed by RAN5 testing spec 38.521-1, take the same example in the test case 6.2A.2.1	“UE maximum output power reduction for CA (2UL CA)”, there is following description:

Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram

While section A.3.2 is a general section contains many diagrams, Figure A.3.1.1.3, also attached in the annex A, clearly only applies to only one Tx connector for UE per carrier, which is aligned with RAN4 core requirements. This principle is also applied to many other testing cases.

Observation 2: Current RAN5 test case procedures also aligned with RAN4, and requires 1Tx antenna connector per carrier for inter-band UL CA and EN-DC.

Actually, the spec impact of different architectures can be more complicated than expected. Taking Tx diversity as an example, there were early proposals that it could be covered by the original spec or simple clarification in late stage of Rel-15, but it finally became a Rel-17 separate WI after more than one year of discussion. Even now, there are still various issues remains on testing and some other details.

For the current proposal of 1Tx + 2Tx inter-band CA, the requirements, how many spec impacts seems still not clear at current stage. There may be more impact than adding a few reference and clarifications.

Observation 3: For 1Tx + 2Tx inter-band CA/DC, and the spec impact would not be limited to band/BC specific requirements.

Based on the previous observations, the following proposal is provided:

Proposal 1: Confirm if new general (non-band/BC specific) requirements are needed for 1Tx + 2Tx Inter-band UL CA/EN-DC.

RAN once had a conclusion in [3] of guidance on how to differentiate spectrum vs non-spectrum items:

Proposal #1-2: for RAN4 spectrum related and non-spectrum related topics,

Boundary between RAN4 spectrum related items and non-spectrum related items,

- Spectrum related items are item which aim to introduce band-specific and/or band combination specific requirements without impacting generic RF core requirements and/or core specifications of other WGs.

- All the other items shall be defined as non-spectrum related.

Approach to specify RAN4 related features with both general requirements and band specific requirements

- A non-spectrum related item with the chosen example band(s) and/or example band combination(s) is needed to study and/or specify the general RAN4 requirements as well as other necessary mechanism and the band or band combination specific requirements for the examples

  - The example band(s) and/or band combination(s) should be chosen sufficient enough to ensure that the finalized general requirements can be applied to other bands and/or band combinations.

From this we can see that if requirements other than band/BC-specific requirements are needed, a spectrum WI might not be enough.

Proposal 2: If new general requirements identified，RAN4 need to discuss how to resolve this issue. RAN level guidance might be needed.

Furthermore, for handheld UE it is confirmed that no normative work would be done in Rel-18. Even 1Tx+2Tx inter-band UL CA / EN-DC is a somewhat “easier” one among 3Tx scenarios, it is still challenging for the power supply/consumption, thermal dissipation, SAR control et. These drawbacks and restrictions can easily beat the questionable gain that may be introduced by the 3Tx architecture in the actual deployment. So the following proposal is also provided:

Proposal 3: For handheld UE, the specific study should be done after completing FWA work.

Proposal 4: The requirements study for handheld UE may need to consider other factors such as power supply/consumption, thermal dissipation, SAR control etc.

Conclusion

In this paper, some general guidelines and requirements analysis are provided for UE with 3Tx for inter-band UL CA and EN-DC.

Observation 1: Many current RAN4 requirements for inter-band UL CA and EN-DC explicitly requires 1Tx antenna connector per carrier.

Observation 2: Current RAN5 test case procedures also aligned with RAN4, and requires 1Tx antenna connector per carrier for inter-band UL CA and EN-DC.

Observation 3: For 1Tx + 2Tx inter-band CA/DC, and the spec impact would not be limited to band/BC specific requirements.

Proposal 1: Confirm if new general (non-band/BC specific) requirements are needed for 1Tx + 2Tx Inter-band UL CA/EN-DC.

Proposal 2: If new general requirements identified，RAN4 need to discuss how to resolve this issue. RAN level guidance might be needed.

Proposal 3: For handheld UE, the specific study should be done after completing FWA work.

Proposal 4: The requirements study for handheld UE may need to consider other factors such as power supply/ consumption, thermal dissipation, SAR control etc.

- References

[1] RP-223517, Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC, RAN 98-e

[2] R4-2220550, WF on higher power limit increasing for Rel-18, Huawei, RAN4#105

[3] RP-212682, Moderator’s summary for discussion [RAN94e-R18Prep-22], RAN4 Chair (Huawei), RAN#94-e

- Annex

- Reference from 38.508-1 (A.3.1.1	Basic Transmitter/Receiver tests)

Figure A.3.1.1.1: Test Equipment connection for basic single cell, RX and TX tests

Figure A.3.1.1.2: Test Equipment connection for single cell, RX and TX tests for NR UL MIMO

Figure A.3.1.1.3: Test Equipment connection for NR CA, basic RX and TX tests

- Inter-band UL CA requirements

Table 1 38.101-1 Inter-band UL CA requirements

| 38.101-1 Inter-band UL CA requirements | Existing requirement definition | Comments |
| --- | --- | --- |
| 6.2A.1.3 UE maximum output power for Inter-band CA | For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. |  |
| 6.2A.2.3 UE maximum output power reduction for Inter-band CA | For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the requirements in subclause 6.2.2 apply. |  |
| 6.2A.3.1.3 UE additional maximum output power reduction for Inter-band CA | Unless otherwise stated, for inter-band carrier aggregation with one uplink carrier assigned to one NR band, the requirements in subclause 6.2.3 apply. |  |
| 6.2A.4.1.3 Configured transmitted power for Inter-band CA | For uplink inter-band carrier aggregation, MPRc and A-MPRc apply per serving cell c and are specified in clause 6.2.2 and clause 6.2.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c is calculated under the assumption that the transmit power is increased independently on all component carriers. |  |
| 6.3A.1.3 Minimum output power for inter-band CA | For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the minimum output power requirements in clause 6.3.1 apply. |  |
| 6.3A.2.3 Transmit OFF power for inter-band CA | For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit OFF power requirements in subclause 6.3.2 apply. |  |
| 6.3A.3.3 Transmit ON/OFF time mask for inter-band CA | For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit ON/OFF time mask requirements in subclause 6.3.3 apply. |  |
| 6.3A.4.3 Power control for inter-band CA | No requirements unique to CA operation are defined. |  |
| 6.4A.1.3 Frequency error for inter-band CA | For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the frequency error requirements in subclause 6.4.1 apply. |  |
| 6.4A.2.3 Transmit modulation quality for inter-band CA | For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit modulation quality requirements in subclause 6.4.2 apply |  |
| 6.5A.1.3 Occupied bandwidth for Inter-band CA | For inter-band carrier aggregation with uplink assigned to two NR bands, the occupied bandwidth is defined per component carrier. |  |
| 6.5A.2.2.3 Spectrum emission mask for Inter-band CA | For inter-band carrier aggregation with uplink assigned to two NR bands, the spectrum emission mask of the UE is defined per component carrier while both component carriers are active and the requirements are specified in clauses 6.5.2.1 and 6.5.2.2. |  |
| 6.5A.3.2.3 Spurious emissions for UE co-existence for Inter-band CA | For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.3-1 apply on each component carrier with all component carriers are active. |  |
| 6.5A.4.2.3 Transmit intermodulation for Inter-band CA | For inter-band carrier aggregation with uplink assigned to two NR bands, the transmit intermodulation requirement is specified in Table 6.5.4-1 which shall apply on each component carrier with both component carriers active. |  |

- Inter-band ENDC requirements

Table 2 38.101-1 Inter-band ENDC requirements

| 38.101-3 Inter-band EN-DC requirements | Existing requirement definition | Comments |
| --- | --- | --- |
| 6.2B.1 UE maximum output power for DC 6.2B.1.3a Inter-band NE-DC within FR1 | The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. |  |
| 6.2B.2 UE maximum output power reduction for DC 6.2B.2.3 Inter-band EN-DC within FR1 | For inter-band EN-DC between E-UTRA and FR1 NR, UE maximum output power reduction specified in TS 36.101 [4] and TS 38.101-1 [2] apply for E-UTRA and NR respectively. |  |
| 6.2B.3 UE additional maximum output power reduction for EN-DC 6.2B.3.3 Inter-band EN-DC within FR1 | For inter-band EN-DC between E-UTRA and FR1 NR, UE additional maximum output power reduction specified in TS 36.101 [4] and TS 38.101-1 [2] apply for E-UTRA and NR respectively. |  |
| 6.2B.4 Configured output power for DC 6.2B.4.1.3 Inter-band EN-DC within FR1 | For inter-band dual connectivity with one uplink serving cell or more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG and one uplink serving cell on the NR CG or more than one uplink serving cells configured for intra-band UL CA, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission power for EN-DC operation, PTotalEN-DC= 10log10(PtotalEN-DC) with PtotalEN-DC as specified in clause 7.6 of TS 38.213 [10]. |  |
| 6.3B.5 Output power dynamics for inter-band EN-DC | The switching time mask defined in this clause is applicable for a UE indicating support of IE singleUL-Transmission for the specific inter-band EN-DC combination for which only single switched UL is supported. The maximum UL switching time is defined as 120 us. Time masks in Figure 6.3B.5-1 and Figure 6.3B.5-2 shall apply. |  |
| 6.4B.1.3 Frequency error for inter-band EN-DC within FR1 | For inter-band EN-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [4] and in clause 6.4.1 in TS 38.101-1 [2], respectively |  |
| 6.4B.2.3 Transmit modulation quality for Inter-band EN-DC within FR1 | For inter-band EN-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.2 in TS 36.101 [4] and in clause 6.4.2 in TS 38.101-1 [2], respectively |  |
| 6.5B.1 Occupied bandwidth for EN-DC 6.5B.1.3 Inter-band EN-DC within FR1 | Occupied bandwidth requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.1 and 6.6.1A of TS 36.101 [4] and for NR single carrier specified in clause 6.5.1 of TS 38.101-1 [2] apply. |  |
| 6.5B.2 Out-of-band emissions for DC 6.5B.2.3 Inter-band EN-DC within FR1 | Unless otherewise stated, the OOBE requirements specified in clause 6.6.2.1 of TS 36.101 [4], sub- clause 6.6.2 of TS 36.101 [4] and clause 6.5.2 of TS 38.101-1 [2] apply for each component carrier. |  |
| 6.5B.3.3.1 General spurious emissions | The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4], clause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier. |  |
| 6.5B.3.3.2 Spurious emission band UE co-existence | This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.2-1 apply on each component carrier with all component carriers are active. |  |
| 6.5B.4 Additional spurious emissions 6.5B.4.4 Inter-band EN-DC within FR1 | The additional spurious emissions requirements specified for E-UTRA in clause 6.6.3.3 and 6.6.3.3A of TS 36.101 [4] and for NR single carrier, CA operation and UL-MIMO specified in clause 6.5.3.3, 6.5A.3.3 and 6.5D.3 of TS 38.101-1 [2] apply for each component carrier. |  |
| 6.5B.5 Transmit intermodulation for DC 6.5B.5.3 Inter-band EN-DC within FR1 | The transmit intermodulation requirement specified in clauses 6.7.1 of TS 36.101 [4] and clauses 6.5.4 and 6.5A.4 of TS 38.101-1 [2] apply for each component carrier in E-UTRA bands and NR bands, respectively. |  |