3GPP TSG-RAN WG4 Meeting # 106												R4-2301576

Athens, Greece, February 27 – March 3, 2022

Source: 	vivo

Title: 	Discussion on Type 3 UE to support non-collocated deployment

Agenda Item:	9.11.2

Document for:	Approval

- Introduction

In [1], the conflict between MRTD and power imbalance was raised due to the shared LNA in Type 3 UE, and in this contribution, we provide our views on this issue.

- Discussion

In the previous meeting, 25 dB power imbalance was reused for Type 2 UE requirement, and this value is derived from the real non-collocated deployment and any reduction implies the shrink of cell coverage.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | NRCA/ ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | NRCA ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| [3a] | 1 | 4 shared |  | 4 | 4 | 4Rx | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding 2LO/BB/Rx and RF split after 2 LNAs out of 4 => common AGC on LNA => 25dB partial range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |
| [3b] | 1 | 4 shared |  | 4 | 4 | 4Rx | NRCA ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding 2LO/BB/Rx and RF split after all 4 LNAs => common AGC on LNA => 25dB partial range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |
| [4a] | 1 | 4 | 6 total | 4 | 4 | 4Rx | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| [4b] | 1 | 4 | 8 total | 4 | 4 | 4Rx | NRCA ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |

As shown in the table above, compared to Type 2 or Type 4, the most obvious difference is that shared LNA is used to reduce the hardware complexity, but this architecture also expects the timing difference should be within CP otherwise the performance of one of the CCs will be impacted. However, the time difference is related to the deployment which results in different power imbalance, and a mapping is provided in [2].

| Power imbalance (dB) | Inter-site distance (m) | Propagation time difference (us) |
| --- | --- | --- |
| 6 | 65 | 0.22 |
| 10 | 99 | 0.33 |
| 15 | 168 | 0.56 |
| 20 | 283 | 0.94 |
| 25 | 479 | 1.6 |

Considering the TAE = 3 us, the total time difference will be 4.6 us under 25 dB power imbalance, then we further compare this value with CP length:

| Numerology | SCS/kHz | CP/us |
| --- | --- | --- |
| 0 | 15 | NCP:4.69 |
| 1 | 30 | NCP:2.34 |
| 2 | 60 | NCP:1.17 ECP :4.17 |
| 3 | 120 | NCP:0.59 |
| 4 | 240 | NCP:0.29 |

We can find that the timing difference is only within CP when SCS = 15kHz under 25 dB power imbalance.

Observation 1: Under 25 dB power imbalance, only the CP length of 15kHz can be larger than the timing difference.

So, one straightforward idea is to limit the SCS for Type 3 UE under the 4-layer MIMO.

Proposal 1：Discuss whether it is feasible to restrict the SCS = 15 kHz for Type 3 UE in 4-layer MIMO operation.

If this method is not acceptable, it means we have to define the reequipment in the premise of timing difference > CP, and the UE performance degradation may be unpredictable and it will be hard to define the requirement based on such conditions, we may need to reevaluate the feasibility of Type 3 UE.

Proposal 2: If the requirement has to be defined based on timing difference is larger than CP length, reevaluate the feasibility of Type 3 UE based on the UE performance degradation level.

- Conclusion

Observation 1: Under 25 dB power imbalance, only the CP length of 15kHz can be larger than the timing difference.

Proposal 1：Discuss whether it is feasible to restrict the SCS = 15 kHz for Type 3 UE in 4-layer MIMO operation.

Proposal 2: If the requirement has to be defined based on timing difference is larger than CP length, reevaluate the feasibility of Type 3 UE based on the UE performance degradation level.

- References

- R4-2220537 WF on the requirement for intra-band non-collocated CA, EN-DC, KDDI, RAN4#105

- R4-2218193 Further discussion on intra-band non-collocated CA/EN-DC, Apple, RAN4#105