3GPP TSG-RAN WG4 Meeting #106	R4-2301601

# Athens, Greece, Feb 27th - Mar 3rd 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0574 | rev | - | Current version: | 17.20.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-2 on channel raster to RE mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is some ambiguity in Section 5.4.2.2 on channel raster to resource element mapping, where the term NRB refers to TS 38.211 however there is no such term defined in TS 38.211, but only RAN4 specs introduce the term as the maximum transmission bandwidth configuration. This ambiguity leads to different understandings: - Understanding #1: Correcting the reference of NRB as RAN4 specs, not to TS 38.211, and it means maximum transmission bandwidth configuration, thus only when the number of PRBs is equal to the predefined maximum transmission bandwidth configuration, a channel raster is required to be mapped to the designated RE. - Understanding #2: Correcting the mapping by using Ngridsize,μ instead of NRB, then it means that the center of resource grid should be mapped to a channel raster point. This CR is revised according to Undertsanding #1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify NRB refers to RAN4 specs and means the maximum transmission bandwidth configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different understandings on the core requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---- < Start of Change > ---

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on channel raster and the corresponding resource element is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRB mod 2 = 0 | NRB mod 2 = 1 |
| --- | --- | --- |
| Resource element index k | 0 | 6 |
| Physical resource block number nPRB |  |  |

k, nRB are as defined in TS 38.211 [9], and NRB is the maximum transmission bandwidth configuration defined in Table 5.3.2-1.

---<End of Change> ---