| 3GPP TR 38.891 V0.3.0 (2022-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; User Equipment (UE) Further enhancements of NR RF requirements for frequency range 2 (FR2) for Rel-18 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background	7

5	Study for FR2 UL 256QAM	7

5.1	General	7

5.2	Simulation	7

5.2.1	System level simulation	7

5.2.2	Link level simulation	7

5.2.2.1               Simulation assumptions	8

5.2.3	MPR simulation	8

5.3	Implementation impact for UE	9

5.4	Specification impact	9

5.5	Conclusion	9

6	Study for beam correspondence	9

6.1	General	9

6.2	Implementation impact to UE	9

6.3	Specification Impact	9

6.3.1	RF	9

6.3.2	RRM	9

6.4	Testing impact	9

6.5	Conclusion	9

Annex <A> (informative): Change history	10

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document captures finding of the study on FR2 UL 256QAM and beam correspondence for RRC_INACTIVE and initial access. The purpose of this TR is to study the performance gain and implementation aspects for FR2 UL 256QAM and to ensure good UE RACH msg1 performance and UL coverage in FR2 deployments by specifying the beam correspondence requirements for RRC_INACTIVE and initial access as defined in Work Item “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” [2].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-220967, “new WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi

[3]                       3GPP TR 38.803: "Study on new radio access technology: RF and co-existence aspects".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Background

At the 3GPP RAN #95 meeting, the Work Item on “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” was approved for Rel-18. The objectives of the core part in the Work Item are as follows:

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

Beam correspondence requirements for RRC_INACTIVE and initial access

- Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping [RAN4 RF]

- For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT

  - Requirements for other transmission within RRC_INACTIVE state are not precluded.

- For initial access, specify requirements and verification of beam correspondence requirements based on msg1 spherical coverage (at least)

Study the potential impact on testability aspects (i.e., test time).

This TR aims to study the objectives and further capture the impact on the specification if applicable.

5	Study for FR2 UL 256QAM

Editor’s note: This clause collects the simulation results and implementation aspects for FR2 UL 256QAM study for Rel-18. If necessary, new agenda can be assigned.

5.1	General

5.2	Simulation

5.2.1	System level simulation

The system level simulation as supplementary for link level simulation is to check whether the UE working on FR2 UL 256QAM can achieve target SNR at BS side and to further confirm FR2 UL 256QAM is feasible.

5.2.1.1   Simulation assumptions

Table 5.2.1.1-1 System level simulation assumptions

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 200m (baseline) 300m (optional) | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | Uma LOS and NLOS in table 5.2.2.1-1 of 38.803 | InH – Office LOS and NLOS in table 5.2.2.1-1 of 38.803 |
| Carrier frequency |  | 29GHz, 39GHz |  |
| BS antenna configuration |  | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 8 dBi | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | First priority: PC1/PC2/PC5: (Mg, Ng, M, N, P) = (1, 1, 4, 4, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi Second priority: PC3: (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |  |
| System bandwidth |  | 200MHz |  |
| Target SNR at BS side |  | 28dB for 29GHz 30dB for 39GHz |  |
| UE max output power |  | PC1: 35 dBm/PC2: 23dBm/PC3: 23 dBm/PC5: 23 dBm |  |
| UE Min peak EIRP |  | n257 PC1:40.0 dBm/PC2:29 dBm/PC3:22.4 dBm/PC5: 30dBm |  |
| Power control |  | Power control parameters set to achieve the target SNR at BS side. |  |

5.2.1.2   Simulation results from Nokia

R4-2218327

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in urban macro scenario are shown in figures 1 and 2, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 1. Urban macro simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 2. Urban macro simulation results at 39 GHz (n260)

It can be observed from figures 1 and 2 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE.

-	~15% of PC1 UE and ~25% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

The UE transmit power and UL SINR CDF at 29 GHz (n257) and 39 GHz (n260) using the agreed parameters in indoor scenario are shown in figures 3 and 4, respectively.

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 3. Indoor simulation results at 29 GHz (n257)

a) PC1 UE with 35 dBm maximum output power

b) PC2/5 UE with 23 dBm maximum output power

Figure 4. Indoor simulation results at 39 GHz (n260)

It can be observed from figures 3 and 4 that at both 29 GHz (n257) and 39 GHz (n260):

-	The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE.

-	Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE).

5.2.1.3   Simulation results from ZTE

R4-2218595In order to check whether the UE working on FR2 UL 256QAM can achieve target SNR at BS side we provide some system level simulation results with target SNR of 31dB and proposed Min peak EIRP. Figure 4~figure 7 show the CDF of SINR for PC1, PC2, PC3 and PC5 in band n257.

Figure 4, CDF of SINR for PC1 under Urban Macro with Fc = 29GHz

Figure 5, CDF of SINR for PC2 under Urban Macro with Fc = 29GHz

Figure 6, CDF of SINR for PC3 under Urban Macro with Fc = 29GHz

Figure 7, CDF of SINR for PC5 under Urban Macro with Fc = 29GHz

From figure 4~figure 7 we can find that 5% UE can achieve above 30dB SINR which show that PC1, PC2 and PC5 @29GHz are feasible for UL 256QAM. If the operating SNR is smaller than 30dB, then the percentage of UE meet the target SNR will increase, for example, ~10% UE can achieve above 28dB SINR even for PC3.

Observation 3: PC1, PC2, PC3 and PC5 are feasible for UL 256QAM for 29GHz if the operating SNR is not higher than 28dB.

5.2.1.4   Simulation results from vivo

R4-2218869

The SINR distribution for Indoor and Uma are shown in Figure 1:

Figure 1 SINR distribution

Except for PC1, The SINR of UE is hard to larger than 25 dB at the BS side.

Observation 1:

- For PC1, 20% of UE’s SINR can be larger than 25 dB.

- For PC2/PC3/PC5, <5% UE’s SINR can larger than 25 dB.

We further counted the percentage of UEs that had used 256QAM in the simulation and the throughput performance gain. The throughput gain is derived from comparing whether the 256QAM is allowed. The results are shown in below:

Figure 2 proportion and performance gain for 256 QAM

For the indoor scenario, PC1 UE can have obvious performance gain, and about 70% UE is once scheduled with 256QAM during the simulation time, but for other power classes, the throughput performance even become worse, this is because the SINR for these UE cannot support such high MCS and the BLER is bad. For the Uma scenario, even though there are many UEs scheduled with 256 QAM, the performance gain is tiny.

Observation 2:

- For the indoor scenario, only PC1 shows obvious throughput gain, and about 69.3%UEs are scheduled with 256 QAM.

- For the Uma scenario, the performance gain of all PC UEs is tiny.

5.2.1.5  Simulation results from Ericsson

R4-2220036

On Figure 9 we show the results of system-level simulations for 29 GHz carrier frequency and FTP traffic model of both 10% and 30%.

Figure 9: System-level simulation results on 256QAM feasibility for PC1 UE in FR2-1 for 29 GHz carrier frequency

Note that in the simulations results, no sophisticated model was used for the misalignment between BS and UE beam directions. So, in the simulations the ideal beam management was assumed, and the impairment offset of 10 dB was added in the last stage.

By looking at the results, we can see that around 30% of UEs can achieve the operating SNR of 30 dB for FTP with 30% load and around 35% of UEs can achieve the operating SNR of 30 dB for FTP with 10% load, which means that 256 QAM for PC1 UEs in FR2-1 and 29 GHz carrier frequency is feasible.

5.2.2	Link level simulation

Link level simulation is targeted as mainstream way to evaluate if FR2 256QAM can achieve benefit by comparing to 64QAM.

5.2.2.1    Simulation assumptions

The link level simulation assumptions are listed as in table 5.2.2.1-1, based on which, to evaluate the throughput difference between 64QAM and 256QAM. The study aims to identify conditions where UL 256QAM provides performance benefits, does not reflect side conditions for setting the UE RF requirement for UL 256QAM.

Table 5.2.2.1-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. 256QAM: CP-OFDM/DFT-s-OFDM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-s-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) DFT-s-OFDM: (,)=(4, 4), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option b): example2 (UE) + example2(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 3%, 3.5%, 4%, rxEVM: 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2 . |

5.2.2.2   Simulation results from Nokia

R4-2215577

The link level simulation results at 29 GHz (n257) and 39 GHz (n260) using the highlighted parameters in table 1 are shown in figures 1 and 2, respectively.

Figure 1. Simulation results at 29 GHz (n257)

Figure 2. Simulation results at 39 GHz (n260)

It can be observed from figure 1 that at 29 GHz (n257):

-	If MCS index is 21, 256QAM goes over 64QAM in around 24-25 dB for all simulated EVMs.

-	If MCS index is 23, 256QAM goes over 64QAM in around 28 dB for (transmit and receive) EVM of 3%, around 32 dB for (transmit and receive) EVM of 3.5%, and loses for (transmit and receive) EVM of 4%.

On the other hand, it can be observed from figure 2 that at 39 GHz (n260):

-	Only with (transmit and receive) EVM of 0%, MCS21 for 256QAM seems to win 64QAM in around 36 dB.

Note that the PN is challenging case at 39 GHz (n260), and due to small allocation, the number of PTRS symbols is quite low so even higher complexity ICI compensation would not really help here.

R4-2218326

The link level simulation results at 48 GHz (n262) using the highlighted parameters in table 1 with phase noise model option a and option d are shown in figures 1 and 2, respectively.

Figure 1. Simulation results with phase noise model option a

Figure 2. Simulation results with phase noise model option d

It can be observed from figures 1 and 2 that at 48 GHz (n262):

-	256QAM does not provide any throughput gain over 64QAM below 40 dB SNR.

5.2.2.3   Simulation results from LG Electronics

R4-2215920

Table 2 shows the cross throughput point, where the cross throughput point indicates the SNR value at which the throughput of 64QAM and 256QAM is same, that is, after the corresponding SNR, the throughput of 256QAM is higher than 64QAM.

Table 2 Cross throughput point (29 GHz and 48 GHz, CBW:100 MHz, EVM:Tx=Rx)

Referebce: MCS24(64QAM, PUSCH Table 1, OFDM)

| Propagation MCS Mode | EVM: 2% |  | EVM: 3% |  | EVM: 3.5% |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 29 GHz | 48 GHz | 29 GHz | 48 GHz | 29 GHz | 48 GHz |
| TDL-A MCS 21 (256 QAM, PDSCH Table 2, OFDM) | 24.5 dB | N/A | 27 dB | N/A | 30 dB | N/A |
| TDL-A MCS 23 (256 QAM, PDSCH Table 2, OFDM) | 29 dB | N/A | N/A | N/A | N/A | N/A |
| TDL-D MCS 21 (256 QAM, PDSCH Table 2, OFDM) | 18.5 dB | 30 dB | 20 dB | N/A | 21 dB | N/A |
| TDL-D MCS 23 (256 QAM, PDSCH Table 2, OFDM) | 23 dB | N/A | 24.5 dB | N/A | 27 dB | N/A |
| AWGN MCS 21 (256 QAM, PDSCH Table 2, OFDM) | 16 dB | 22.5 dB | 16.5 dB | 24 dB | 17 dB | 27.5dB |
| AWGN MCS 23 (256 QAM, PDSCH Table 2, OFDM) | 19 dB | N/A | 20 dB | N/A | 21 dB | N/A |

29 GHz (n257), EVM(Tx=Rx):3.5%

| Fig1. Throughput performance between 256QAM and 64QAM in TDL-A | Fig2. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig3. Throughput performance between 256QAM and 64QAM in AWGN |  |

29 GHz (n257), EVM(Tx=Rx):3%

| Fig4. Throughput performance between 256QAM and 64QAM in TDL-A | Fig5. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig6. Throughput performance between 256QAM and 64QAM in AWGN |  |

29 GHz (n257), EVM(Tx=Rx):2%

| Fig7. Throughput performance between 256QAM and 64QAM in TDL-A | Fig8. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig9. Throughput performance between 256QAM and 64QAM in AWGN |  |

48 GHz (n262), EVM(Tx=Rx):3.5%

| Fig10. Throughput performance between 256QAM and 64QAM in TDL-A | Fig11. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig12. Throughput performance between 256QAM and 64QAM in AWGN |  |

48 GHz (n262), EVM(Tx=Rx):3%

| Fig13. Throughput performance between 256QAM and 64QAM in TDL-A | Fig14. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig15. Throughput performance between 256QAM and 64QAM in AWGN |  |

48 GHz (n262), EVM(Tx=Rx):2%

| Fig16. Throughput performance between 256QAM and 64QAM in TDL-A | Fig17. Throughput performance between 256QAM and 64QAM in TDL-D |
| --- | --- |
| Fig18. Throughput performance between 256QAM and 64QAM in AWGN |  |

From the simulation results, the followings are observed for UL 256QAM performance gain,

Observation1: In 29 GHz & TDL-A,

- There is the performance gain in MCS21 when EVM ≥ 3.0%

- There is no performance gain in MCS23

Observation2: In 29 GHz & TDL-D

- There is the perfornace gain when EVM = 3.5%

Observation3: In 29 GHz & AWGN

- There is the perfornace gain at a relatively low SNR when EVM = 3.5%.

Observation4: In 48 GHz,

- There is no performance gain in most cases

R4-2218681

Table 2 Cross throughput point (29 GHz and 39 GHz, CBW:100 MHz, EVM:Tx=Rx=3.5%)

Referebce: MCS24(64QAM, PUSCH Table 1, OFDM)

| Propagation Mode | 29 GHz |  |  |  | 39 GHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MCS 20 | MCS 21 | MCS 22 | MCS 23 | MCS 20 | MCS 21 | MCS 22 | MCS 23 |
| TDL-A (256 QAM, PDSCH Table 2, OFDM) (dB) | 25.5 | 30 | N/A | N/A | N/A | N/A | N/A | N/A |
| TDL-D (256 QAM, PDSCH Table 2, OFDM) (dB) | 18.5 | 21 | 23 | 27 | 21 | 24 | 30 | N/A |

29 GHz (n257)

Fig1. Throughput performance between 256QAM and 64QAM in TDL-A

Fig2. Throughput performance between 256QAM and 64QAM in TDL-D

39 GHz (n260)

Fig3. Throughput performance between 256QAM and 64QAM in TDL-A

Fig4. Throughput performance between 256QAM and 64QAM in TDL-D

Observation 2: In 29GHz TDL-A, the throughput gain of 256QAM is shown in MCS index 20 and 21, and operating SNR can vary from 25.5dB to 30dB depending on the MCS index number.

Observation 3: In 29GHz TDL-D, the throughput gain of 256QAM is shown in MCS index 20~23, and operating SNR can vary from 18.5dB to 27dB depending on the MCS index number.

Observation 4: In 39GHz TDL-A, no throughput gain of 256QAM  is shown in MCS index 20~23.

Observation 5: In 39GHz TDL-D, the throughput gain of 256QAM is shown in MCS index 20 ~22, and operating SNR can vary from 21dB to 30dB depending on the MCS index number.

Observation 6: If the operating SNR is considered based on TDL-A, SNR of 25.5 dB or more need to be set for operation at 29GHz, however it is not practical. And N/A is shown at 39GHz, so it is also not practical.

Observation 7: If the operating SNR is considered based on TDL-D with limited MCS, an operating SNR value of 18.5dB to 27dB at 29GHz may be considered, and an operating SNR value of 21dB to 30dB at 39GHz may be considered.

5.2.2.4   Simulation results from vivo

R4-2216128

- 29 GHz

- TDL-A

- TDL-D

- AWGN

The simulation results above are summarized as follows.

Observation 1：The UL 256 QAM under 29 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.0 dB | 22.8 dB | 20 dB | 21 dB | 18 dB | 19.2 dB |
| CP-OFDM | 22.5 dB | 23.8 dB | 20.3 dB | 21.5 dB | 19.2 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.2 dB | 23.1 dB | 20.3 dB | 21.2 dB | 18.5 dB | 19.5 dB |
| CP-OFDM | 22.7 dB | 24 dB | 20.8 dB | 21.8 dB | 19.5 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.9 dB | 23.5 dB | 20.6 dB | 21.5 dB | 19.2 dB | 19.7 dB |
| CP-OFDM | 23.2 dB | 24.7 dB | 21.2 dB | 22 dB | 19.8 dB | 19.9 dB |

- 39 GHz

- TDL-A

- TDL-D

- AWGN

Observation 2：The UL 256 QAM under 39 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24 dB | 27 dB | 22.3 dB | 25.8 dB | 19.9 dB | 22 dB |
| CP-OFDM | 28 dB | N/A | 24 dB | N.A. | 21.5 dB | 26 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.4 dB | 27.5 dB | 23 dB | 26 dB | 20 dB | 23 dB |
| CP-OFDM | 29.2 dB | N/A | 25 dB | N.A. | 21.8 dB | 28 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.8 dB | 28.2 dB | 23.3 dB | 28 dB | 20.3 dB | 23 dB |
| CP-OFDM | 34 dB | N/A | 25.8 dB | N.A. | 22 dB | 30 dB |

- 48 GHz

- TDL-A

- TDL-D

- AWGN

Observation 3：The UL 256 QAM is hard to bring performance gain under 48 GHz

5.2.2.5   Simulation results from Huawei

R4-2216245

The link lever simulation results are shown in Figure 2-1, Figure 2-2 and Figure 2-3 for TDL-D in different frequency ranges. From the simulation results, it is shown that support 256 QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition.

Figure 2-1 Throughput comparison of 256QAM and 64QAM for RANK1_29 GHz

Figure 2-2 Throughput comparison of 256QAM and 64QAM for RANK1_39GHz

Figure 2-3 Throughput comparison of 256QAM and 64QAM for RANK1_48GHz

R4-2219144

The link lever simulation results are shown in Figure 2-1, Figure 2-2, Figure 2-3 and Figure 2-4 for TDL-D in different frequency ranges. From the simulation results, it is shown that support 256 QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition.

Figure 2-1 Throughput comparison of MCS 21 256QAM and 64QAM for RANK1_29 GHz

Figure 2-2 Throughput comparison of MCS 23 256QAM and 64QAM for RANK1_29 GHz

Figure 2-3 Throughput comparison of MCS 21 256QAM and 64QAM for RANK1_39GHz

Figure 2-4 Throughput comparison of MCS 23 256QAM and 64QAM for RANK1_39GHz

5.2.2.6   Simulation results from Sony [R4-2216251]

R4-2216251

We performed simulations and compared the throughput between 64QAM and 256QAM for three different transmit and receive EVM values (3%, 3.5%, and 4%). TX EVM has been the same as RX EVM in each simulation. The simulations are done according to the agreed simulation assumptions [3]. Only Rank-1, and AWGN for 29 GHz, respectively 48 GHz, are simulated so far. 100MHz channel BW and no HARQ have been used. Phase noise is added according to option 2 in the simulation assumptions, i.e., example2, at both the UE and BS.

- 29 GHz

We performed simulations to address whether 256-QAM works in FR2-1 and to demonstrate its performance gain. The result is shown in Figure 1.

Figure 1 Throughput comparing 64QAM and 256QAM with AWGN for 29GHz, EVM 3%, 3.5% and 4%.

Figure 1 shows that throughput for 64QAM (red curve) has already saturated at an SNR level of 16dB. For 256QAM, SNR levels of 22 dB (EVM=3%), 23 dB (3.5%) and 24 dB (EVM=4%) are required to exceed the throughput performance of 64QAM. The simulations have been carried out with a simple receiver, and further studies of advanced receivers are needed to determine whether the above results could be improved.

Observation 1	For 256QAM to exceed throuput performance of 64QAM SNR levels of 22 dB (EVM=3%) to 24 dB (EVM=4%) are required.

R4-2218753

We performed simulations to determine whether 256-QAM works for FR2-1 and demonstrate its performance gain. Figure 1 shows the results.

Figure 1 Throughput comparing 64QAM and 256QAM with AWGN for 29GHz, EVM 3.5%.

Figure 1 shows that throughput for 64QAM (red curves) has already saturated at an SNR level of 16dB. For 256QAM, SNR levels of 20 dB for MCS21 and 25 dB for MCS23 (EVM 3.5%) are required to exceed the throughput performance of 64QAM. To reach throughput saturation for 256QAM, SNR levels of 21dB for MCS21 and 28 dB for MCS23 are needed.

Observation 1	For 256QAM, to reach saturation in throughput performance, SNR levels of 21dB for MCS21 and 28dB for MCS23 are needed for 29GHz.

Simulations have also been performed for 39GHz with the same setup as for 29GHz. Figure 2 shows the results.

Figure 2	 Throughput comparing 64QAM and 256QAM with AWGN for 39GHz, EVM 3.5%.

Figure 2 shows 256QAM performance for 39GHz compared to 64QAM. SNR levels of 21.5dB are needed to exceed the throughput performance of 64QAM for MCS21. For MCS23, an SNR of 36dB is required to exceed the throughput performance of 64QAM. An SNR level of 24dB is needed to reach throughput saturation for 256QAM for MCS21. For MCS23, we didn’t see saturation within the simulated SNR range.

Observation 2	For 256QAM, to reach saturation in throughput performance, SNR levels of 24dB for MCS21 are needed for 39GHz. To exceed the performance of 64QAM, an SNR of 21.5dBm is required.

5.2.2.7   Simulation results from Xiaomi [R4-2216350]

R4-2216350

We compare the UL 256QAM and UL 64QAM performance under different channel conditions and carrier frequencies to determine whether UL 256QAM can provide gains over UL 64QAM for FR2-1. For all simulation, we assumed 50MHz channel bandwidth with 120kHz SCS and phase noise model of example2 (UE) + example2(BS) based on above simulation assumptions.

- CP-OFDM

- 29GHz

Figure 2.1-1 Throughput performance for 29GHz under AWGN, CP-OFDM

Figure 2.1-2 Throughput performance for 29GHz under TDL-D, CP-OFDM

Figure 2.1-3 Throughput performance for 29GHz under TDL-A, CP-OFDM

- 39GHz

Figure 2.1-4 Throughput performance for 39GHz under AWGN, CP-OFDM

Figure 2.1-5 Throughput performance for 39GHz under TDL-D, CP-OFDM

Figure 2.1-6 Throughput performance for 39GHz under TDL-A, CP-OFDM

- 48GHz

Figure 2.1-7 Throughput performance for 48GHz under AWGN, CP-OFDM

- DFT-s-OFDM

- 29GHz

Figure 2.1-8 Throughput performance for 29GHz under AWGN, DFT-s-OFDM

Figure 2.1-9 Throughput performance for 29GHz under TDL-D, DFT-s-OFDM

Figure 2.1-10 Throughput performance for 29GHz under TDL-A, DFT-s-OFDM

- 39GHz

Figure 2.1-11 Throughput performance for 39GHz under AWGN, DFT-s-OFDM

Figure 2.1-12 Throughput performance for 39GHz under TDL-D, DFT-s-OFDM

Figure 2.1-13 Throughput performance for 39GHz under TDL-A, DFT-s-OFDM

- 48GHz

Figure 2.1-14 Throughput performance for 39GHz under TDL-A, DFT-s-OFDM

From the simulation results, table 2.1-1 summary the SNR required to achieve gains for 256QAM compare to 64QAM:

Table 2.1-1 UL 256QAM required SNR to achieve gains compare to 64QAM

| Parameters for UL 256QAM |  |  | SNR required to achieve gains for 256QAM [dB] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 29GHz |  |  |  |  | 39GHz |  |  | 48GHz |  |  |
| Channel | MCS |  |  | 3%+3% | 3.5%+3.5% | 4%+4% | 3%+3% |  | 3.5%+3.5% | 4%+4% | 3%+3% | 3.5%+3.5% | 4%+4% |  |
| AWGN 1x2 RANK1 | MCS21 | CP-OFDM |  | 19.3 | 19.5 | 19.6 | 20 |  | 21.5 | 22 | 24 | 25 | 28 |  |
|  |  | DFT-OFDM |  | 19.3 | 19.5 | 19.6 | 20.1 |  | 21.1 | 21.7 | 22.5 | 23.6 | 25 |  |
|  | MCS23 | CP-OFDM |  | 23 | 23.5 | 25.5 | NA |  | NA | NA | NA | NA | NA |  |
|  |  | DFT-OFDM |  | 22.5 | 23.5 | 24.7 | NA |  | NA | NA | NA | NA | NA |  |
| TDL-D 2x2 RANK2 | MCS21 | CP-OFDM |  | 21.8 | 22.5 | 23.1 | 24.1 |  | 25.2 | 28 | - | - | - |  |
|  |  | DFT-OFDM |  | 22 | 22.6 | 23.3 | 24.1 |  | 25.7 | 28 | - | - | - |  |
|  | MCS23 | CP-OFDM |  | 25.8 | 27.3 | NA | NA |  | NA | NA |  |  |  |  |
|  |  | DFT-OFDM |  | 25.7 | 26.1 | 28 | NA |  | NA | NA | - | - | - |  |
| TDL-A 2x2 RANK2 | MCS21 | CP-OFDM |  | 21.8 | 22.7 | 23.8 | 25.2 |  | 26.7 | NA | - | - | - |  |
|  |  | DFT-OFDM |  | 21.8 | 22.7 | 23.8 | NA |  | NA | NA | - | - | - |  |
|  | MCS23 | CP-OFDM |  | 26.5 | 28 | NA | NA |  | NA | NA | - | - | - |  |
|  |  | DFT-OFDM |  | 25.8 | 27.5 | NA | NA |  | NA | NA | - | - | - |  |
| - means the related simulations haven’t been done due to time limit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation:

And it is shown that supporting UL 256 QAM can provide significant performance gain over UL 64QAM for 3.5% EVM.

For AWGN channel with 3.5% Tx EVM+3.5% Rx EVM, a SNR of >19.5 dB is needed for 29GHz, a SNR of >21.1 dB is needed for 39GHz and a SNR of >23.6 dB is needed for 48GHz.

For TDL-A and TDL-D fading channel with 3.5% Tx EVM+3.5% Rx EVM, a SNR of >22.5 dB is needed for 29GHz, a SNR of >25.2 dB is needed for 39GHz, due to lime limit, the related simulation for 48GHz haven’t been done.

R4-2219075

We further compare the UL 256QAM and UL 64QAM performance under different channel conditions and carrier frequencies to determine whether UL 256QAM can provide gains over UL 64QAM for 48GHz. For all simulations, we assumed 50MHz channel bandwidth with 120kHz SCS and phase noise model of example2 (UE) + example2(BS) based on the agreed simulation assumptions [2].

- CP-OFDM

Figure 2.1-1 Throughput performance for 48GHz under AWGN, CP-OFDM

Figure 2.1-2 Throughput performance for 48GHz under TDL-D, CP-OFDM

Figure 2.1-3 Throughput performance for 48GHz under TDL-D, CP-OFDM

- DFT-s-OFDM

Figure 2.1-4 Throughput performance for 48GHz under AWGN, DFT-s-OFDM

Figure 2.1-5 Throughput performance for 48GHz under TDL-D, DFT-s-OFDM

Figure 2.1-6 Throughput performance for 48GHz under TDL-A, DFT-s-OFDM

From the simulation results for 48GHz, table 2.1-1 updated the SNR required to achieve gains for 256QAM compare to 64QAM:

Table 2.1-1 UL 256QAM required SNR to achieve gains compare to 64QAM

| Parameters for UL 256QAM |  |  | 48GHz SNR required to achieve gains for 256QAM [dB] |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel | MCS |  | 3%+3% | 3.5%+3.5% | 4%+4% |
| AWGN 1x2 RANK1 | MCS21 | CP-OFDM | 24 | 25 | 28 |
|  |  | DFT-OFDM | 22.5 | 23.6 | 25 |
|  | MCS23 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
| TDL-D 2x2 RANK2 | MCS21 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
|  | MCS23 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
| TDL-A 2x2 RANK2 | MCS21 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |
|  | MCS23 | CP-OFDM | NA | NA | NA |
|  |  | DFT-OFDM | NA | NA | NA |

5.2.2.8   Simulation results from ZTE [R4-2216426]

R4-2216426

To study the gain and operating SNR, link level simulation approach was adopted to compare 256QAM with 64QAM. The simulation assumptions are from [2] as listed in Annex A1, and the parameters used in this simulation are marked in red. The throughput curves for CP-OFDM Rank1 transmission are listed in figure A.1~ figure A.9 in Annex for reference.

Figure A.1: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @29GHz AWGN channel

Figure A.2: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @29GHz TDLD channel

Figure A.3: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @29GHz TDLA channel

Figure A.4: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @39GHz AWGN channel

Figure A.5: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @39GHz TDLD channel

Figure A.6: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @39GHz TDLA channel

Figure A.7: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @48GHz AWGN channel

Figure A.8: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @48GHz TDLD channel

Figure A.9: Throughput performance comparison between FR2 UL 256QAM and FR2 UL 64QAM @48GHz TDLA channel

The target SNR values are summarized in table 1, where “-” means that 256QAM can not provide better performance in the given SNR range compared to 64QAM.

Table 1. Target SNR for CP-OFDM, Rank 1 transmission

| index | frequency | Channel | Test setup (64QAM/256QAM) | Target SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | EVM 3.0+3.0 | EVM 3.5+3.5 | EVM 4.0+4.0 |
| 1 | 29GHz | AWGN | MCS23/21 | 21.31 | 21.71 | 22.19 |
| 2 |  |  | MCS24/23 | 25.92 | 27.66 | 31.06 |
| 3 |  | TDL-D | MCS23/21 | 23.42 | 23.73 | 24.16 |
| 4 |  |  | MCS24/23 | 28.44 | 30.10 | 34.28 |
| 5 |  | TDL-A | MCS23/21 | 25.90 | 26.45 | 27.23 |
| 6 |  |  | MCS24/23 | 31.49 | 34.31 | - |
| 7 | 39GHz | AWGN | MCS23/21 | 21.28 | 21.64 | 21.95 |
| 8 |  |  | MCS24/23 | 25.86 | 27.64 | 30.62 |
| 9 |  | TDL-D | MCS23/21 | 26.36 | 27.50 | 29.23 |
| 10 |  |  | MCS24/23 | - | - | - |
| 11 |  | TDL-A | MCS23/21 | 30.48 | 32.35 | 36.62 |
| 12 |  |  | MCS24/23 | - | - | - |
| 13 | 48GHz | AWGN | MCS23/21 | - | - | - |
| 14 |  |  | MCS24/23 | - | - | - |
| 15 |  | TDL-D | MCS23/21 | - | - | - |
| 16 |  |  | MCS24/23 | - | - | - |
| 17 |  | TDL-A | MCS23/21 | - | - | - |
| 18 |  |  | MCS24/23 | - | - | - |

Based on the simulation results, the following observations are given:

Observation 1: For 29GHz:

256QAM performance gain can be expected in the following cases:

- AWGN and TDL-D channel,

- TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected,

- TDL-A channel when MCS23(256QAM)/MCS24(64QAM) and EVM3.0+3.0 or EVM3.5+3.5 are selected

However, 256QAM performance gain can not be expected in the following cases:

- TDL-A channel when MCS23(256QAM)/MCS24(64QAM) and EVM4.0+4.0 are selected.

Observation 2: For 39GHz:

256QAM performance gain can be expected in the following cases:

- AWGN

- TDL-D and TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected

However, 256QAM performance gain can not be expected in the following cases:

- TDL-D and TDL-A channel when MCS23(256QAM)/MCS24(64QAM) are selected.

For 48GHz, using the same phase noise model, the phase noise becomes larger as frequency increases.

Observation 3: For 48GHz:

256QAM performance gain can not be expected for AWGN, TDL-D and TDL-A channel for all the MCS.

R4-2218595

For MCS22 evaluation, the simulation assumptions are from [2] as listed in Annex A1, and the parameters used in this simulation are marked in red. The throughput curves for MCS22 for CP-OFDM Rank1 transmission are listed in figure1~figure 3.

Figure 1, Throughput performance comparison between FR2 UL 256QAM MCS 22 and FR2 UL 64QAM MCS24 @39GHz AWGN channel

Figure 2, Throughput performance comparison between FR2 UL 256QAM MCS 22 and FR2 UL 64QAM MCS24 @39GHz TDL-D channel

Figure3, Throughput performance comparison between FR2 UL 256QAM MCS 22 and FR2 UL 64QAM MCS24 @39GHz TDL-A channel

The updated target SNR values are summarized in table 1, where “-” means that 256QAM can not provide better performance in the given SNR range compared to 64QAM. As EVM was reached in last meeting so the simulation results of EVM 3.0% and EVM 4.0% are not provided.

Table 1. Target SNR for CP-OFDM, Rank 1 transmission

| index | frequency | Channel | Test setup (64QAM/256QAM) | Target SNR |
| --- | --- | --- | --- | --- |
|  |  |  |  | EVM 3.5%+3.5% |
| 1 | 29GHz | AWGN | MCS23/21 | 20.80 |
| 2 |  |  | MCS24/23 | 27.47 |
| 3 |  | TDL-D | MCS23/21 | 22.42 |
| 4 |  |  | MCS24/23 | 27.71 |
| 5 |  | TDL-A | MCS23/21 | 24.03 |
| 6 |  |  | MCS24/23 | 29.07 |
| 7 | 39GHz | AWGN | MCS23/21 | 22.00 |
|  |  |  | MCS24/22 | 27.55 |
| 8 |  |  | MCS24/23 | - |
| 9 |  | TDL-D | MCS23/21 | 26.00 |
|  |  |  | MCS24/22 | 29.61 |
| 10 |  |  | MCS24/23 | - |
| 11 |  | TDL-A | MCS23/21 | 26.09 |
|  |  |  | MCS24/22 | 30.70 |
| 12 |  |  | MCS24/23 | - |
| 13 | 48GHz | AWGN | MCS23/21 | - |
| 14 |  |  | MCS24/23 | - |
| 15 |  | TDL-D | MCS23/21 | - |
| 16 |  |  | MCS24/23 | - |
| 17 |  | TDL-A | MCS23/21 | - |
| 18 |  |  | MCS24/23 | - |

Based on the simulation results, the following observations are given:

Observation 1: For 39GHz:

UL 256QAM performance gain can be expected in the following cases:

- AWGN, TDL-D and TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected

- AWGN, TDL-D and TDL-A channel when MCS22(256QAM)/MCS24(64QAM) are selected

Observation 2: UL 256QAM performance gain can not be expected for 48GHz.

5.2.2.9   Simulation results from Ericsson

R4-2220036

On Figure 1 – Figure 8 we provide simulation results for various scenarios described in figure captions and figure legends.

Figure 1: 29GHz, 100MHz, No HARQ, AWGN 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 2: 29GHz, 100MHz, No HARQ, Fading 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 3: 29GHz, 100MHz, No HARQ, AWGN 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

Figure 4: 29GHz, 100MHz, No HARQ, Fading 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

Figure 5: 29GHz, 100MHz, HARQ=8, AWGN 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 6: 29GHz, 100MHz,  HARQ=8, Fading 2x2 Rank 1 channel, txEVM= rxEVM=3.5%

Figure 7: 29GHz, 100MHz, HARQ=8, AWGN 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

Figure 8: 29GHz, 100MHz, HARQ=8, Fading 2x2 Rank 2 channel, txEVM= rxEVM=3.5%

In Table 2 we have summarized results where we indicate two operating SNR points where 256QAM MCS21 outperforms 64QAM MCS23 and 64QAM MCS24, respectively.

| HARQ Type | Channel type | Cross-point between 256QAM MCS21 and 64QAM MCS23 | Cross-point between 256QAM MCS21 and 64QAM MCS24 |
| --- | --- | --- | --- |
| No HARQ | AWGN 2x2 Rank 1 | 26.2 dB | 26.8 dB |
|  | AWGN 2x2 Rank 2 | 28.7 dB | 29 dB |
|  | Fading 2x2 Rank 1 | 27.9 dB | 28.1 dB |
|  | Fading 2x2 Rank 2 | 32.4 dB | 33.2 dB |
| HARQ=8 | AWGN 2x2 Rank 1 | 26 dB | 26.4 dB |
|  | AWGN 2x2 Rank 2 | 28.1 dB | 28.8 dB |
|  | Fading 2x2 Rank 1 | 27.1 dB | 27.8 dB |
|  | Fading 2x2 Rank 2 | 31.2 dB | 32.2 dB |

Table 2: Summary of link-level simulation results

We conclude first that between not having HARQ and having HARQ=8 repetitions there are no big differences when it comes to the cross-point (operating SNR) between 256QAM MCS21 and 64QAM MCS23/MCS24, respectively. For the AWGN channel, the operating SNR is lower by 0.3 dB in average in the “No HARQ” case compared with “HARQ=8” case, while for the TDL-D fading channel the operating SNR is lower by 0.8 dB in the “No HARQ” case compared with “HARQ=8” case.

For “No HARQ” case, for AWGN channel the average operating SNR is equal to 27.7 dB.

For “No HARQ” case, for TDL-D fading channel the average operating SNR is equal to 30.4 dB.

5.2.2.10   Summary for 29GHz simulation results

| Parameter |  |  | Nokia R4-2215577 | LG Electronics R4-2215920 R4-2218681 | vivo R4-2216128 | Huawei R4-2216245 R4-2219144 | Sony R4-2216251 R4-2218753 | Xiaomi R4-2216350 | ZTE R4-2216426 R4-2218595 | Ericsson R4-2220036 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz |
| CBW |  |  | 50MHz | 100MHz | 50MHz/100MHz | 50MHz | 100MHz | 50MHz | 100MHz | 100MHz |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| HARQ |  |  | none | none |  | none | none | none | none | none/8 |
| RANK |  |  | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 |
| Phase noise model |  |  | Option a): example1 (UE) + example1(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option a): example1 (UE) + example1(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS20 |  | 25.5 |  |  |  |  |  |  |
|  |  | MCS21 |  | 30 | 22.7/24 |  |  | 22.7 | 26.45/24.03 |  |
|  |  | MCS22 |  | NA |  |  |  |  |  |  |
|  |  | MCS23 |  | NA | 26.3/28.4 |  |  | 28 | 34.31/29.07 |  |
|  | TDL-D | MCS20 |  | 18.5 |  |  |  |  |  |  |
|  |  | MCS21 | 24-25 | 21 | 20.8/21.8 | 20.1 | 23 | 22.5 | 23.73/22.42 | 27.9/27.1 |
|  |  | MCS22 |  | 23 |  |  |  |  |  |  |
|  |  | MCS23 | 32 | 27 | 25.7/28 | 23 |  | 27.3 | 30.10/27.71 |  |
|  | AWGN | MCS21 |  | 17 | 19.5/19.7 |  | 20 | 19.5 | 21.71/20.8 | 26.2/26 |
|  |  | MCS23 |  | 21 | 22.2/23.8 |  | 25 | 23.5 | 27.66/27.47 |  |

Average value for 29GHz operating SNR based on TDL-D:

|  | Operating SNR (dB) with EVM 3.5% |  |  |  |
| --- | --- | --- | --- | --- |
|  | TDL-D, MCS20 | TDL-D, MCS21 | TDL-D, MCS22 | TDL-D, MCS23 |
| Nokia | - | 25 | - | 32 |
| LGE | 18.5 | 21 | 23 | 27 |
| vivo | - | 21.8 | - | 28 |
| Huawei | - | 20.1 | - | 23 |
| Xiaomi | - | 22.5 | - | 27.3 |
| ZTE | - | 22.42 | - | 27.71 |
| Ericsson | - | 27.9 | - | - |
| Average | 18.5 | 23 | 23 | 27.5 |

5.2.2.11   Summary for 39GHz simulation results

| Parameter |  |  | Nokia R4-2215577 | LG Electronics R4-2218681 | vivo R4-2216128 | Huawei R4-2216245 R4-2219144 | Sony R4-2218753 | Xiaomi R4-2216350 | ZTE R4-2216426 R4-2218595 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz |
| CBW |  |  | 50MHz | 100MHz | 50MHz/100MHz | 50MHz | 50MHz | 50MHz | 100MHz |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| HARQ |  |  | none | none | none | none | none | none | none |
| RANK |  |  | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 |
| Phase noise model |  |  | Option a): example1 (UE) + example1(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS20 |  | NA |  |  |  |  |  |
|  |  | MCS21 |  | NA | 29.2/NA |  |  | 26.7 | 32.35/26.09 |
|  |  | MCS22 |  | NA |  |  |  |  | 30.70 |
|  |  | MCS23 |  | NA | NA |  |  | NA | NA |
|  | TDL-D | MCS20 |  | 21 |  |  |  |  |  |
|  |  | MCS21 | 36 | 24 | 25/NA | 21 |  | 25.2 | 27.50/26 |
|  |  | MCS22 |  | 30 |  |  |  |  | 29.61 |
|  |  | MCS23 |  | NA | NA | 25.4 |  | NA | NA |
|  | AWGN | MCS21 |  |  | 21.8/28 |  | 21.5 | 21.5 | 21.64/22 |
|  |  | MCS22 |  |  |  |  |  |  | 27.55 |
|  |  | MCS23 |  |  | NA |  | 36 | NA | 27.64 |

Average value for 39GHz operating SNR based on TDL-D:

|  | Operating SNR (dB) with EVM 3.5% |  |  |  |
| --- | --- | --- | --- | --- |
|  | TDL-D, MCS20 | TDL-D, MCS21 | TDL-D, MCS22 | TDL-D, MCS23 |
| Nokia | - | 36 | - | - |
| LGE | 21 | 24 | 30 | NA |
| vivo | - | 25 | - | NA |
| Huawei | - | 21 | - | 25.4 |
| Xiaomi | - | 25.2 | - | NA |
| ZTE | - | 26 | 29.21 | NA |
| Average | 21 | 26.2 | 29.6 | NA |

5.2.2.12   Summary for 48GHz simulation results

| Parameter |  |  | LG Electronics R4-2215920 | vivo R4-2216128 | Huawei R4-2216245 | Xiaomi R4-2216350 R4-2219075 | ZTE R4-2216426 R4-2219595 | Nokia R4-2218326 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 48 GHz | 48 GHz | 48 GHz | 48 GHz | 48 GHz | 48 GHz |  |
| CBW |  |  | 100MHz | 50MHz/100MHz | 50MHz | 50MHz | 100MHz | 50MHz |  |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |  |
| HARQ |  |  | none | none | none | none | none | none |  |
| RANK |  |  | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 | RANK1 |  |
| Phase noise model |  |  | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option a): example1 (UE) + example1(BS) | Option d): example1 (UE) + example2(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS21 | NA | NA |  | NA | NA |  |  |
|  |  | MCS23 | NA | NA |  | NA | NA |  |  |
|  | TDL-D | MCS21 | NA | NA | 22.6 | NA | NA | NA | NA |
|  |  | MCS23 | NA | NA |  | NA | NA | NA | NA |
|  | AWGN | MCS21 | 27.5 | NA |  | 25 | NA |  |  |
|  |  | MCS23 | NA | NA |  | NA | NA |  |  |

5.2.3	MPR simulation

5.2.3.1   Simulation assumptions

Antenna configuration and PA calibration point for MPR simulation

      - The MPR evaluation was performed by using 32 PAs, 16 for each polarizations within an antenna array for PC1/2/5 keeping align with the antenna configuration agreed in system level simulation.

      - PA calibration point should follow current definition in Spec 38.101-2:

  - The waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

  - Calculate MPR as total backoff needed for 256QAM from this calibration point.

Emission requirements for MPR simulation

Follow the current Spec 38.101-2, otherwise specified

      - Occupied bandwidth (Table 6.5.1-1 in TS 38.101-2)

      - General NR spectrum emission mask for FR2-1 (Table 6.5.2.1-1 in TS 38.101-2)

      - NR ACLR1 for FR2-1 (Table 6.5.2.3-1 in TS 38.101-2)

      - General in-band emissions limit for FR2-1 (Tables 6.4.2.3.2-1 for PC1, 6.4.2.3.3-1 for PC2, 6.4.2.3.6-1 for PC5 in TS 38.101-2)

      - General NR spurious emission limits for FR2-1 (Table 6.5.3-2 in TS 38.101-2)

      - Maximum error vector magnitude (EVM) 3.5% for 29GHz and 39GHz as agreed in RAN4 #104-bis-e meeting.

5.3	Implementation impact for UE

5.4	Specification impact

5.5	Conclusion

6	Study for beam correspondence

6.1	General

6.2	Implementation impact to UE

6.3	Specification Impact

6.3.1	RF

6.3.2	RRM

6.4	Testing impact

6.5	Conclusion

Editor’s note:

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4 #104-e | R4-2212590 |  |  |  | TR skeleton | 0.0.1 |
| 2022-10 | RAN4 #104-bis-e | R4-2217730 |  |  |  | TP for TR 38.891 on link level simulation assumptions for FR2 UL 256QAM | 0.1.0 |
| 2022-11 | RAN4 #105 | R4-2219122 |  |  |  | TP for TR 38.891 on link level simulation results and system level simulation assumption for FR2 UL 256QAM | 0.2.0 |

Change history of this template:

| 2001-07 | Copyright date changed to 2001; space character added before TTC in copyright notification; space character before first reference deleted. | 1.3.3 |
| --- | --- | --- |
| 2002-01 | Copyright date changed to 2002. | 1.3.4 |
| 2002-07 | Extra Releases added to title area. | 1.3.5 |
| 2002-12 | "TM" added to 3GPP logo. | 1.3.6 |
| 2003-02 | Copyright date changed to 2003. | 1.3.7 |
| 2003-12 | Copyright date changed to 2004. Chinese OP changed from CWTS to CCSA | 14.0 |
| 2004-04 | North American OP changed from T1 to ATIS | 1.5.0 |
| 2005-11 | Stock text of clause 3 includes reference to 21.905. | 1.6.0 |
| 2005-11 | Caters for new TSG structure. Minor corrections. | 1.6.1 |
| 2006-01 | Revision marks removed. | 1.6.2 |
| 2008-11 | LTE logo line added, © date changed to 2008, guidance on keywords modified; acknowledgement of trade marks; sundry editorial corrections and cosmetic improvements | 1.7.0 |
| 2010-02 | 3GPP logo changed for cleaner version, with tag line; LTE-Advanced logo line added; © date changed to 2010; editorial change to cover page footnote text; trade marks acknowledgement text modified; additional Releases added on cover page; proforma copyright release text block modified | 1.8.0 |
| 2010-02 | Smaller 3GPP logo file used. | 1.8.1 |
| 2010-07 | Guidance note concerning use of LTE-Advanced logo added. | 1.8.2 |
| 2011-04-01 | Guidance of use of logos on cover page modified; copyright year modified. | 1.8.3 |
| 2013-05-15 | Changed File Properties to MCC macro default. Removed R99, added Rel-12/13. Modified Copyright year. Guidance on annex X Change history. | 1.8.4 |
| 2014-10-27 | Updated Release selection on cover. In clause 3, added "3GPP" to TR 21.905. | 1.8.5 |
| 2015-01-06 | New Organizational Partner TSDSI added to copyright block. Old Releases removed. | 1.9.0 |
| 2015-12-03 | Provision for LTE Advanced Pro logo Update copyright year to 2016 | 1.10.0 |
| 2016-03-08 | Standarization of the layout of the Change History table in the last annex.(Unreleased) | 1.11.0 |
| 2016-06-15 | Minor adjustment to Change History table heading | 1.11.1 |
| 2017-03-13 | Adds option for 5G logo on cover | 1.12.0 |
| 2017-05-03 | Smaller 5G logo to reduce file size | 1.12.1 |
| 2019-02-25 | Replacement of frames on cover pages by in-line text. Clarification of help text on when to use 5G logo. Removal of defunct keywords frame on page 2. Add Rel-16, Rel-17 options, eliminated earlier, frozen, Releases (cover page, below title) Corrections to some guidance text, addition of guidance text concerning automatic page headers under Word 2016 ff. Use of modal auxiliary verbs added to Foreword. More explicit guidance on Bibliography and Index annexes. Converted to .docx format. | 1.13.0 |