3GPP TSG-RAN WG4 Meeting # 106	R4-2301644

# Athens, Greece, February 27 – March 03, 2023

Agenda Item:	9.24.2.3

Source: 	OPPO

Title: 	Discussion on network A requirements

Document for:	Approval

1	Introduction

MUSIM gaps were discussed and introduced in Rel-17. However, the corresponding requirements were postponed due to the lack of TU in RAN4. The WID [1] was revised to study the missed RRM requirements for Rel-17 MUSIM gaps. Some issues were discussed in the last meeting and the conclusions are captured in [2]. This contribution will provide our considerations on NW-A requirements.

2	Discussion

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling - WF - Suggest the following options to be used in future discussion: - Option 1: Frameworks of gap cancellation in concurrent gaps design and LBT failure in NR-U design can be used as starting point when discussing NW A requirement impact. - Option 2: Reuse the principle used in Rel-17 concurrent gaps WI as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Navailable / Ntotal for network A requirements when MUSIM gaps are configured - Option 3: Existing UE requirements applicable for Network A shall apply when UE is allocated MUSIM gaps

In case of MUSIM gap collision with L3 measurement without gap or L1 measurements, our view is that MUSIM gap should be prioritized by default. Then UE is supposed to measure the SMTC or RS resources outside gap, and the legacy scaling factor Kp or P can be reused.

Proposal-1: Support option 2, reuse the principle in Rel-17 concurrent gaps WI as the baseline to define NW-A L1/L3 measurement requirements when MUSIM gaps are configured.

Intra-frequency measurement withoug gap: CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps or NCSG, or according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps, or according to CSSFwithin_ncsg,i in clause 9.1.5.3 for measurement conducted within NCSG, i.e. when intra-frequency SMTC is fully overlapping with NCSG. When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps, Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3. Kp = 1 when Navailable = 0.

SSB based RLM When concurrent gaps are configured, - P value for an RLM-RS resource to be measured is defined as - Ntotal / Noutside_MG in FR1 - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0 - Ntotal / Navailable in FR2 with Navailable > 0 - For a window W of duration max(TL1, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion: - Ntotal is the total number of RLM-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and - Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W - Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W - TL1 is periodicity of the target RLM-RS.

One special case is that when SMTC is fully overlapping with MUSIM gap. According the current TS 38.133 spec, when the SMTC for intra-frequency measurement without gap is fully overlapping with MG, the CSSFintra is set as CSSFwithin_gap,i in clause 9.1.5.2 and the scaling factor Kp is 1, implying that it will share the MG occasion with other MOs. While L1 resources are assumed to be measured outside gap and the Noutside_MG should be larger than 0 as shown above. In our understanding, it is not reasonable to perform L3 measurement in NW-A with MUSIM gap since it is configured dedicated for NW-B. It is better to preclude such case similar as L1 measurement.

Proposal-2: For L3 measurement without gap, SMTC should not be fully overlapping with MUSIM gap.

Issue 3-1-3: On the time window W for aperiodic gap - Proposals - P1: Window length W for aperiodic gap should be discussed providing the principle of Rel-17 concurrent WI is reused for the L1/L3 measurement requirement specification when MUSIM gaps are configured. (vivo) - P2: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer. (Huawei) - P3: max(SMTC period, MGRP_max)+[M], where MGRP_max is the largest periodicity among all the periodic gaps and [M] is a time margin for the one-shot aperiodic MUSIM gap. (MTK)

In order for paging reception in NW-B, aperiodic MUSIM gap is introduced in Rel-17. So it is necessary to define requirements when aperiodic MUSIM gap collides with measurement in NW-A. One simple way is to reuse the existing time window W and do not consider aperiodic gap. When the measurement in NW-A is overlapped with aperiodic MUSIM gap (with higher priority by default), simply clarify that the measurement period could be longer without explicitly define the extension.

Proposal-3: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer.

3	Conclusion

This contribution gave our general views on NW-A requirements for Rel-17 MUSIM gaps and the following proposals:

Proposal-1: Support option 2, reuse the principle in Rel-17 concurrent gaps WI as the baseline to define NW-A L1/L3 measurement requirements when MUSIM gaps are configured.

Proposal-2: For L3 measurement without gap, SMTC should not be fully overlapping with MUSIM gap.

Proposal-3: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer.

4	Reference

- RP-220955, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo

- R4-2220443, WF on NR Dual Tx/Rx Multi-SIM, vivo