3GPP TSG-RAN WG4 Meeting #106 						 R4-2301682

Athens, Greece, February 27 – March 3, 2022

Title:	RF simulation results for transparent schemes

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	9.26.2.3

Document for:	Discussion

# 1	Introduction

This contribution relates to a new work item agreed in RAN#94-e, namely “Further NR coverage enhancements” [1]. We consider power domain enhancements and the following objectives captured in the WID:

- Study and if necessary specify following power domain enhancements

  - […]

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

In this contribution we provide results of the UE transmitter performance for power domain enhancements for transparent schemes. The corresponding results for non-transparent schemes are shown in [5]. Based on the following agreement made in RAN4 #104bis-e, FDSS w/o SE for DFT-s-OFDM is the transparent scheme according to WID. Hence, we’ll focus on that.

Agreement:

- Frequency domain spectrum shaping without spectrum extension for DFT-S-OFDM is the transparent scheme thus far according to the WID

- Other techniques can be discussed depending on RAN Plenary decision

# 2	Simulation parameters

Simulation parameters used in these simulations are shown in Table 1 for FR1, and in Table 2 for FR2, respectively.

Table 2 Simulation parameters for FR1 for FDSS

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | Case 1: 20 MHz Case 2: 100 MHZ |
| SCS | Case 1: 15/30/60 kHz Case 2: 30 kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Number of RBs | 16, 32, 64, … < NRB |
| Allocation type | Sweep over the channel |
| Extension factor | 0 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap, FD implementation - (0.335 1 0.335) - (0.28 1 0.28) - 2-Tap for FDSS QPSK w/o SE - (1 0.28) - Truncated RRC - No filter (reference case) |
| Power class | PC 3 |

Table 3 Simulation parameters for FR2 for FDSS (green: agreed already)

| Carrier frequency | 28GHz |
| --- | --- |
| Channel BW | 400MHz |
| SCS | 120kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Number of RBs | 16, 32, 64, … < NRB |
| Allocation type | Sweep over the channel |
| Extension factors | 0, (for reference 0.125, 0.25, 0.375) |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap, FD implementation - 0.335 1 0.335 - Truncated RRC - No filter (reference case) |
| Power class | PC 3 |

# 3	Simulation results

## 3.1 FR1 MPR results for FDSS

Simulation results with required MPR for FR1 are shown in this section. For completeness, the required MPR for legacy DFT-s-OFDM (i.e., without FDSS) is included in the figures.

### 3.1.1 FR1 MPR results for 20 MHz channel, 15 kHz SCS

Figure 1 shows the required MPR for transparent schemes of 16, 32, 64 and 96 PRB. It can be noted that OBO gain from transparent schemes (compared to case without filter) is typically less than 1 dB. The results also show that the two-taps filter [1 0.28] agreed to be part of the study in RAN4 #105 requires higher OBO than the three-taps filter [0.335 1 0.335] or TRRC filter. The biggest OBO gain is achieved with three-tap filter “0.28”. However, when compared to non-transparent schemes (in [5]), it can be noted non-transparent schemes outperform transparent schemes with a clear margin. Furthermore, as shown in [5], in the big picture transparent schemes don’t provide net gain with QPSK (in other words, OBO gain < Rx loss).

Observation 1: It can be noted that OBO gain from transparent schemes (compared to the case without filter) is typically less than 1 dB. For the largest RB allocations, the OBO gain is about 1.5 dB.

Observation 2: The two-taps filter [1 0.28] requires higher OBO than the three-taps filter [0.335 1 0.335] or TRRC filter.

Observation 3: When compared to non-transparent schemes (in [5]), it can be noted that non-transparent schemes outperform transparent schemes with a clear margin.

Observation 4: Furthermore, as shown in [5], transparent schemes don’t provide net gain with QPSK (in other words, OBO gain < Rx loss).

Figure 1. Required MPR for 20 MHz channel, 15 kHz SCS with the evaluated filters TRCC, [0.335 1 0.335], [0.28, 1 0.28] and [1, 0.28], for 16 PRB (top left), 32 PRB (top right), 64 PRB (bottom left) and 96 PRB (bottom right)

3.1.2 FR1 MPR results for 20 MHz channel, 30 kHz SCS

Figure 2 shows the required MPR for transparent schemes of 8, 16, 32, and 48 PRB. When compared to results with 15 kHz SCS (Figure 1), it can be noted that for similar FDRA, the OBO behaviour is very similar between 15 kHz and 30 kHz SCSs. Additionally, it can be noted that the two-taps filter [1 0.28] requires higher OBO than the three-taps filters [0.28, 1 0.28] and for higher PRBs, the two- taps filter requires higher OBO than the three-taps filter [0.335 1 0.335] or TRRC filter.

Observation 5: For similar FDRA, OBO behaviour is very similar between different SCSs.

.

Figure 2. Required MPR for 20 MHz channel, 30 kHz SCS with the evaluated filters TRCC, [0.335 1 0.335], [0.28, 1 0.28] and [1, 0.28], for 8 PRB (top left), 16 PRB (top right), 32 PRB (bottom left) and 48 PRB (bottom right).

### 3.1.1 FR1 MPR results for 20 MHz channel, 60 kHz SCS

Figure 3 shows the required MPR for transparent schemes of 4, 8, 16, and 24 PRB. When compared to results with 15 kHz SCS (Figure 1) and with 30 kHz SCS (Figure 2), it can be noted again, that for similar FDRA, the OBO behaviour is very similar between different SCSs. Additionally, it is noted two-taps filter [1 0.28] requires higher OBO than the three-taps filter [0.28, 1 0.28] and for higher PRBs the two- taps filter requires higher OBO than the three-taps filter [0.335 1 0.335] or TRRC filters.

.

Figure 3. Required MPR for 20 MHz channel, 60 kHz SCS with the evaluated filters TRCC, [0.335 1 0.335], [0.28, 1 0.28] and [1, 0.28], for 4 PRB (top left), 8 PRB (top right), 16 PRB (bottom left) and 24 PRB (bottom right).

### 3.1.2 FR1 MPR results for 100 MHz channel, 30 kHz SCS

Figure 4 shows the required MPR for transparent schemes of 16, 32, 64, 128 and 256 PRBs. The trend is very similar to with the 20 MHz CBW case: It can be noted that OBO gain from transparent schemes (compared to case without filter) is typically less than 1 dB. The gain is biggest with the most aggressive filter. When compared to non-transparent schemes (in [5]), it can be noted non-transparent schemes they outperform transparent schemes with a clear margin.

Figure 4. Required MPR for 100 MHz channel, 30 kHz SCS with the evaluated filters TRCC, [0.335 1 0.335], [0.28, 1 0.28] and [1, 0.28], for 16 PRB (top left), 32 PRB (top right), 64 PRB (bottom left), 128 PRB (bottom right) and 256 PRB (bottom center.)

Observation 6: There are no major differences in OBO performance between 20 MHz CBW and 100 MHz CBW cases.

## 3.2 FR2 MPR results for FDSS

Simulation results with required MPR for FR2 are shown in this section. For completeness, the required MPR for legacy DFT-s-OFDM (i.e., without FDSS) is included in the figures.

## 3.2.1 FR2 MPR

Figure 5 shows the required MPR for different combinations of filter and extensions in the case of 16, 32, 64, 128 and 256 PRBs. Transparent schemes are marked with “0%” (extension). Different filter-extension combinations offer the lowest MPR depending on the allocation size. Overall, [0.335 1 0.335] with 37.5% extension offers a good performance over the evaluated allocations. TRRC and [0.335 1 0.335] with 25% extension are overall a good filter-extension choice.

Figure 5 Required MPR for 400 MHz channel, 120 kHz SCS with the evaluated filters, for 16 PRB (top left), 32 PRB (top right), 64 PRB (middle left) and 128 PRB (middle right) and 256 PRB (bottom)

Observation 7: Non-transparent schemes outperform transparent schemes with a clear margin in FR2.

# 4.	Conclusion

In this contribution we provided preliminary results of the UE transmitter performance for power domain enhancements. The focus of this paper was on transparent schemes. Based on the results we make the following observations:

Observation 1: It can be noted that OBO gain from transparent schemes (compared to the case without filter) is typically less than 1 dB. For the largest RB allocations, the OBO gain is about 1.5 dB.

Observation 2: The two-taps filter [1 0.28] requires higher OBO than the three-taps filter [0.335 1 0.335] or TRRC filter.

Observation 3: When compared to non-transparent schemes (in [5]), it can be noted that non-transparent schemes outperform transparent schemes with a clear margin.

Observation 4: Furthermore, as shown in [5], transparent schemes don’t provide net gain with QPSK (in other words, OBO gain < Rx loss).

Observation 5: For similar FDRA, OBO behaviour is very similar between different SCSs.

Observation 6: There are no major differences in OBO performance between 20 MHz CBW and 100 MHz CBW cases.

Observation 7: Non-transparent schemes outperform transparent schemes with a clear margin in FR2.

# References

- RP-213579, “New WI: Further NR coverage enhancements”, China Telecom

- R4-2215515, “Enhancements to reduce MPR/PAR”, Nokia, Nokia Shanghai Bell

- R4-2217745, “WF on enhancements to reduce MPR/PAR”, Nokia, Nokia Shanghai Bell

- R1-2210326, “Final FL summary of power domain enhancements (AI 9.14.2)”, Moderator (Nokia, Nokia Shanghai Bell)

- R4-2301683, “RF simulation results for non-transparent schemes”, Nokia, Nokia Shanghai Bell