3GPP TSG-RAN WG4 Meeting # 106	R4-2301693

Meeting, February 27th – March 3rd , 2023

Title:	TP to TR 38.718.01-01 addition of CA_n46C uplink

Source:	Nokia, BT

Agenda item:	8.9.2

Document for:	Approval

# 1	Introduction

This TP adds contiguous intra-band uplink CA configuration to CA_n46C as defined in revised WID on NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y). RP-222846 [1]

# 2. Reference

- RP-222846, Revised WID: Rel-18 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y)

************************************* Start of TP*****************************************

## 5.x	CA_2DL_n46C_2UL_n46C

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n46 | n46 |

Table 5.X.1-2: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n46C | CA_n46C | 60, 80 | 60, 80 |  |  |  | 160 | 0 |

### 5.X.2	UE maximum output power for Intra-band contiguous CA

Table 5.X.2.1-1: UE Power Class for intra-band contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46C |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

### 5.X.3	UE additional maximum output power reduction for CA

Not needed as there are no additional emission requirements.

### 5.X.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Spurious emissions requirements for UE coexistence are not applicable to bands restricted to stand-alone operation with shared spectrum channel access as identified in Table 5.2-1.

### 5.X.5	Reference sensitivity power level for Intra-band contiguous CA

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements

### 5.X.6	In-band blocking

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements

### 5.X.7	Out-of-band blocking

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements

### 5.X.8	Narrow band blocking

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements

************************************* End of TP*****************************************