3GPP TSG-RAN WG4 Meeting#106                              R4-2301727

Athens, Greece,27th Feb – 3rd Mar, 2023

Agenda item:	9.13.2.1

Source: 	ZTE Corporation

Title: 	Further discussion on ATG UE Tx RF requirements

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. During the previous RAN4 meetings, we have reached some initial progress for ATG UE in [6], however there are still lots of open issues left for further discussions. For the ACLR/SEM and ACS requirement of ATG UE, it should be dependent on the outcome of ATG coexistence study as following. In this contribution, we want to share further views on ATG UE RF requirements based on the initial evaluation results obtained so far.

Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

Discussion

## 2.1. ACLR requirement of ATG UE

Based on the initial simulation results obtained so far for Case 2 and Case 10 as shown in Figure 1 and Figure 2, ACI caused by ATG UL towards TN UL are still quite limited. In short, based on the initial simulation results in Case 2 and Case 10 dominating the ATG UE UL ACLR requirement, it should be sufficient to reuse the legacy FR1 UE PC3 ACLR requirement 30dBc requirement for ATG UE.

Figure 1a. simulation results for Case 2: ATG UL interfering TN UL @4GHz [non co-located deployment]

Figure 1b. simulation results for Case 2: ATG UL interfering TN UL @4GHz [co-located deployment]

Figure 2a. simulation results for Case 10: ATG UL interfering TN UL @2GHz [non co-located deployment]

Figure 2b. simulation results for Case 10: ATG DL interfering TN UL @2GHz [co-located deployment]

Observation 1: based on the initial simulation results in Case 2 and Case 10, it should be sufficient to reuse the legacy FR1 UE PC3 ACLR requirement 30dBc requirement for ATG UE.

## 2.2. Other Tx RF requirements for ATG UE

In the following section, some further considerations on the general topics of ATG UE are provided as following.

Issue 1-1-1: The solution to solve UE access control for ATG.

During the last RAN4 meeting, there were some discussions on how to resolve the normal handheld UE access control for ATG network. First of all, ATG BS is usually deployed on top of the mountain or high towers and steer its beam towards the sky to track the aircraft in real time. In other words, it’ s most likely that TN UE will not access the ATG network or cell reselection to other normal TN cells due to the low link quality from TN network towards the ground area. In addition, based on the current RAN2 signalling, the normal UE could be also barred to access via a couple of ways (e.g. dedicated slice for ATG network and NPN for ATG network or configure the specific access class and ban the rest of it.)

Proposal 1: not need to further discuss the UE access control for ATG network and leave it up to the implementation.

Issue 3-1-1: Assumption for ATG operating mode

Given that the workload for ATG in Rel-17 has already been quite heavy, single carrier operation could be prioritized in the Rel-18 similar as Rel-16 FR1 HST and Rel-17 HST FR2 HST. For CA operation for ATG, this could be postponed to further release if there are strong market demand from interested operators.

Proposal 2: to define single carrier operation in Rel-18 for ATG and postpone the CA operation in future release.

Issue 3-1-3: subclause to capture ATG UE requirement

In the last RAN4 meeting, there were some initial discussion on how to capture the ATG UE RF requirement into TS 38.101-1. From our understanding, to have separate subclause with suffix J in the TS 38.101-1 is reasonable approach since it could be expected that the some of RF requirement for ATG UE would be different from the legacy normal NR UEs.

Proposal 3: to capture the RF requirement of ATG UE in separate subclause with suffix J in the TS 38.101-1.

In addition, in the following table 1, we share some further views on the ATG UE RF requirement and its details could be found as following:

Table 1. Summary of the proposals for ATG UE requirements

| Requirement | Band-specific | Applicability |
| --- | --- | --- |
| Transmitter Characteristics |  |  |
| General | No | Generic requirements from TN should apply. |
| Tx power | Yes | WF in RAN4#105 meeting: RAN4 can specify a range of ATG UE MOP, e.g.. 29dBm ~ FFS. ATG UE can indicate its MOP by using UE capability. The tolerance of ATG UE MOP can be ±2dB. The indicated capability takes into account the band in which the UE is operating and any NS value signalled. It is not precluded that there is no upper boundary for MOP. Not to define the specific power class for ATG UE similar as IAB-MT. Main reason/motivation not to define the specific power class for ATG UE is due to that the type of ATG CPE might be varying among different operators in different bands due to its installation restrictions on the aircraft. For example, for FR1 high bands, then antenna array could be used for ATG CPE with its beamforming capability, while this might be not feasible for some FR1 low bands due to its installation restrictions on the board in the practice. In short, we support the following option 1 which could be left for manufacturing declaration. Option 1: manufacturer declaration. Proposal 4: not to define the specific power class for ATG UE; During the previous RAN4 meetings, there were some comments whether any regulatory requirement could be referred to define power limits for ATG UE. At least from our understanding, usually the specific requirements are coming from the protection of other Aeronautical equipment on board. Regarding the MOP for ATG UE, it should be clarified that it should be based on conducted transmitter power similar as FR1 UE RF requirement instead of EIRP in FR2. In addition, it is also quite difficult to specify the EIRP/TRP based RF requirement in the current 38.101-1 spec based on the spec structure. Regarding the the range of ATG UE MOP as agreed in last RAN4 meeting, the finer granularity could allow the more flexibility of ATG UE implementation to cater for different market request. Proposal 5: to specify a range of ATG UE MOP (conducted transmitter power), e.g.. 29dBm ~ FFS with 1dB step size; |
| MPR | No | No MPR requirement defined similar as IAB-MT |
| A-MPR | Yes | No A-MPR requirement defined similar IAB-MT; |
| Configured Tx power | No | No configured Tx power defined similar as IAB-MT |
| Output Power Dynamics | No | Issue 1-2-5: Minimum output power <Way forward >: To further evaluate based on the minimum couple loss between ATG UE and BS and other parameters. Based on the initial calibration results for ATG coexistence study obtained so far, if the minimum uplink SINR could be around -8dB, then minimum output power should be -174dBm/Hz+10*log10(100*10^6)+5-8+100dB=3dBm Where 100dB refer to the minimum coupling loss extracted from the following cdf curve of coupling loss; Considering the some implementation margins, the minimum output power could be around -3dB if 6dB implementation margin is considered. Proposal 6: the minimum output power for ATG UE could be [-3dBm/100MHz] and scale with other BW. |
| Transmit signal quality |  |  |
| - Frequency error | No | To use the existing requirement defined for NTN UE in TS 38.101-5 as baseline. Basically ATG BS on the group could also indicate the required ephemeris information to ATG CPE to assist the timing and frequency compensation. Proposal 7: to use the existing requirement defined for NTN UE in TS 38.101-5 as baseline. |
| - Transmit modulation quality | No | To follow the existing requirements defined for TN UE in TS 38.101-1. |
| Transmit OFF power and Transmit ON/OFF time mask |  | We don’t see the strong necessity to relax this requirement even though the coexistence requirement with other victim UE is not so common in the practice since ATG UE is far away from the TN UEs on the ground. Proposal 8: to reuse the reuse the current requirements for Transmit OFF power and Transmit ON/OFF time mask as TS 38.101-1. |
| Output RF spectrum emissions |  |  |
| - Occupied bandwidth | No | Expect the same as TN in TS 38.101-1. |
| - Out of band emission |  |  |
| - SEM | No | This depends on the coexistence study and agreement for ACLR requirement. |
| - Additional SEM | Yes | Not applicable |
| - ACLR | No | Please see the observation in section 2.1. |
| - Spurious emission |  |  |
| - General | No | Generic requirements from TN in TS 38.101-1. should apply |
| - For UE coexistence | Yes | Not applicable since it’s far away from TN UEs on the ground. |
| Transmit intermodulation | No | Not applicable |

Conclusions

In this contribution, we shared some further views for ATG UE Tx RF requirements and proposals are made as following:

Observation 1: based on the initial simulation results in Case 2 and Case 10, it should be sufficient to reuse the legacy FR1 UE PC3 ACLR requirement 30dBc requirement for ATG UE.

Proposal 1: not need to further discuss the UE access control for ATG network and leave it up to the implementation.

Proposal 2: to define single carrier operation in Rel-18 for ATG and postpone the CA operation in future release.

Proposal 3: to capture the RF requirement of ATG UE in separate subclause with suffix J in the TS 38.101-1.

Proposal 4: not to define the specific power class for ATG UE;

Proposal 5: to specify a range of ATG UE MOP (conducted transmitter power), e.g.. 29dBm ~ FFS with 1dB step size;

Proposal 6: the minimum output power for ATG UE could be [-3dBm/100MHz] and scale with other BW.

Proposal 7: to use the existing requirement defined for NTN UE in TS 38.101-5 as baseline.

Proposal 8: to reuse the reuse the current requirements for Transmit OFF power and Transmit ON/OFF time mask as TS 38.101-1.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214461,WF on BS requirements for ATG, ZTE, Approved.

- R4-2215147,WF on UE requirements for ATG, Apple, Approved.

- R4-2214247,	Email Discussion Summary for [104-e][136] NR_ATG_UERF, CMCC, Noted

- R4-2217738	WF on ATG terminal RF requirement, Huawei, HiSilicon, Approved.

- R4-2220543	WF on ATG UE RF requirements, Huawei, approved.

- R4-2220542,WF on ATG co-existence evaluation, CMCC, Approved.

- R4-2220540,TP for TR 38.876 to capture scenarios and network layout, Ericsson, approved.