3GPP TSG-RAN WG4 Meeting#106                               R4-2301734

Athens,Greece,27th Feb – 3rd Mar,2023

Agenda item:	9.19.2.2.1

Source: 	ZTE Corporation

Title: 	Further discussion on full duplex from BS perspective

Document for:	Approval

Introduction

In RAN#94e meeting, the work item [RP-221352] on study on evolution of NR duplex operation was approved as one of Rel-18 RAN1 package. In the last RAN4 meeting, there were further discussions on the feasibility for SBFD BS and companies are encouraged to provide the details of self interference analysis for SBFD BS. In this contribution, we want to share some further inputs for it.

Discussion

## 2.1. FR1 full duplex BS

In the following section, we want to share some initial views on feasibility analysis for self interference of full duplex BS in FR1.

Table 2.1-1. self interference analysis for FR1 Medium range BS and Local area BS supporting SBFD

| FR1 |  |  |  | ZTE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Medium Range BS | Local Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | xxx dBm | 31dBm for 100MHz DL and 30dBm for 80MHz DL | 24 for 100MHz DL and 23.0dBm for SBFD DL |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | xxx dBc | 45dBc+10*log10(80/20)=51dBc | 45dBc+10*log10(80/20)=51dBc |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX | DPD | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | xxx dBc | 50-60 | 50 |
|  |  | Spatial isolation techniques used |  | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation | Spatial separation between TX/RX | Spatial separation between TX/RX |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc | N/A | N/A |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  | N/A |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  |  | 30dBm-51(ACLR)-50(antenna isolation)-30(digital cancellation/sub-band filtering)-10*log10(20*10) =-124.01dBm/100kHz 30dBm-51(ACLR)-60(antenna isolation)-20(digital cancellation/sub-band filtering)-10*log10(20*10) =-124.01dBm/100kHz Note 1: the digital IC and sub-band filter in the transmitter could achieve the same purpose for suppress the transmitter leakage into its receiver Note 2: with increasing antenna isolation, then sub-band filter attenuation or digital IC could be reduced as well. | 23dBm-51(ACLR)-50(antenna isolation)-20(digital cancellation)-10*log10(20*10) =-121.01/100kHz |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | xxx dBc | N/A | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | xxx dBc | N/A | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) | 30dBc analog filter for Rx sub-band | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | xxx dBc |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  |  | 30dBm-50(antenna isolation)-30 (sub-band filter) = -50dBm equal to 50dBm for ACS requirement and -38dBm for IBB requirements of 20MHz reception | 23dBm-50(antenna isolation)=-27dBm Which is a bit higher than in-band blocking requirement of LA -35dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc | 46dBc for ACS requirement | 46dBc |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX | 30dBc for analog filter of Rx receiver | N/A |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  | TBD | -2dBm |
|  |  |  | Rx IM3 contribution (dBm) |  | =3*Pinf-2*IIP3 | 3*(-35)-2(-2)-10*log10(20*10)=-124.01dBm/100kHz |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | xxx dBm | 30dBm-50(antenna isolation)-30(sub-band filter)-46dB (ACS)-10*log10(20*10) = -118.01dBm/100kHz 31dBm-60(antenna isolation)-30(sub-band filter)-46dB (ACS)-10*log10(20*10) = -128.01dBm/100kHz | 23dBm-50(antenna isolation)-46dB (ACS)-10*log10(20*10)-20= -116.01dBm/100kHz |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc | N/A | N/A |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc | N/A | N/A |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | xxx dBc | 20dBc | 20dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | xxx dBc | [120.9dBss] | 110.9dB |
| Noise floor ⑩dBm |  |  |  | xxx dBm/CBW | 10dB | 13dB |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | xxx dBm | -174dBm/Hz+10*log10(100*10^3)+10dB-6dB= -120dBm/100kHz | -174dBm/Hz+10*log10(100*10^3)+13dB-6dB= -117dBm/100kHz |
| Required RSIC budget (①-⑪dBc) |  |  |  | xxx dBc | 30dBm-10*log10(80*10)-(-120dBm/100kHz) = 120.9dB for single antenna 36dBm-10*log10(80*10)-(-120dBm/100kHz) = 126.9dB for four antenna | 23dBm-10*log10(80*10)-(-117dBm/100kHz) = 110.9dB for single antenna 29dBm-10*log10(80*10)-(-117dBm/100kHz) = 116.9dB for four antenna |
| SBFD configuration |  |  |  |  | 80MHz DL, 20MHz | 80MHz DL, 20MHz |
| Guardband assumption (if exist) |  |  |  |  | Left up to the implementation. This highly depend on how to achieve the ACS performance and Q factor of analog filter if needed | Left up to the implementation. This highly depend on how to achieve the ACS performance and Q factor of analog filter if needed |
| bandwidth over which suppression is achieved |  |  |  |  | 80MHz DL and 20MHz UL | 80MHz DL and 20MHz UL |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

## 2.2. Other

Agreement:

- FFS the effect of clutter on achievable RSIC performance.

- In last RAN4 meeting, there were some initial discussion on the clutter impact on the RSIC performance, from our understanding, this mainly have the impacts on the digital IC capability on the channel estimation of carrier leakage signal.

- Proposal 1: to consider the clutter impact on digital IC in RSIC performance if needed.

Agreement:

- FFS gNB receiver saturation, non-linearity, and AGC model is based on peak input power.

- The gNB receiver non-saturation is extremely important to make sure that the receiver could work normally. Usually the receiver saturation is determined by the in-band blocking signal power level to design receiver dynamic range. Even though in the practice, the the receiver saturation level is a bit higher than in-band blocking level due to the implementation margin left, however from the standardization perspective, we prefer to keep the in-band blocking level as baseline and companies could share their capability on the in-band blocking capability or gNB receiver saturation which is supposed to be higher than IBB requirements.

- Proposal 2: to consider in-band blocking level as baseline for gNB receiver saturation, non-linearity and higher capability should depend on vendors’ declaration.

Conclusions

In this contribution, we want to share further feasibility analysis on the MR SBFD and LA SBFD BS and proposals are made as following:

- Proposal 1: to consider the clutter impact on digital IC in RSIC performance if needed.

- Proposal 2: to consider in-band blocking level as baseline for gNB receiver saturation, non-linearity and higher capability should depend on vendors’ declaration.

References

- RP-221352, Study on evolution of NR duplex operation, CMCC, Samsung, Approved

- TR38.828,Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR; (Release 16)

- R4-2214377,WF on feasibility study from RF perspective, Samsung, approved.

- R4-2214376,Reply LS on interference modelling for duplex evolution, Samsung, CMCC, Qualcomm, approved.

- R4-2217464, WF on SBFD feasibility study and RF impact: BS aspects, Samsung, approved.

- R4-2220244 WF for the feasibility from BS aspect, Samsung, approved.