3GPP TSG-<TSG/RAN4> Meeting # 106	R4-2301748

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR | 0018 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.176-1: Maintenance for IAB-MT performance requirement R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2023-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The term of ‘UE’ is still used in some places, which is not correct since this specification is for IAB-MT performance requirement, rather than ‘UE’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct in the corresponding places where the requirements are for IAB-MT, not UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement for IAB-MT will contain content related to UE while there is no UE in the requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6, 8.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<start of the change 1>

4.6	Manufacturer declarations

The following IAB type 1-H declarations listed in table 4.6-1, when applicable to the IAB-DU or IAB-MT under test, are required to be provided by the manufacturer for the conducted requirements testing of the IAB type 1-H. Declarations may be provided independently for IAB-MT and IAB-DU.

For the IAB type 1-H declarations required for the radiated requirements testing, refer to TS 38.176-2 [3].

Table 4.6-1 Manufacturer declarations for IAB-type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | IAB-DU type 1-H | IAB-MT type 1-H |
| D.1 | IAB requirements set | Declaration of one of the IAB requirement's set as defined for IAB type 1-H. | x | x |
| D.2 | IAB class | IAB class of the IAB, declared as Wide Area IAB, Medium Range IAB, or Local Area IAB. | x | x |
| D.3 | Operating bands and frequency ranges | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the IAB-DU or IAB-MT and if applicable, frequency range(s) within the operating band(s) that the IAB can operate in. Declarations shall be made per TAB connector for IAB type 1-H. | x | x |
| D.4 | Spurious emission category | Declare the IAB-DU or IAB-MT spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | x | x |
| D.5 | Additional operating band unwanted emissions | The manufacturer shall declare whether the IAB-DU or IAB-MT under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.4.5 apply. | x | x |
| D.6 | Co-existence with other systems | The manufacturer shall declare whether the IAB-DU or IAB-MT under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. | x | x |
| D.7 | Co-location with other IAB | The manufacturer shall declare whether the IAB-DU or IAB-MT under test is intended to operate co-located with IAB of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. | x | x |
| D.8 | Single band connector or multi-band connector | Declaration of the single band or multi-band capability of single band connector(s) or multi-band connector(s), declared for every connector. | x | x |
| D.9 | Contiguous or non-contiguous spectrum operation support | Ability to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier. Declared per single band connector or multi-band connector, per operating band. | x | x |
| D.10 | void | void |  |  |
| D.11 | Maximum IAB RF Bandwidth | Maximum IAB RF Bandwidth in the operating band for single-band operation. Declared per supported operating band, per TAB connector for IAB type 1-H. (Note 2) | x | x |
| D.12 | Maximum IAB RF Bandwidth for multi-band operation | Maximum IAB RF Bandwidth for multi-band operation. Declared per supported operating band, per TAB connector for IAB type 1-H. | x | x |
| D.13 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (D.27). | x | x |
| D.14 | NR supported channel bandwidths and SCS | NR supported SCS and channel bandwidths per supported SCS. Declared per supported operating band, per TAB connector for IAB type 1-H. | x | x |
| D.15 | CA only operation | Declaration of CA-only operation (with equal power spectral density among carriers) but not multiple carriers, declared per operating band per TAB connector for IAB type 1-H. | x | x |
| D.16 | Single or multiple carrier | Capable of operating with a single carrier (only) or multiple carriers. Declared per supported operating band, per TAB connector for IAB type 1-H. | x | x |
| D.17 | Maximum number of supported carriers per operating band in single band operation | Maximum number of supported carriers per supported operation band in single band operation. Declared per supported operating band, per TAB connector for IAB type 1-H. (Note 2) | x | x |
| D.18 | Maximum number of supported carriers per operating band in multi-band operation | Maximum number of supported carriers per supported operation band in multi-band operation. (Note 2) | x | x |
| D.19 | Total maximum number of supported carriers in multi-band operation | Maximum number of supported carriers for all supported operating bands in multi-band operation. Declared for all connectors (D.18). | x | x |
| D.20 | Other band combination multi-band restrictions | Declare any other limitations under simultaneous operation in the declared band combinations (D.38) for each multi-band connector which have any impact on the test configuration generation. Declared for every multi-band connector. | x | x |
| D.21 | Rated carrier output power (Prated,c,AC, or Prated,c,TABC) | Conducted rated carrier output power, per single band connector or multi-band connector. Declared per supported operating band, per TAB connector for IAB type 1-H. (Note 1, 2) | x | x |
| D.22 | Rated total output power (Prated,t,AC, or Prated,t,TABC) | Conducted total rated output power. Declared per supported operating band, per TAB connector for IAB type 1-H. For multi-band connectors declared for each supported operating band in each supported band combination. (Note 1, 2) | x | x |
| D.23 | Rated multi-band total output power, Prated,MB,TABC | Conducted multi-band rated total output power. Declared per supported operating band combinations, per multi-band connector. (Note 1) | x | x |
| D.24 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a IAB in a particular operating band with transmission on all TAB connectors supporting the operating band. | x | x |
| D.25 | Maximum supported power difference between carriers | Maximum supported power difference between carriers. Declared per supported operating band, per TAB connector for IAB type 1-H. (Note 3). | x | x |
| D.26 | Maximum supported power difference between carriers is different operating bands | Supported power difference between any two carriers in any two different supported operating bands. Declared per supported operating band combination, per multi-band connector. | x | x |
| D.27 | Operating band combination support | List of operating bands combinations supported by single-band connector(s) and/or multi-band connector(s) of the IAB. Declared per TAB connector for IAB type 1-H. | x | x |
| D.28 | void | void |  |  |
| D.29 | Intra-system interfering signal declaration list | List of single band connector(s) or multi-band connector(s) for which an intra-system interfering signal level is required to be declared. Declaration is required if the intra-system interfering signal level is larger than the co-location interfering signal level. | x | x |
| D.30 | Intra-system interfering signal level | The interfering signal level in dBm. Declared per supported operating band, per TAB connector for IAB type 1-H covered by D.29. | x | x |
| D.31 | TAE groups | Set of declared TAB connector beam forming groups on which the TAE requirements apply. All TAB connectors belong to at least one TAB connector beam forming group (even if it's a TAB connector beam forming group consisting of one connector). The smallest possible number of TAB connector beam forming groups need to be declared such that there is no TAB connector not contained in at least one of the declared TAB connector beam forming groups. Declared per supported operating band. | x |  |
| D.32 | Equivalent connectors | List of TAB connector of IAB type 1-H, which have been declared equivalent. Equivalent connectors imply that the TAB connector of IAB type 1-H, are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the TAB connector of IAB type 1-H are identical and the transmitter unit and/or receiver unit driving the TAB connector of IAB type 1-H are of identical design. | x | x |
| D.33 | TAB connector RX min cell group | Declared as a group of TAB connectors to which RX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for receiving a cell when the IAB type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. | X | x |
| D.34 | TAB connector TX min cell group | Declared group of TAB connectors to which TX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for transmitting a cell when the IAB type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. | x | x |
| D.35 | void | void |  |  |
| D.36 | Relation between supported maximum RF bandwidth, number of carriers and Rated total output power | If the rated total output power and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated total output power; - The reduced total output power at the maximum number of supported carriers. | x | x |
| D.37 | TAB connectors used for performance requirement testing | To reduce test complexity, declaration of a representative (sub)set of TAB connectors to be used for performance requirement test purposes. At least one TAB connector mapped to each demodulation branch is declared. | x | x |
| D.38 | Inter-band CA | Band combinations declared to support inter-band CA (per CA capable multi-band connector(s), as in D.15). Declared for every multi-band connector which support CA. | x | x |
| D.39 | Intra-band contiguous CA | Bands declared to support intra-band contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D.15). Declared per TAB connector for IAB type 1-H. | x | x |
| D.40 | Intra-band non-contiguous CA | Bands declared to support intra-band non-contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D.15). Declared per or TAB connector for IAB type 1-H.. | x | x |
| D.41 | void | void |  |  |
| D.42 | void | void |  |  |
| D.43 | void | void |  |  |
| D.IAB-1 | Same RF implementation. | Declaration whether IAB-MT and IAB-DU have same RF implementation. | x | x |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type as specified in TS 38.211 [9], i.e., type A, type B or both. | x |  |
| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s), i.e., pos0, pos1 or both. | x |  |
| D.102 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [9], i.e., format 0, format 1, format 2, format 3, format 4. | x |  |
| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [9], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [9], i.e., 15 kHz, 30 kHz or both. | x |  |
| D.104 | Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. | x |  |
| D.105 | Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. | x |  |
| D.106 | PUCCH multi-slot | Declaration of multi-slot PUCCH support. | x |  |
| D.107 | UL CA | For the highest supported SCS, declaration of the carrier combination with the largest aggregated bandwidth. If there is more than one combination, the carrier combination with the largest number of carriers shall be declared. | x |  |
| D.108 | Modulation order | Declaration of the supported modulation order, i.e. QPSK, 16QAM, 64QAM | x |  |
| D.109 | DFT-s-OFDM | Declaration of the supported of DFT-s-OFDM, i.e. supported or not supported. | x |  |
| D.200 | 256QAM for PDSCH for FR1 | Declaration of the supported of 256QAM modulation scheme for PDSCH for FR1, i.e. supported or not supported. |  | x |
| D.201 | Maximum number of ports across all configured NZP-CSI-RS resources per CC | Declaration of the maximum number of ports across all configured NZP-CSI-RS resources per CC, i.e. 2, 4, 8, 12, 16, 24, 32, 40, 48 … ,256 or not supported. |  | x |
| D.202 | Maximum number of PDSCH MIMO layers | Declaration of the the maximum number of spatial multiplexing layer(s) supported by the IAB-MT for DL reception, i.e. 2, 4, 8 or not supported. |  | x |
| NOTE 1: If an IAB-DU or IAB-MT is capable of 256QAM DL operation then two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions and the other declaration is applicable when not configured for 256QAM transmissions. NOTE 2: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. When separately declared, they shall still use the same declaration identifier. NOTE 3: The power difference is declared at highest rated output power. |  |  |  |  |

<unchanged part omitted>

<end of the change 1>

<start of the change 2>

8.2.3.3	Reporting of Precoding Matrix Indicator (PMI)

8.2.3.3.1	Definition and applicability

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the IAB-MT reported PMI compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated with equal probability of each applicable i1 and i2 combination and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements for transmission mode 1 with higher layer parameter codebookType set to 'typeI-SinglePanel' are specified in terms of the ratio:

In the definition of , for 4TX and 8TX PMI requirements, is 90 % of the maximum throughput obtained at  using the precoders configured according to the IAB-MT reports, and is the throughput measured at with random precoding.

<unchanged part omitted>

<end of the change 2>

<start of the change 3>

8.2.3.3.4.2	Test procedure

1)	Connect the IAB-MT tester generating the wanted signal and AWGN generators to all IAB-MT TAB connectors for diversity reception via a combining network as shown in annex D.5 and D.6.

2)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 8.2.3.3.4.2-1.

Table 8.2.3.3.4.2-1: AWGN power level at the IAB-MT input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 30 kHz | 40 | -77.2 dBm / 38.16MHz |

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A and the test parameters in table 8.2.3.3.4.2-2.

Table 8.2.3.3.4.2-2: Test parameters for testing PMI reporting

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 | 40 |
| Subcarrier spacing |  | kHz | 30 | 30 |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U | 7D1S2U, S=6D:4G:4U |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 4 (N1,N2) = (2,1) | High XP 8 x 4 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Annex J.3 | As specified in Annex J.3 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 8 |
|  | CDM Type |  | FD-CDM2 | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 4, (0,-) | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (5,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 | 10/1 |
| ReportConfigType |  |  | Periodic | Periodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Sub-band Size |  | RB | 16 | 16 |
| csi-ReportingBand |  |  | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  | slot | 10/9 | 10/9 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 | 0x FFFF |
|  | RI Restriction |  | 00000001 | 00000010 |
| CQI/RI/PMI delay |  | ms | 5.5 | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Measurement channel |  |  | M-FR1-A.3.5-5 | M-FR1-A.3.5-6 |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the IAB-MT reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4) for Test 1 or slot#(n-6) for Test 2, this reported PMI cannot be applied at the gNB downlink before slot#(n+4) for Test 1 or slot#(n+6) for Test 2 respectively. Note 4: Randomization of the principle beam direction shall be used as specified in Annex F.2.4.2.4. |  |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

5)	Adjust the equipment so that required SNR specified in clause 8.2.3.3.1 is achieved at the IAB-MT input.

6)	For each test specified in table 8.2.3.3.4.2-2 applicable for the IAB-MT, calculate .

<unchanged part omitted>

<end of the change 3>