3GPP TSG-RAN WG4 Meeting #106	R4-2301858

Athens, Greece, February 27 – March 3, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	BS RF requirements for Enhanced RedCap

Agenda item:	9.30.3

Document for:	Discussion

Introduction

In this contribution, we provide our view on the impact of Rel-18 enhanced RedCap [1] to BS RF requirements.

Discussion

Core objectives

According to the core objectives of the WID [1], as shown below, RAN4 should work on power saving and further reducing the UE complexity in FR1.

Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

Impact to BS RF requirements

As shown, the revised WID in RP-223544 only foresees UE BB bandwidth reduction within RF bandwidth of 20 MHz. This will require to study if new BS demodulation requirements for PUSCH are needed for 5 MHz BW for RedCap, but this is related to BB rather than RF requirements. Based on this, following observation can be deduced.

As for Rel-17 RedCap, there is no impact to BS RF requirements from Rel-18 enhanced RedCap.

Following proposals are submitted:

RAN4 to agree there is no impact to BS RF requirements from Rel-18 enhanced RedCap.

The agenda item shall be removed.

Conclusion

In this contribution we have provided our view on the impact of Rel-18 enhanced RedCap to BS RF requirements.

Following observation is made.

- As for Rel-17 RedCap, there is no impact to BS RF requirements from Rel-18 enhanced RedCap.

Following proposals are submitted:

- RAN4 to agree there is no impact to BS RF requirements from Rel-18 enhanced RedCap.

The agenda item shall be removed.

- References

- RP-223544, Revised WID: "Revised WID on Enhanced support of reduced capability NR devices", Ericsson, RAN#98-e