3GPP TSG-RAN WG4 # 106	R4-2301863

Athens, Greece, February 27 – March 3, 2023

Agenda item:	9.18.2

Source:	MediaTek Inc.

Title:	Discussion on Application Layer Data Throughput

Document for:	Discussion

# Introduction

In RAN4#105, the Way Forward document was approved [1]. We provide discussion with proposals for the Rel-18 work in this contribution. In addition, we have provided simulation results in separate document [2].

# Discussion

# 2.1 Background

In the following Table 1-1 some main parameters of baseline simulation assumptions of the WF document [1] are listed.

Table 1-1: Simulation assumptions for Absolute Physical Layer Throughput alignment with link adaptation.

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
| CQI-table |  |  | Table 2 | Table 2 | Table 2 |
| Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| The Outer Loop Link Adaptation (OLLA) |  |  | Disabled (Follow CQI) |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

# 2.2 Observations and proposals

In Rel-17 study number of CSI-RS ports and maximum rank was limited to 2. In practical network deployments higher number of antenna ports is used with maximum rank of 4. We are proposing to extend simulation study with higher order codebooks with maximum rank of 4. We have added Rank4 tests as Test 4 and Test 5 in our simulation results.

Observation #1: In Rel-17 study number of CSI-RS ports and maximum rank are limited to 2.

Proposal #1: Extend study to number of CSI-RS ports to 4 or 8 and maximum rank to 4.

- Test 4: FR1 FDD, SCS/CBW=15kHz/10MHz, 4Tx, 4Rx, Rank4

- Test 5: FR1 TDD, SCS/CBW=30kHz/40MHz, 4Tx, 4Rx, Rank4, TDD UL/DL configuration: 7D1S2U

## Test scope

RAN4 use the same as the scope of the ATP SI captured in 5.10.3 in TR37.901-5 and FR2-2 is with less priority.

- Test 1: FR1 FDD, SCS/CBW=15kHz/10MHz, 2Tx, 2Rx/4Rx

- Test 2: FR1 TDD, SCS/CBW=30kHz/40MHz, 2Tx, 2Rx/4Rx, TDD UL/DL configuration: 7D1S2U

- Test 3: FR2-1 TDD, SCS/CBW=120kHz/100MHz, 2Tx, 2Rx, TDD UL/DL configuration: DDSU

In Rel-17 study maximum number of HARQ transmission was limited to 1 disabling HARQ retransmissions. In practical network deployments HARQ retransmissions are enabled. Also, this test case will be implemented as application layer throughput test where we should take case physical layer throughput is consistent with application layer throughput. This can be better guaranteed with L1 HARQ retransmission. Therefore, we are proposing to extend simulation study with HARQ retransmissions enabled.

Observation #2: In Rel-17 study maximum number of HARQ transmission is 1.

Proposal #2: Extend study to maximum number of HARQ transmissions to 4.

In Rel-17 study reported CQI is followed. In practical network deployments the outer loop link adaptation (OLLA) is used to enforce network's own link quality target. Tests utilizing OLLA would serve also as functional tests to check that UE CQI reporting tolerates OLLA, and that the overall control loop remains stable. There already exists many CQI and RI reporting functional tests but not a single test with OLLA. Therefore, we are proposing to extend study with OLLA. OLLA implementation is not defined by 3GPP and infra vendors have freedom of proprietary design. However, for simulation results alignment purpose we propose to define some OLLA implementation to be used in simulations. Public OLLA implementation definition also would set good base for test equipment.

Observation #3: In Rel-17 study the outer loop link adaptation (OLLA) is not used and reported CQI is followed.

Proposal #3: Extend study to use the outer loop link adaptation (OLLA).

Proposal #4: Define the outer loop link adaptation (OLLA) design for simulation results alignment.

## Test parameters and simulation assumptions

RAN4 use the same assumption in the following aspects as in the Rel-17 SI on feasibility of introducing requirements for physical layer TP requirements with link adaptation for application layer throughput captured in clause 5.10.3 in TR37.901-5

- Maximum rank and CSI-RS port number i.e., maximum rank 2 with 2 CSI-RS ports

Note) It’s not precluded to further discuss the possibility of extension the study with rank 4 and number of CSI-RS ports to 4 or 8 in future release.

- “Disabling OLLA” for physical layer TP requirements as a baseline

. Interested companies can bring further analysis on the OLLA impact

- Reuse channel models in SI phase

. FR1: Table 5.10.3-1 in TR 37.901-5, i.e., TDLA30-5

. FR2: Table 5.10.3-1 in TR 37.901-5, i.e., TDLA30-35

Note) For FR1, other options such as high-doppler not precluded pending on further evaluation

- Maximum number of HARQ transmission:

. ‘Set the maximum number of HARQ transmission to 1’ is a baseline assumption

Note) Further analysis the TP difference between physical layer and upper layer with re-Transmission disabled are not precluded.

- Reuse the same test parameters as defined in Table 5.10.3-1 in TR 37.901-5 including (not precluding parameters in 1.2 above)

  - MMSE-IRC receiver type

  - CQI Table and Codebook related configurations

  - Configure CQI table 2 for FR1 and CQI table 1 for FR2

  - Configure Type I Single-Panel Codebook

  - Not configure CodebookSubsetRestriction

  - CSI delay

- 6 ms for FR1 FDD, 5.5 ms for FR1 TDD, 1.375 ms for FR2-1 TDD

If 4 antenna port and Rank 4 tests would be added, then the maximum throughput definition needs to be updated accordingly.

Phy Layer TP test metric

- Average SNR of impairments results to achieve T% of maximum throughput + X dB margin, with Gspan = [2.5] dB and X = [0.5] dB

- The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank 2 for 2Rx/4Rx UE

We have provided simulation results in separate document [2]. In additional to baseline assumption simulations, this document also contains simulation results with extended scope to 4 antenna ports with Rank 4, HARQ retransmissions and OLLA. For Rank 4 simulations we have used extended SNR range [0:2:30] to enable usage or higher ranks. We wanted to share lot of results for proactive performance alignment and to alleviate any worries companies may have of extensions.

Companies are encouraged to provide (potentially updated) simulation results for next meeting for alignment.

- Alignment shall be done based on all provided results for metrics as did in Rel-17 SI, i.e. including throughput v.s. SNR, BLER, median CQI and median RI with SNR range [0:2:20] for FR1 and [0:2:16] for FR2-1.

Observation #4: Companies should have provided updated simulation results in this meeting.

Proposal #5: Make decision on SNR point selection based on updated simulation results.

Proposal #6: Make decision on Test metric based on updated simulation results.

Test SNR point selection criteria

- Option 1: 2 SNR points for each test

  - Option 1A: Cover both low and higher modulation order/layer

  - Option 1B:

    - For 2Rx: Choose one in rank 1 and one in rank 2.

    - For 4Rx: Choose both T points in rank 2 region, one in the medium SNR away from rank transition region, and one close to 20dB (peak SNR).

  - Option 1C:

    - For higher SNR test points, reuse the existing RI test cases SNR=20dB for FR1, SNR=16dB for FR2

    - For lower SNR points, set SNR=6dB

- Option 2: 1 SNR point for each test

  - Option 2A: Median SNR value that RI changes from Rank 1 to Rank 2

Test metric

- Option 1:

  - For FR1: Test the SNR at 10% and 40% max TP.

  - For FR2: Test the SNR at 10% and (35% or 40%) max TP.

- Option 2:

  - For FR1 FDD 2Rx, T% = 40%;

  - For FR1 FDD 4Rx, T% = 60%;

  - For FR1 TDD 2Rx, T% = 40%;

  - For FR1 TDD 4Rx, T% = 60%;

  - For FR2 2Rx, T% = 40%;

- Option 3: T% = 40% for all tests

- Other options are not precluded

Observation #5: In the current specification there are either demodulation tests or CSI tests but not combined test demodulation test with link adaptation.

Proposal #7: We slightly prefer Option 2 as measurement criteria of Throughput to SNR is more like demodulation tests.

Section for ATP specification

- Option 1: Specify the absolute physical layer throughput requirements with link adaptation under the CSI reporting requirements in TS38.101-4, i.e., clause 6.x for FR1 and clause 8.x for FR2

- Option 2: Create new sub-clause 5.6 and new sub-clause 7.6 for ATP requirements

As already noted in Observation 4, companies should have provided updated simulation results in this meeting. It is assumed that modem performance has improved over generations and the latest simulation should match the latest generation performance. Therefore, performance requirements based on the latest results may be difficult to pass for the early generation products.

Proposal #8: We prefer Option 2 as requirements defined with the latest result may be difficult to pass for the early generation products.

Applicability and release independency

- Option 1: The requirement with link adaptation should be applicable for all NR UEs without any new applicability   rules, and the requirement should be release independent from Rel-15

- Option 2: The requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15 considering that companies are providing the latest results.

# Conclusion

In this paper we provided the view on the application layer data throughput. The following observations and proposals are made:

Observation #1: In Rel-17 study number of CSI-RS ports and maximum rank are limited to 2.

Proposal #1: Extend study to number of CSI-RS ports to 4 or 8 and maximum rank to 4.

Observation #2: In Rel-17 study maximum number of HARQ transmission is 1.

Proposal #2: Extend study to maximum number of HARQ transmissions to 4.

Observation #3: In Rel-17 study the outer loop link adaptation (OLLA) is not used and reported CQI is followed.

Proposal #3: Extend study to use the outer loop link adaptation (OLLA).

Proposal #4: Define the outer loop link adaptation (OLLA) design for simulation results alignment.

Observation #4: Companies should have provided updated simulation results in this meeting.

Proposal #5: Make decision on SNR point selection based on updated simulation results.

Proposal #6: Make decision on Test metric based on updated simulation results.

Observation #5: In the current specification there are either demodulation tests or CSI tests but not combined test demodulation test with link adaptation.

Proposal #7: We slightly prefer Option 2 as measurement criteria of Throughput to SNR is more like demodulation tests.

Proposal #8: We prefer Option 2 as requirements defined with the latest result may be difficult to pass for the early generation products.

# Reference

- R4-2220276, “Way forward on physical layer throughput requirements”, Intel

- R4-2xxxxxx, “Simulation results for Application Layer Data Throughput”, MediaTek