3GPP TSG-RAN WG4 # 106	R4-2301864

Athens, Greece, February 27 – March 3, 2023

Agenda item:	9.18.2

Source:	MediaTek Inc.

Title:	Simulation results for Application Layer Data Throughput

Document for:	Discussion

# Introduction

In RAN4#105, simulation assumptions are listed in discussion summary document [1]. For RAN4#106, companies are encouraged to provide updated simulation results. In following Table 1-1 baseline simulation assumptions are listed.

Table 1-1: Simulation assumptions for Absolute Physical Layer Throughput alignment with link adaptation in [1].

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
| CQI-table |  |  | Table 2 | Table 2 | Table 2 |
| Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| The Outer Loop Link Adaptation (OLLA) |  |  | Disabled (Follow CQI) |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

# Simulations

In Chapter 2.1 we introduce some extensions to baseline simulations assumptions. Chapter 2.2 contains all simulations results.

## 2.1 Extended simulation parameters

In addition to baseline simulation assumptions, we want to provide results with extended configuration options. We are going to extend baseline assumptions for Tests 1 to 3 with HARQ retransmissions shown in Table 2-1. Furthermore, we are going to extend baseline assumptions for Tests 1 to 3 with Outer Loop Link Adaptation (OLLA) shown in Table 2-2. In Table 2-3 new Tests 4 and 5 are introduced that use 4 antenna ports with maximum Rank 4. These tests are then further extended with HARQ retransmissions with Table 2-4 parameters and with OLLA with Table 2.5 parameters.

Table 2-1: Simulation assumption extensions of Test 1-3 to Table 1-1 with HARQ re-TX.

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| Maximum number of HARQ transmission |  | 4 | 4 | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |  |  |
| The Outer Loop Link Adaptation (OLLA) |  | Disabled (Follow CQI) |  |  |

Table 2-2: Simulation assumption extensions of Test 1-3 to Table 1-1 with OLLA.

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| Maximum number of HARQ transmission |  | 1 | 1 | 1 |
| Redundancy version coding sequence |  | N/A |  |  |
| The Outer Loop Link Adaptation (OLLA) |  | Enabled |  |  |

Table 2-3: Simulation assumption extensions of new Test 4-5 baseline.

| Parameter |  | Unit | Test 4 | Test 5 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Duplex Mode |  |  | FDD | TDD |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  | ULA Low 4x4 | ULA Low 4x4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
| CQI-table |  |  | Table 2 | Table 2 |
| Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel |
| Maximum number of HARQ transmission |  |  | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |
| The Outer Loop Link Adaptation (OLLA) |  |  | Disabled (Follow CQI) |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |

Table 2-4: Simulation assumption extensions of Test 4-5 to Table 2-3 with HARQ re-TX.

| Parameter | Unit | Test 4 | Test 5 |
| --- | --- | --- | --- |
| Maximum number of HARQ transmission |  | 4 | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |  |
| The Outer Loop Link Adaptation (OLLA) |  | Disabled (Follow CQI) |  |

Table 2-5: Simulation assumption extensions of Test 4-5 to Table 2-3 with OLLA.

| Parameter | Unit | Test 4 | Test 5 |
| --- | --- | --- | --- |
| Maximum number of HARQ transmission |  | 1 | 1 |
| Redundancy version coding sequence |  | N/A |  |
| The Outer Loop Link Adaptation (OLLA) |  | Enabled |  |

## 2.2 Simulation results

In this chapter we show following simulations results

- Figure 2-1: Test 1 FR1 FDD 2x2

- Figure 2-2: Test 1 FR1 FDD 2x4

- Figure 2-3: Test 1 FR1 TDD 2x2

- Figure 2-4: Test 1 FR1 TDD 2x4

- Figure 2-5: Test 1 FR2

- Figure 2-6: Test 1 FR1 FDD 4x4

- Figure 2-7: Test 1 FR1 TDD 4x4

Figure 2-1: Test 1 FR1 FDD 2x2 simulation results.

Figure 2-2: Test 1 FR1 FDD 2x4 simulation results.

Figure 2-3: Test 2 FR1 TDD 2x2 simulation results.

Figure 2-4: Test 2 FR1 TDD 2x4 simulation results.

Figure 2-5: Test 3 FR2 2x2 simulation results.

Figure 2-6: Test 4 FR1 FDD 4x4 simulation results.

Figure 2-7: Test 5 FR1 TDD 4x4 simulation results.

# Conclusion

In the contribution, we provide simulation results, including physical layer throughout, median RI, median CQI based on median RI and BLER with link adaptation for simulation result collection and alignment to define requirements in RAN4.

# Reference

- R4-2220276, “Way forward on physical layer throughput requirements”, Intel