3GPP TSG-RAN WG4 Meeting #106	R4-2301881

Athens, Greece, 27th February – 3rd March 2023

Agenda Item:	9.4.2

Source:	Ericsson

Title:	TP to TR 38.877: General paragraph for feasibility aspects

Document for:	Approval

# 1	Introduction

During RAN4#105, a work split was proposed for sub-sections of the feasibility section in the FR2 wideband BS study TR. This TP proposes text for section 5.1 (General) according to the work split.

# 2	Text Proposal

## 5.1	General

According to the definition in section 6.1, a multi-band FR2-1 BS is a BS that transmits in two or more FR2-1 bands using common active components. In this section, the feasibility of an FR2-1 multi-band BS is considered. The feasibility is examined taking into account technology challenges and emerging solutions. The evaluation considers whether the proposed set of FR2-1 requirements is likely to be achievable with present or future implementation possibilities. The study does not aim to conclude on whether a multi-band FR2 solution is more optimal or effective (considering complexity, power, weight etc.) than other possibilities, for example mounting individual radio panels for each band within a BS enclosure (which would be a multiple single band implementation).

The feasibility evaluation considers as a first possibility multiband operation for bands within the same Frequency Band Group (FBG). That is, combinations of bands within 24-30GHz and combinations of bands within 37-40GHz. As a second priority, the feasibility of a solution covering the complete FR2-1 range from 24-43GHz is considered.

Aspects that impact the implementation of an FR2-1 multi-band BS and potentially the feasibility of meeting requirements include:

- Antenna array construction and properties across the frequency range

- Beamforming architecture

- Losses in routing the RF signals to antenna array components, if applicable

- Analogue filtering necessity and solutions in the inter-RF gap

- Linearity and RF performance of the transmitter across the frequency range

- Need for and possibility of transmitter linearization considering the large frequency range

- Linearity and noise figure for the receiver across the frequency range

- Receiver architecture possibilities considering the large bandwidth

- ADC and DAC feasible bitwidth and sampling rates

- Digital data transport volume and speed considering the large bandwidth

The above aspects are elaborated in the following sections.