3GPP TSG-RAN WG4 Meeting #106	R4-2301887

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	6.2.6.1.1

Source:	Ericsson

Title:	Simulation results for PDSCH demodulation in 52.6 – 71 GHz band.

Document for:	Information

# 1	Introduction

In this paper and based on the progress summarized in WF [1], we update our simulation results on the PDSCH performance in the frequency range 52.6 GHz to 71 GHz. We carried out new simulations for all the agreed test cases using the agreed channel models.

Extended maximum Testable SNR for reduced allocation The extended table below is agreed in RAN4#105, containing the Maximum DL testable SNR for band n263 used for the definition of the requirements. [ ] are included to keep alignment with the Table 7.2.3-2 in 38.884, v 18.2.0 used as reference. CBW (MHz); Num RBs / SCS (KHz); Test method. IFF. Single band UE; 100; 66 / 120; [9.8]. 32 / 120; [13.2]. 400; 66 / 480; [2.6]. 32 / 480; [6.6]. 20 / 480; [8.9]. RB allocation for 120kHz/100MHz for MCS 17 for 70% requirements - Tentative Agreement from offline discussion: Considering that most of the companies submitted alignment and impairment results for both options of RB allocation under consideration ([66, 32] RBs), companies agree to introduce both requirements, subject to the following applicability rule: - For UE supporting FR2-2, the PDSCH Demod test for 120kHz/100MHz with MCS17 (64QAM, 0.43) with 32RBs allocation applies only if the correspondent requirement cannot be tested using 66 RBs due to testable SNR restrictions; Issue 1-2-2: RB allocation for 480kHz/400MHz for MCS 13 for 70% requirements Agreement: 20 RBs; Tentative PDSCH requirements definition - Tentative Agreement from offline discussion: Based on the simulation results collected in this meeting, the table below is extracted from R4-2218306. For the highlighted requirement values, the alignment simulation results span was over 2.5 dB. To define at least tentative requirements for these tests, a compromise has been reached by excluding the 2 lowest reported SNRs, considered as outliers, and choosing the lowest SNR reported among the 3 reported values left. The span for non-highlighted requirements were averaged across all reported values. Duplex CBW/SCS; Antenna configuration; RB allocation; MCS and rank; Channel model; % of Peak Throughput; Tentative Requirement; Max Testable SNR (Info). TDD 100MHz/120kHz; 2x2 Low; 66; MCS4, Rank 1; TDLA30-650; 70; 1.0; 9.8. 66; MCS13, Rank 1; TDLA30-200; 70; 8.9; 9.8. 66; MCS13, Rank 1; TDLA30-650; 30; 2.6; 9.8. 66; MCS17, Rank 1; TDLD30-200; 70; 11.4; 9.8. 32; MCS17, Rank 1; TDLD30-200; 70; 11.5; 13.2. TDD 400MHz/480kHz; 2x2 Low; 66; MCS4, Rank 1; TDLA10-200; 70; 0.9; 2.6. 20; MCS13, Rank 1; TDLD10-200; 70; 8.0; 8.9. Simulation alignment for RAN4#106 - Tentative Agreement from offline discussion: The companies have agreed to further check their simulations and submit the following results to the next meeting, to facilitate the alignment effort and to try to reduce the span in the results: - Demod simulation results for alignment, excluding PN impairment modeling; - Impairment results for the definition of the requirements, including RF and PN impairment; On the basis of the compromise reached for the test cases highlighted for this section, the companies have also agreed on a way forward to eventually update the requirement in the next meeting based on updated results: - If the updated alignment results span for the highlighted cases is below 2.5dB, the requirements will be updated based on the averaged results; - If the updated alignment results span for the highlighted cases is still above 2.5dB, - if the results show no significant change with respect to results collected in RAN#105, the current compromise will be used for the definition of the requirements; - otherwise, companies reserve the option to revise the current compromise;

# 2	PDSCH Simulation Results

## 2.1 Simulation setup

As per WF [1], we carried out simulations using the agreed propagation environments. Furthermore, and for the sake of fairness, we performed our simulations considering both phase noise (PN) models in TR 38.808 [2], referring to as Set 1 and Set 2, where Set 1 has been proposed in [3] and Set 2 also known as TR 38.803 Example 2. In the sequel, we will use the general parameters as follow.

- Numerologies:

- SCS = 120 KHz, CBW = 100 MHz (66 PRBs and [32] PRBs).

- SCS = 480 KHz, CBW = 400 MHz (66 PRB and [20, 32] PRB).

- MCS 4, 13, and 17

- Max Doppler frequency/DMRS

- UE speed at 3 Km/h (f_D = 200 Hz), DMRS 1+1 ((2,11) symbol indexes)

- UE speed at 10 Km/h (f_D = 650 Hz), DMRS 1+1 ((2,11) symbol indexes)

- Two PN models in TR 83.808 [2]:

    - Set 1 in TR 38.808: PN model in [3], referred to as R4_2010176

The design margin (DM) should be set to 0 dB for BS while to be set to 5 dB for UE

    - Set 2 in TR 38.808: PN model TR38.803 Ex2, Set 2 in TR 38.808

- Carrier frequency: 70 GHz

- Antenna configuration: 2x2 ULA Low

- Rank 1

- Rel-15 PTRS configuration, K = 2, L = 1

- Phase noise at the UE receiver only

- Benchmark: No PN and No compensation

- CPE compensation only

We summarize the parameters above in the following table

70% Peak Throughput

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 30 ns delay spread) TDLD (10 ns and 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). |
| NOH | 6 |
| Number of HARQ Processes | 8 for 120kHz and 16 for 480kHz SCS |

30% Peak throughput

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 13 |
| NOH | 6 |
| Number of HARQ Processes | 16 |

## 2.2 Simulation results

In this section, we will present the normalized PDSCH throughput as function of the signal to noise ratio (SNR). As per RAN4 PDSCH metric, we will refer to SNR1 and SNR2 as the corresponding SNR value to 70% (or 30 %) of the peak throughput value when using PN models Set 1 and Set 2, respectively.

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of peak Throughput | Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models and MCSs but limited to 1 MIMO layer (Rank 1). | 1-1, 1-2, 1-3, 1-4 2-1, 2-2, 2-3 |
| 30% of peak Throughput | Verify the PDSCH mapping Type A normal performance with 2 receive antenna conditions under TDLA30-650 channel (Rank 1). | 3-1 |

## 2.2.1 SCS/CBW = 120 KHz/100 MHz results

Considering our benchmark, when no PN applied at both sides (ideal case), the achieved SNRs at 70% of the peak throughput have been referred to as SNR0 and gathered along with SNR1 and SNR2 in Tables 1.1 and 1.2. Furthermore, in the sequel, we will refer to D1 = SNR1 – SNR0 and D2 = SNR2 – SNR0 as the SNR gap (dB) between the PN CPE compensation output and the NO PN case.

For 120 KHz SCS and 100 MHz CBW, Table 1.1 gathers the results where 66 PRBs have been allocated and MCS (4, 13 and 17) have been considered where the max DL testable SNR level is at [9.8] dB as well as considering MCS 17 with partial PRB allocation [32], where the max DL testable SNR level is at [13.2] dB.

Table 1.1 - Reference SNR values whether PN has been considered or not for SCS 120 KHz/ CBW 100 MHz for MCS (4, 13, and 17) and using 66 PRBs.

|  | CBW (MHz) / SCS (KHz) = 100/120 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 1-1 | 4 (66) | TDLA30-650 | -2.8 | -2.6 | 0.2 | -2.6 | 0.2 |
| Test 1-2 | 13 (66) | TDLA30-200 | 5.2 | 5.6 | 0.4 | 5.6 | 0.4 |
| Test 1-3 | 17 (66) | TDLD30-200 | 7.5 | 8.1 | 0.6 | 8.2 | 0.7 |
| Test 1-4 | 17 (32) | TDLD30-200 | 7.4 | 8.1 | 0.7 | 8.1 | 0.7 |

TDLD10-200 simulation results have been depicted in Fig. 1 (see Appendix).

Observation 1: Referring to Table 1.1, considering additional impairments to the CPE compensation-based results, and considering the maximum DL testable SNR, we can conclude for 100 MHz/120kHz that MCS17 can be testable if we use partial RB allocation (32 RBs).

## 2.2.2 SCS/CBW = 480 KHz/400 MHz results

Following same arguments as in Section 2.2.1, the results have been gathered in Tables 2.1 where both the full and partial PBR allocation have been considered. It is worth mentioning here the max DL testable SNR is at [2.6] dB when 66 RBs have been considered, while the max DL testable SNRs are at [8.9] dB and [6.6] dB, when considering 20 RBs and 32 RBs, respectively.

Table 2.1 - Reference SNR values whether PN has been considered or not for SCS 480 KHz/ CBW 400 MHz for low MCS (4 and 13).

|  | CBW (MHz) / SCS (KHz) = 400/480 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 2-1 | 4 (66) | TDLA10-200 | -2.8 | -2.6 | 0.2 | -2.5 | 0.3 |
| Test 2-2 | 13 (20) | TDLD10-200 | 3.8 | 4.2 | 0.4 | 4.3 | 0.5 |
| Test 2-3 | 13 (32) | TDLD10-200 | 3.9 | 4.3 | 0.4 | 4.3 | 0.4 |

TDLD10-200 simulation results have been depicted in Fig. 2 (see Appendix).

Observation 2: Referring to Table 2.1 and considering 400 MHz/480 KHz with partial allocation of 20 RBs, the obtained SNR requirements for MCS 13 in TDLD10-200 allow for additional RF impairments’ margin while remaining below the max DL testable SNR.

## 2.2.3 Reference SNRs at 30% of the peak Throughput

In this section, we evaluate the reference SNRs at 30% of the peak throughput for 120 kHz SCS considering MCS 13 and full PRB allocation (66). The results have been gathered in Tables 3.1 where CPE compensation only was used.

Table 3.1 - Reference SNR values whether PN has been considered or not for SCS 120 kHz/ CBW 100 MHz for MCS 13.

|  | CBW (MHz) / SCS (KHz) = 100/120 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 30% Max Throughput | SNR1 (dB) @ 30% Max Throughput | D1 (dB) | SNR2 (dB) @ 30% Max Throughput | D2 (dB) |
| Test 3-1 | 13 (66) | TDLA30-650 | -1.1 | -0.9 | 0.2 | -0.9 | 0.2 |

Observation 3: Referring to Table 3.1, when considering performance metric at 30% of the peak throughput for 100 MHz/120 kHz, it has been noted that MCS 13 is testable under TDLA30-650 while considering full RB allocation and additional margin that count for RF impairments.

# 3	Summary

In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed.

Observation 1: Referring to Table 1.1, considering additional impairments to the CPE compensation-based results, and considering the maximum DL testable SNR, we can conclude for 100 MHz/120kHz that MCS17 can be testable if we use partial RB allocation (32 RBs).

Observation 2: Referring to Table 2.1 and considering 400 MHz/480 KHz with partial allocation of 20 RBs, the obtained SNR requirements for MCS 13 in TDLD10-200 allow for additional RF impairments’ margin while remaining below the max DL testable SNR.

Observation 3: Referring to Table 3.1, when considering performance metric at 30% of the peak throughput for 100 MHz/120 kHz, it has been noted that MCS 13 is testable under TDLA30-650 while considering full RB allocation and additional margin that count for RF impairments.

# References

- R4-2220187, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- 3GPP TR 38.808 v17.0.0, “Study on supporting NR from 52.6 GHz to 71 GHz”, (Release 17).

- R4-2010176, “On phase noise for 52.6-71 GHz”, Ericsson.

# Appendix	Simulation assumption

## A.1	Propagation environments

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

## A.2	Simulation results for TDLD channels

(a) MCS17, RB = 66                           (b) MCS17, RB = 32

Fig 1 – PDSCH normalized throughput as function of SNR using (SCS, CBW) = (120 KHz, 100 MHz)

in TDLD30-200 channels.

(a) MCS13, RB = 32                           (b) MCS13, RB = 20

Fig 2 – PDSCH normalized throughput as function of SNR using (SCS, CBW) = (480 KHz, 400 MHz)

in TDLD30-200 channels.