3GPP TSG-RAN WG4 Meeting #106	R4-2301894

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	9.6.6.1

Source:	Ericsson

Title:	Discussions on simulation results for 8Rx UE in FR1

Document for:	Discussion

# 1	Introduction

In this paper, we present the simulations results for 8Rx UE demodulation and CSI reporting in FR1. Based on last meeting discussions [1], we carried out simulation for PDSCH, PDCCH, SDR, and CQI reporting. Several observations have been drawn to help in shaping more agreements on physical channels and CSI reporting for 8Rx UE in FR1. It is worth reminding that all results have been provided under ideal conditions, no impairments.

# 2	PDSCH Simulation Results

## 2.1 Simulation setup

According to WF [1], we will use as general parameters:

TDD n77/78:

- fc = 3.5 GHz

- CBW/SCS: 40 MHz/30 kHz

- Up to 64QAM for Rank 4, 6 and 8, DMRS 2+2

- Up to 256QAM for Rank 2, DMRS 1+1

- Max Doppler frequency

UE speed at 3 Km/h (f_D = 10 Hz), for Rank > 2

UE speed at 30 Km/h (f_D = 100 Hz), for Rank = 2

- Pre-loaded Tx EVM: 6% for QPSK/16QAM/64QAM and 3% for 256QAM

- Additive Rx EVM: 0%.

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of peak Throughput | Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions, with different channel models, different MCSs but up to 8 MIMO layer (=< Rank 8). | 1-1, 1-2, 1-3, 1-4, 1-5 2-1, 2-2, 2-3 3-1, 3-2, 3-3, 3-4 |

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 3.5 GHz |
| Subcarrier Spacing [KHz] | 30 kHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA, TDLC (30 ns delay spread) |
| Antenna configuration | 2x8, 4x8, and 8x8 ULA Low |
| Velocity | 3 km/h, 30 km/h |
| PA Model | None |
| Pre-loaded Tx EVM | 6% (QPSK/16QAM/64QAM) and 3% (256QAM) |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 2, Rank 4, Rank 6 and Rank 8 |
| DMRS Configuration | 2+2 DMRS symbols at (2,3,10,11) symbol indexes for Rank > 2 1+1 DMRS symbols at (2, 11) symbol indexes for Rank 2 |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). From MCS Table 2 (TS38.214): Up to MCS 20 (256QAM) |
| NOH | 0 |
| Number of HARQ Processes | 8 |

## 2.2 Simulation results

In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 70% of the peak throughput. All figures have been provided in the Appendix.

Test cases for Rank > 4:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format an.d code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 1.4 |
| 1-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 13.7 |
| 1-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.2 |
| 1-4 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | 9.2 |
| 1-5 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 MCS 17 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | 13.3 |
| 1-6 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 20 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | 16.3 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

Observation 1: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 8 DL MIMO Layers using MCS17 under TDLA30-10 propagation environment. The SNR value is at 19.2 dB, thus 5 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

Observation 2: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 6 DL MIMO Layers using MCS20 under TDLA30-10 propagation environment. The SNR value is at 16.3 dB, thus 8 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

Test cases for Rank = 4:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | -3.7 |
| 2-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 5 |
| 2-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 20 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 11.3 |
| 2-4 | 3.5 | 40 / 30 (106) | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 18.3 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

Observation 3: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 4 DL MIMO Layers using MCS26 under TDLA30-10 propagation environment. The SNR value is at 18.3 dB, thus 6 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

Test cases for Rank = 2:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 2 | 2 | TDLC30-100 | 2x8, ULA Low | 70 | -6.5 |
| 3-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 7 | 2 | TDLC30-100 | 2x8, ULA Low | 70 | 1.5 |
| 3-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 13 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6.8 |
| 3-4 | 3.5 | 40 / 30 (106) | 256QAM, 0.67 MCS 20 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 13.9 |
| 3-5 | 3.5 | 40 / 30 (106) | 256QAM, 0.89 MCS 26 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 20 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

Observation 4: We can note that 8RX UE can support 2 DL MIMO Layers using MCS7 (16QAM) under TDLC30-100 propagation environment (Vehicular). Furthermore, 8RX UE can support 2 DL MIMO Layers using MCS26 (256QAM) under TDLA30-10 with 10.4 dB below the maximum SNR levels for conducted requirements (at 3% of Tx EVM).

# 3	PDCCH Simulation Results

## 3.1 Simulation setup

We consider the following parameters to carry out simulation for PDCCH for 8Rx UE in FR1:

TDD n77/78:

- fc = 3.5 GHz

- CBW/SCS: 40 MHz/30 kHz

- Propagation environment: TDLA30-10

- Pre-loaded Tx EVM: 6%

- Additive Rx EVM: 0%.

| Test num | Interleaver Size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNR (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 102 | 1 | 2 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |
| 2 | 3 | 2 | 102 | 1 | 4 | TDLA30-10 | 1x8 ULA Low | 1_1 | 53 | New |
| 3 | 3 | 6 | 90 | 1 | 8 | TDLA30-10 | 2x8 ULA Low | 1_1 | 53 | New |
| 4 | NA | 6 | 48 | 2 | 16 | TDLA30-10 | 2x8 ULA Low | 1_0 | 41 | New |
| 5 | NA | 2 | 48 | 2 | 16 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |

## 3.2 Simulation results

In this section, we present the PDCCH performance in terms of the probability of miss-detection of the downlink scheduling grant (Pm-dsg) as function of SNR (dB). The PDCCH performance metric is defined as the needed SNR (dB) for a Pm-dsg (BLER) at 1%.

| Test num | Interleaver Size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNR (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 102 | 1 | 2 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | -1.9 |
| 2 | 3 | 2 | 102 | 1 | 4 | TDLA30-10 | 1x8 ULA Low | 1_1 | 53 | -3.8 |
| 3 | 3 | 6 | 90 | 1 | 8 | TDLA30-10 | 2x8 ULA Low | 1_1 | 53 | -7.3 |
| 4 | NA | 6 | 48 | 2 | 16 | TDLA30-10 | 2x8 ULA Low | 1_0 | 41 | -12.0 |
| 5 | NA | 6 | 48 | 2 | 16 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | -11.5 |

Observation 5: We can note that 8RX UE can support all proposed aggregation levels in TDLA30-10.

Fig 1 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (30 kHz, 40 MHz) in TDLA30-10 channels.

# 4	SDR Simulation Results

## 4.1 Simulation setup

Test case 1: SDR for up to 64QAM

- SCS = 30 KHz, CBW = 40 MHz (106 RBs).

- Antenna config 8x8 Low, Rank 8, AWGN

- DMRS 2+2 (2, 3, 10 and 11 symbol indexes)

- Tx EVM: 6%

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 6 | 1 | 27 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Test case 2: SDR for 256QAM

- SCS = 30 KHz, CBW = 40 MHz (106 RBs).

- Antenna config 8x8 Low, Rank 8, AWGN

- DMRS 2+2 (2, 3, 10 and 11 symbol indexes)

- Tx EVM: 3%

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 26 |
| 8 | 8 | 0.8 | 23 |
| 8 | 8 | 0.75 | 22 |
| 8 | 8 | 0.4 | 12 |
| Note 1: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Test case 3: SDR for 1024QAM

- SCS = 30 KHz, CBW = 40 MHz (106 RBs).

- Antenna config 8x8 Low, Rank 8, AWGN

- DMRS 1+1 (2 and 11 symbol indexes)

- Tx EVM: 2.5%

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

## 4.2 Simulation results

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- |
| 8 | 6 | 1 | 27 | 23.8 |
|  |  | 0.8 | 24 | 17.7 |
|  |  | 0.75 | 23 | 15.7 |
|  |  | 0.4 | 14 | 7.7 |
|  | 4 | 1 | 16 | 8.7 |
|  |  | 0.8 | 16 |  |
|  |  | 0.75 | 16 |  |
|  |  | 0.4 | 11 | 4.7 |
|  | 2 | 1 | 9 | 3.7 |
|  |  | 0.8 | 9 |  |
|  |  | 0.75 | 9 |  |
|  |  | 0.4 | 5 | -0.3 |

Observation 6: We can note that SDR results with Rank 8 can be verified for FR1 8Rx UE using 64QAM with MCS26.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 | 24.5 |
|  |  |  | 0.8 | 21 | 19.7 |
|  |  |  | 0.75 | 19 | 17.3 |
|  |  |  | 0.4 | 9 | 6.7 |

Observation 7: We can note that SDR results with Rank 2 can be verified for FR1 8Rx UE using 1024QAM with MCS24.

# 5	CQI Reporting Simulation Results

## 5.1 Simulation setup

Considering the following parameters as stated in WF [1], it is worth mentioning that we will be limited to CSI reporting under static conditions only, where we use

- CBW/SCS = 40 MHz/30 kHz

- CQI indexes up to 256QAM given by Table 2 (Table 5.1.3.1-2) in TS 38.214.

- Antenna configuration: 4T8R

- Pre-loaded Tx EVM: 3%

- Additive Rx EVM: 0%

- Rank4

We will consider the following table (Table A.4-3 in TS38.101-4) throughout the simulations using TBS3.4

| TBS Scheme |  |  |  | TBS.3-1 | TBS.3-2 | TBS.3-3 | TBS.3-4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |  |  |
| Number of PDSCH MIMO layers |  |  |  | 3 | 4 | 3 | 4 |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |  |  |
| Available RE-s for PDSCH |  |  |  | 6240 | 6240 | 12720 | 12720 |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |  |  |
| 1 | 0.2344 | 0 | QPSK | 4360 | 5896 | 8976 | 11784 |  |  |
| 2 | 0.3770 | 1 |  | 7048 | 9480 | 14344 | 18976 |  |  |
| 3 | 0.8770 | 3 |  | 16392 | 22032 | 33816 | 45096 |  |  |
| 4 | 1.4766 | 5 | 16QAM | 27656 | 36896 | 56368 | 75792 |  |  |
| 5 | 1.9141 | 7 |  | 35856 | 48168 | 73776 | 98376 |  |  |
| 6 | 2.4063 | 9 |  | 45096 | 60456 | 92200 | 122976 |  |  |
| 7 | 2.7305 | 11 | 64QAM | 51216 | 67584 | 104496 | 139376 |  |  |
| 8 | 3.3223 | 13 |  | 62504 | 81976 | 127080 | 167976 |  |  |
| 9 | 3.9023 | 15 |  | 73776 | 98376 | 147576 | 196776 |  |  |
| 10 | 4.5234 | 17 |  | 83976 | 112648 | 172176 | 229576 |  |  |
| 11 | 5.1152 | 19 |  | 96264 | 127080 | 196776 | 262376 |  |  |
| 12 | 5.5547 | 21 | 256QAM | 104496 | 139376 | 213176 | 278776 |  |  |
| 13 | 6.2266 | 23 |  | 116792 | 155776 | 237776 | 319784 |  |  |
| 14 | 6.9141 | 25 |  | 129128 | 172176 | 262376 | 352440 |  |  |
| 15 | 7.4063 | 27 |  | 139376 | 184424 | 278776 | 376896 |  |  |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-2 of TS 38.214 |  |  |  |  |  |  |  |  |  |

## 5.2 Simulation results

| SNR | Median CQI | Pr(Median CQI – 1 < CQI < Median CQI+1) [%] | BLER of Median CQI-1 [%] | BLER of Median CQI [%] | BLER of Median CQI+1 [%] |
| --- | --- | --- | --- | --- | --- |
| -6 | 2 | 100 | 0 | 63.1 | 100 |
| -5 | 2 | 100 | 0 | 0.1 | 100 |
| -4 | 2 | 100 | 0 | 0 | 100 |
| -3 | 3 | 100 | 0 | 100 | 100 |
| -2 | 3 | 100 | 0 | 32.7 | 100 |
| -1 | 3 | 100 | 0 | 0 | 100 |
| 0 | 3 | 100 | 0 | 0 | 100 |
| 1 | 4 | 100 | 0 | 92.6 | 100 |
| 2 | 4 | 100 | 0 | 9.3 | 100 |
| 3 | 5 | 100 | 0 | 86.7 | 100 |
| 4 | 5 | 100 | 0 | 0.1 | 100 |
| 5 | 6 | 100 | 0 | 86.5 | 100 |
| 6 | 6 | 100 | 0 | 0 | 100 |
| 7 | 7 | 100 | 0 | 77.8 | 100 |
| 8 | 7 | 100 | 0 | 0 | 100 |
| 9 | 8 | 100 | 0 | 21.5 | 100 |
| 10 | 9 | 100 | 0 | 100 | 100 |
| 11 | 9 | 100 | 0 | 0.4 | 100 |
| 12 | 10 | 100 | 0.1 | 100 | 100 |
| 13 | 10 | 100 | 0 | 12.3 | 100 |
| 14 | 11 | 100 | 0 | 100 | 100 |
| 15 | 11 | 100 | 0 | 87.6 | 100 |
| 16 | 12 | 100 | 0.3 | 100 | 100 |
| 17 | 12 | 100 | 0 | 2.5 | 100 |
| 18 | 13 | 100 | 0 | 100 | 100 |
| 19 | 13 | 100 | 0 | 17.6 | 100 |
| 20 | 14 | 100 | 1.3 | 100 | 100 |
| 21 | 14 | 100 | 0.1 | 21.6 | 100 |
| 22 | 14 | 100 | 0 | 1.8 | 100 |
| 23 | 15 | 100 | 0 | 18.7 | NaN |
| 24 | 15 | 100 | 0 | 0.1 | NaN |
| 25 | 15 | 100 | 0 | 0 | NaN |

Fig.2 - Simulation results depicting the median CQI as function of SNR using 30 kHz/40 MHz and Rank 4.

Observation 8: The results show that the lowest SNR to report efficiently a CQI with Rank 1 related to

- 256QAM is at SNR = 16/17 dB (CQI 12).

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

# 6	Summary

In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed.

Observation 1: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 8 DL MIMO Layers using MCS17 under TDLA30-10 propagation environment. The SNR value is at 19.2 dB, thus 5 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

Observation 2: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 6 DL MIMO Layers using MCS20 under TDLA30-10 propagation environment. The SNR value is at 16.3 dB, thus 8 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

Observation 3: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 4 DL MIMO Layers using MCS26 under TDLA30-10 propagation environment. The SNR value is at 18.3 dB, thus 6 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

Observation 4: We can note that 8RX UE can support 2 DL MIMO Layers using MCS7 (16QAM) under TDLC30-100 propagation environment (Vehicular). Furthermore, 8RX UE can support 2 DL MIMO Layers using MCS26 (256QAM) under TDLA30-10 with 10.4 dB below the maximum SNR levels for conducted requirements (at 3% of Tx EVM).

Observation 5: We can note that 8RX UE can support all proposed aggregation levels in TDLA30-10.

Observation 6: We can note that SDR results can be verified for FR1 8Rx UE using 64QAM with MCS26.

Observation 7: We can note that SDR results with Rank 2 can be verified for FR1 8Rx UE using 1024QAM with MCS24.

Observation 8: The results show that the lowest SNR to report efficiently a CQI with Rank 1 related to

- 256QAM is at SNR = 16/17 dB (CQI 12).

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

# References

- R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei.

- R4-2301893, “UE demodulation and CSI reporting requirements for 8 Rx in FR1”, Ericsson.

# Appendix

## A.1 Table 5.1.3.1-2: MCS index table 2 for PDSCH (TS38.214)

Table 5.1.3.1-2: MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

## A.2 PDSCH Simulations

(a) Up to 64QAM, Rank 8, TDLA30-10           (b) Up to 64QAM, Rank 6, TDLA30-10

Fig. 3 -  FR1 8 Rx PDSCH simulation results depicting the normalized throughput as function of SNR, where Rank 6 and Rank 8 have been considered for up to 64QAM with 40 MHz/30 kHz (106 RBs).

Fig. 4 -  FR1 8 Rx PDSCH simulation results depicting the normalized throughput as function of SNR, where Rank 4 has been considered for up to 64QAM with 40 MHz/30 kHz (106 RBs).

(a) Up to 16QAM, Rank 2, TDLC30-100             (b) Up to 256QAM, Rank 2, TDLA30-10

Fig. 5 -  FR1 8 Rx PDSCH simulation results depicting the normalized throughput as function of SNR, where Rank 2 has been considered for up to 256QAM with 40 MHz/30 kHz (106 RBs).

## A.3 SDR Simulations

Fig. 6 - PDSCH simulation results depicting the normalized throughput as function of SNR to determine the practical MCS for SDR requirements when using up to 64QAM for FR1 8 Rx UE with Rank 8.

Fig. 7 - PDSCH simulation results depicting the normalized throughput as function of SNR to determine the practical MCS for SDR requirements when using 1024QAM for FR1 8 Rx UE with Rank 2.