3GPP TSG-RAN WG4 Meeting # 106		R4-2301925

# Athens, Greece, February 27 – March 3, 2022

Agenda Item:	11.1.4

Source: 		Qualcomm Incorporated

Title: 	Discussion on fallback ambiguitiy for CA_46O/N/M

Document for:	Approval

# 1 Introduction

RAN2 sent the Reply LS on new continuous BW classes for legacy network to RAN4 in which the fallback ambiguity issue for CA_46O/N/M was raised.

# 2 Discussion

As stated in the reply LS[1], RAN2 identified a potential issue related to RAN4’s “Fallback Group” requirement for bandwidth class for CA_n46O/N/M. It is explained using the following example of CA band combinations associated bandwidth classes of FBG3.

Table 1: FBG3 CA band combinations

| NR CA config. | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Max. aggregated BW (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46M | 20, 40, 60 | 20, 40 | 20, 40 |  |  | 140 | 0 |
| CA_n46N | 20, 40, 80 | 20, 40 | 20, 40 | 20, 40 |  | 200 | 0 |
| CA_n46O | 20, 60 | 20, 40 | 20, 40 | 20, 40 | 20, 40 | 220 | 0 |

Assume the UE supports and configures with CA_n46O with the following CC bandwidths:

- {60MHz, 40MHz, 40MHz, 40MHz, 40MHz}

According to the requirement for fallback band combinations in TS38.306, the UE shall support the fallback band combination with 4CCs with the following CC bandwidths.

- {60MHz, 40MHz, 40MHz, 40MHz}

However, the bandwidth of 60MHz is not defined for CA_n46N in the first/leftmost CC in TS38.101-1.

In addition, we can see the similar issue when CA_n46N falls back to CA_n46M. It is saying UE supports and configures with CA_n46N with the following CC bandwidths:

- {80MHz, 40MHz, 40MHz, 40MHz}

Then the UE shall support the fallback band combination with 3CCs with the following CC bandwidths.

- {80MHz, 40MHz, 40MHz}

But the channel bandwidth of 80MHz is not defined in CA_n46M in the first/leftmost CC in TS38.101-1.

Observation 1: There are issues for the fallback ground of CA_n46O/N/M in TS38.101-1.

To solve the issues, there are two options:

- Option 1: Revise the channel bandwidth of first/leftmost CC of CA_n46N from 80MHz to 60MHz in BCS0

| NR CA config. | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Max. aggregated BW (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46M | 20, 40, 60 | 20, 40 | 20, 40 |  |  | 140 | 0 |
| CA_n46N | 20, 40, 60 | 20, 40 | 20, 40 | 20, 40 |  | 200 | 0 |
| CA_n46O | 20, 60 | 20, 40 | 20, 40 | 20, 40 | 20, 40 | 220 | 0 |

- Option 2: Add a new BCS, e.g., BCS1 to include the missing channel bandwidth of 60MHz and 80MHz for CA_n46N and CA_n46M respectively.

| NR CA config. | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Max. aggregated BW (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46M | 20, 40, 60 | 20, 40 | 20, 40 |  |  | 140 | 0 |
|  | 20, 40, 80 | 20, 40 | 20, 40 |  |  | 160 | 1 |
| CA_n46N | 20, 40, 80 | 20, 40 | 20, 40 | 20, 40 |  | 200 | 0 |
|  | 20, 40, 60 | 20, 40 | 20, 40 | 20, 40 |  | 180 | 1 |
| CA_n46O | 20, 60 | 20, 40 | 20, 40 | 20, 40 | 20, 40 | 220 | 0 |

Proposal 1: RAN4 to select the solution from above two options to solve the fallback issues in CA_n46O/N/M.

Proposal 2: RAN4 to approve the draft LS in Appendix.

# 3 Conclusions

This contribution provided our considerations on fallback ambiguity issues for CA_n46O/N/M and we have the following observation and proposals:

Observation 1: There are issues for the fallback ground of CA_n46O/N/M in TS38.101-1.

Proposal 1: RAN4 to select the solution from above two options to solve the fallback issues in CA_n46O/N/M.

Proposal 2: RAN4 to approve the draft LS in Appendix.

# 4 References

R2-2213312, “Reply LS on new contiguous BW classes for legacy networks”

# Appendix (Draft LS)

3GPP TSG-RAN WG4 Meeting # 106						R4-23xxxxx

# Athens, Greece, February 27 – March 3, 2022

Title:	LS on fallback ambiguity for CA_n46O/N/M

Release:	Rel-18

Work Item:	XXXX

Source:	RAN4

To:	RAN2

Contact person:

Name:	Bin Han

E-mail Address:	binhan@qti.qualcomm.com

# 1   Overall Description:

RAN4 discussed and identified the fallback issues in CA_n46O/N/M. RAN4 concluded to adopt the following Option 1 or Option 2 to revise the NR CA configurations in TS 38.101-1. The content of attached CR xxxx was agreed.

- Option 1: Option 1: Revise the channel bandwidth of first/leftmost CC of CA_n46N from 80MHz to 60MHz in BCS0

| NR CA config. | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Max. aggregated BW (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46M | 20, 40, 60 | 20, 40 | 20, 40 |  |  | 140 | 0 |
| CA_n46N | 20, 40, 60 | 20, 40 | 20, 40 | 20, 40 |  | 200 | 0 |
| CA_n46O | 20, 60 | 20, 40 | 20, 40 | 20, 40 | 20, 40 | 220 | 0 |

- Option 2: Add a new BCS, e.g., BCS1 to include missing channel bandwidth of 60MHz and 80MHz for CA_n46N and CA_n46M respectively.

| NR CA config. | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Max. aggregated BW (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46M | 20, 40, 60 | 20, 40 | 20, 40 |  |  | 140 | 0 |
|  | 20, 40, 80 | 20, 40 | 20, 40 |  |  | 160 | 1 |
| CA_n46N | 20, 40, 80 | 20, 40 | 20, 40 | 20, 40 |  | 200 | 0 |
|  | 20, 40, 60 | 20, 40 | 20, 40 | 20, 40 |  | 180 | 1 |
| CA_n46O | 20, 60 | 20, 40 | 20, 40 | 20, 40 | 20, 40 | 220 | 0 |

# 2   Actions

To RAN2:

RAN4 would like to respectfully request RAN2 to take the above information into account.

# 3   Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #106-bis-e                   April 2023         				Online

TSG-RAN WG4 Meeting #107			May 2023			      	             Korea