3GPP TSG-RAN WG4 Meeting # 106	R4-2301926

Athens, Greece, 27 February – 3 March, 2023

Source:	Ericsson

Title:	GNSS position for UE transmit timing tests

Agenda item:	6.1.5.1

Document for:	Approval

# Introduction

In RAN4# 105 WF on NTN RRM measurement requirements was approved [1]. In this contribution we present our views on the open issues as following.

# Discussion

## Test case design for Cell reselection

### Issue 2-1: UE GNSS location change in location-based intra/inter-frequency cell reselection test.

- Option 1: - For location-based cell reselection test cases, UE GNSS location change shall be no less than distanceThresh + 50 m. - Option 2: - As baseline, no location margin needs to be added in the test cases where “Update UE Location Information” procedure is used to set the GNSS location.

In approved WF [2] the following pre-condition for timing requirements, like UE Initial Timing Error, Te, in TS 38.133 section 7, was defined to be:

- GNSS accuracy assumption for timing requirements

  - For UL SCS = 15 kHz and 30 kHz: 2-D position error is 50 m

  - For UL SCS = 60kHz in FR1: FFS

Since we have a pre-condition traceable to approved WF of 2-D position error of 50 m then it is reasonable to assume this in UE GNSS location change in location-based intra/inter-frequency cell reselection test as well.

Proposal 1: Assume of 2-D position error of 50 m in UE GNSS location change in location-based intra/inter-frequency cell reselection test, when developing performance test margins.

## Test case design for UE timing requirements

### Issue 4-1: Acquisition of UE location in UE timing test cases

- Option 1: - UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused. - Option 2: - For NTN UE timing test cases, it is suggested to use AT command approach to acquire UE location in order to simplify the test configuration.

In [3] a draft CR on UE transmit timing tests for NTN was endorsed during RAN4#105. However the issue 4-1 was not resolved. The test sequence verifies that the UL time is within specified limits of the first detected path of DL SSB. The test requirements in [2] states:

- After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±Te_NTN[±Tmargin] of the first detected path of DL SSB.

- The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Clause 7.1C.2 Table 7.1C.2.1-1 until the UE transmit timing offset is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±Te_NTN[±Tmargin] respective to the first detected path (in time) of DL SSB. Skip this step for test 2 with DRX configured.

- The test system shall verify that the UE transmit timing offset stays within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±Te_NTN[±Tmargin] of the first detected path of DL SSB.

However, when the Te_NTN values were derived there were certain GNSS position errors assumed. In approved WF [2], 50 meters was allocated to the UE when it comes to 2D position error. The UL function depends strongly on a working GNSS system in the UE.

We support Option1: “UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused”.

This should be added to an updated CR: “The test system can emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.4.1”.

Proposal 2: Add this text to updated CR R4-2220423, DraftCR on UE transmit timing tests for NTN: “The test system can emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.4.1”.

# Summary

Proposal 1: Assume of 2-D position error of 50 m in UE GNSS location change in location-based intra/inter-frequency cell reselection test, when developing performance test margins.

Proposal 2: Add this text to updated CR R4-2220423, DraftCR on UE transmit timing tests for NTN: “The test system can emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.4.1”.

# References

- R4-2220317	WF on performance part for NR NTN, Xiaomi

- R4-2115346	WF on GNSS-related and timing requirements for NR NTN, Xiaomi