3GPP TSG-RAN WG4 Meeting # 106	R4-2301927

Athens, Greece, 27 February – 3 March, 2023

Source:	Ericsson

Title:	Reference Time Instances for UL Timing Accuracy Requirements

Agenda item:	6.1.5.2

Document for:	Approval

# Introduction

In RAN4# 105 WF on NTN RRM measurement requirements was approved [1]. In this contribution we present our views on the open issues as following.

# Discussion

## Test case design for UE timing requirements

### Issue 4-2: Reference Time Instances for UL Timing Accuracy Requirements

- Option 1: (QC) - In RRM test cases, when a test equipment adjusts downlink transmission frame boundary/Doppler shift and UL reception timing, asymmetric propagation delays on DL and UL for the same slot index shall be taken into account. To model the round trip delay over service link (N_{TA,UE-specific}), the following definitions of reference slot for S3 and S4 (based in Fig. 3) are adopted and captured in Annex B for RAN5 and TE vendors’ reference. - The S3 represents a one-way propagation delay of a UE UL transmission over the service link from the UE to the satellite. The UL transmission arrives at the satellite at T3, which is the time instance when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model. - The S4 represents a one-way propagation delay of a DL transmission over the service link from the satellite to the UE. The satellite relays the DL transmission from the GW to UE at T4, which is the time instance when the DL transmission corresponding to the same slot index as the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model. Figure 3. Timing Relation between UE DL Reception and UE UL Transmission - Option 2: (CATT) - The test requirement for UE initial transmit timing is Te_NTN, and Reference timing for uplink transmission in test cases is (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc, here - The DL timing shall be adjusted according to the distance change between serving satellite and UE based on a reference orbit for the serving satellite. - NTA,common is the common TA for reference timing is calculated based on the definition in TS38.211. - NTA,UE-specific is the UE specific TA for reference timing derived from the satellite location and UE location, and equal to (D1 + D2) / (c×Tc) . - D1 is distance between UE and satellite when receiving the UE transmit signal slot. - D2 is distance between UE and satellite when at time of NTA,common×Tc after receiving the UE transmit signal slot. - Option 3: (Huawei) - For NTN UE timing testing, it is suggested to define a reference orbit for the serving satellite, and the DL timing shall be adjusted according to the distance change between serving satellite and UE. - For NTN UE timing test cases, the additional timing error for the translation from reference orbit to ephemeris information can be considered.

To make the DraftCR on UE transmit timing tests for NTN, Huawei, HiSilicon, [2], complete, we should consider adding a reference orbit to TS appendix. This position corresponds to option 3 in WF [1].

Proposal 1: For NTN UE timing testing, it is suggested to define a reference orbit for the serving satellite, and the DL timing shall be adjusted according to the distance change between serving satellite and UE.

# Summary

Proposal 1: For NTN UE timing testing, it is suggested to define a reference orbit for the serving satellite, and the DL timing shall be adjusted according to the distance change between serving satellite and UE.

# References

- R4-2220317	WF on performance part for NR NTN, Xiaomi

- R4-2220423	DraftCR on UE transmit timing tests for NTN, Huawei, HiSilicon