3GPP TSG-RAN4 Meeting #106	R4-2301968

# Athens, Greece, 27 February – 3 March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2972 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TCs for PDC measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-12-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In RAN4#104, SRS RMC is added for PDC measurement. However, it is only defined for 120kHz SCS while PDC TCs are also defined for 15kHz and 30kHz SCS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add SRS RMC for PDC TCs with 15kHz and 30kHz SCS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - FR1 PDC TCs are incomplete due to missing SRS RMC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## A.3.24	SRS configuration

Table A.3.24-1: Sounding Reference Symbol Configuration for SCS=15kHz

|  | SRS.1 TDD | PDC-SRS.1 | POS-SRS.1 |  |
| --- | --- | --- | --- | --- |
| Field | Value |  |  | Comment |
| c-SRS | 12 | Same as NRB,c in the test case | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 | n.a. |  |
| b-hop | 0 | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | usagePDC-r17 | n.a. |  |
| startPosition | 1 | 5 | 5 | resourceMapping setting |
| nrofSymbols | 1 | 4 | 4 |  |
| repetitionFactor | n1 | n.a. | n.a. | without repetition. |
| transmissionComb | n2 | n4 | n4 |  |
| combOffset-n2 | 0 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic | Periodic | Periodic |  |
| periodicityAndOffset-p | sl40, 1 | sl160, 20 | sl160, 20 | SRS transmission periodicity |

Table A.3.24-2: Sounding Reference Symbol Configuration for SCS=30kHz

|  | SRS.2 TDD | PDC-SRS.2 | POS-SRS.2 |  |
| --- | --- | --- | --- | --- |
| Field | Value |  |  | Comment |
| c-SRS | 24 | Same as NRB,c in the test case | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 | n.a. |  |
| b-hop | 0 | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | usagePDC-r17 | n.a. |  |
| startPosition | 3 | 5 | 5 | resourceMapping setting |
| nrofSymbols | 1 | 4 | 4 | SRS symbols belong to the same SRS resource. |
| repetitionFactor | n1 | n.a. | n.a. | without repetition. |
| transmissionComb | n2 | n4 | n4 |  |
| combOffset-n2 | 0 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | port1 | Number of antenna ports used for SRS resource transmission |
| resourceType | Periodic | Periodic | Periodic |  |
| periodicityAndOffset-p | sl80, 3 | Sl320, 40 | Sl320, 40 | SRS transmission periodicity |

Table A.3.24-3: Sounding Reference Symbol Configuration for SCS=120kHz

|  | SRS.3 TDD | PDC-SRS.3 | POS-SRS.3 |  |
| --- | --- | --- | --- | --- |
| Field | Value |  |  | Comment |
| c-SRS | 17 | Same as NRB,c in the test case | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 | n.a. |  |
| b-hop | 0 | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | usagePDC-r17 | n.a. |  |
| startPosition | 1 | 5 | 5 | resourceMapping setting |
| nrofSymbols | 1 | 4 | 4 | SRS symbols belong to the same SRS resource. |
| repetitionFactor | n1 | n1 | n.a. | without repetition. |
| transmissionComb | n2 | n4 | n4 |  |
| combOffset-n2 | 0 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | port1 | Number of antenna ports used for SRS resource transmission |
| resourceType | Periodic | Periodic | Periodic |  |
| periodicityAndOffset-p | sl320, 3 | Sl1280, 160 | Sl1280, 160 | SRS transmission periodicity |

<End of Change 1>