3GPP TSG-RAN4 Meeting #106	R4-2301985

# Athens, Greece, 27 February – 3 March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | xxxx | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RRM requirements for eMTC over NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2022-12-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RAN2 agreed that provison of neighbor cell assistance info is not supported in Rel-17. It means all the RRM requirements invovling neighbor cell measurement and mobility cannot apply. - For HO and CHO, the applicability condition of the requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove Idle mode reselection, re-establishment, re-direction and Connected mode measurement requirements for eMTC. - For HO and CHO, add the applicability condition that UE needs to have valid and applicable assistance info of the target cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - There will be requirements defined that will never apply based on Rel-17 RAN2 spec. - Agreed side condition of the requirements is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7A, 5.5A, 6.7A, 6.8A, 8.13A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 4.7A	Cell Selection and Re-selection Requirements for UE category M1 for Satellite Access

The UE category M1 applicability of the requirements in section 4.7A is defined in Section 3.6.1. The requirements in this subclause apply if UE category M1 is in normal and enhanced coverage area of the serving cell served by a satellite access node. The UE category M1 normal and enhanced coverge applicability of the requirements is defined in section 3.6.1.

### 4.7A.1	Cell Selection

The requirements defined in section 4.1 apply for this section.

### 4.7A.2	Cell Re-selection for UE category M1 for Satellite Access

The cell reselection procedure allows the UE to select a more suitable cell and camp on it.

When the UE is in either Camped Normally state or Camped on Any Cell state on a cell, the UE shall attempt to detect, synchronise, and monitor intra-frequency and inter-frequency cells indicated by the serving cell. For intra-frequency and inter-frequency cells the serving cell may not provide explicit neighbour list but carrier frequency information and bandwidth information only. UE measurement activity is also controlled by measurement rules defined in TS36.304, allowing the UE to limit its measurement activity.

If the UE is in normal coverage as defined in section 3.6.1, the requirements in section 4.7A.2.1 apply. If the UE is in enhanced coverage as defined in section 3.6.1, the requirements in section 4.7A.2.2 apply.

If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after t-Service [2] when the serving cell stop serving the area.

Editor’s note: FFS on how to capture following agreement:

When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage. Definition of in coverage is FFS.

#### 4.7A.2.1	Cell Re-selection requirements for UE category M1 in normal coverage

##### 4.7A.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage

The requirements in this subclause apply for the UE in the normal coverage area of the serving cell served by a satellite access node. The UE is considered to be in normal coverage area of serving cell according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot of the serving cell defined in Annex B.1.3 for a corresponding Band.

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in [1] for the serving cell at least every DRX cycle.

The UE shall filter the RSRP and RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

If the UE is not configured with eDRX_IDLE cycle and has evaluated according to Table 4.7A.2.1.1-1 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

If the UE is configured with eDRX_IDLE cycle and has evaluated according to Table 4.7A.2.1.1-2 in Nserv consecutive DRX cycles within a single PTW that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

Editor’s note: FFS on following:

[If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s in Table 4.7A.2.1.1-1 starting from at least [2] DRX cycles before ‘t-Service-r17’.

If the UE is not configured with eDRX_IDLE cycle and configured with DRX cycle ≥ [1.28] s then the UE shall meet the requirements defined for DRX cycle of [640] ms in Table 4.7A.2.1.1-1 during at least 2 configured DRX cycles immediately after ‘t-ServiceStart-r17’ [2].

If the UE is configured with eDRX_IDLE cycle then the UE shall meet the requirements defined for eDRX_IDLE cycle of 5.12 s in Table 4.7A.2.1.1-2 during 2 eDRX_IDLE cycles immediately after ‘t-ServiceStart-r17’ [2] regardless of the configured eDRX_IDLE cycle.]

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T, the UE shall initiate cell selection procedures for the selected PLMN as defined in [1], where T=10 s if the UE is not configured with eDRX_IDLE cycle, and T=MAX(10 s, one eDRX_IDLE cycle) if the UE is configured with eDRX_IDLE cycle.

When the UE is provided with ‘t-serviceStart-r17’ and has discontinuous coverage capabilities, then after ‘t-service-r17’ is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage. Editor’s note: Definition of in coverage is FFS.

Table 4.7A.2.1.1-1: Nserv

| DRX cycle length [s] | Nserv [number of DRX cycles] |
| --- | --- |
| 0.32 | 4 |
| 0.64 | 4 |
| 1.28 | 2 |
| 2.56 | 2 |

Table 4.7A.2.1.1-2: Nserv for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Nserv [number of DRX or eDRX cycles Note 3] |
| --- | --- | --- | --- |
| 5.12 | N/A | N/A | 2 |
| 10.24 ≤ eDRX_IDLE cycle length ≤ 2621.44 | 0.32 | ≥1.28 (1) | 2 |
|  | 0.64 | ≥1.28 (1) | 2 |
|  | 1.28 | ≥2.56 (2) | 2 |
|  | 2.56 | ≥5.12 (4) | 2 |
| Note 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s, number of DRX cycles otherwise. |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

The UE is allowed to perform RSRP measurements based on RSS signals provided UE is configured with rss-ConfigCarrierInfo [2] and following conditions are met:

-	Serving cell RSS are available within the paging MPDCCH narrowband for Nserv successive DRX cycles and the last subframe of the RSS occasion is in the window [n-5, n-1] where n is the first subframe of paging MPDCCH narrowband, and

-	RSS power offset (PRSS) with respect to CRS as defined in RSS-Config [2], where PRSS ≥ 0 dB, and

-	UE is not configured with eDRX_IDLE cycle, and DRX cycle length is 0.32s or 0.64s.

-	Nserv defined in Table 4.7A.2.1.1-1 applies if serving cell is measured based on RSS.

Table 4.7A.2.1.1-1: Nserv

| DRX cycle length [s] | Nserv [number of DRX cycles] |
| --- | --- |
| 0.32 | 3 |
| 0.64 | 3 |
| 1.28 | N/A |
| 2.56 | N/A |

If UE performs RSRP measurement based on RSS for serving cell, it is not expected to perform RSRP measurement based on CRS on that serving cell.

##### 4.7A.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage

The UE which supports wakeUpSignal-r15 shall meet the requirement defined for the DRX cycle length of N*DRX_cycle in Section 4.7A.2.1.1, provided the following conditions are met:

-	WUS has been configured in the serving cell using WUS-Config-r15, and

-	The serving cell measurement relaxation is signalled by the network using num-DRX-CyclesRelaxed, and

-	Serving cell S criteria is met with at least 2 dB margin.

-	The relaxed monitoring criteria for neighbour cells in TS 36.304 [1] clause 5.2.4.12.1 is fulfilled,

Editor’s note: FFS on following additional conditions when the UE is configured with ‘t-Service’ [2] in the serving cell:

-	[If the UE is not configured with eDRX_IDLE cycle and if the UE is configured with ‘t-Service’ [2] in the serving cell and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [4] DRX cycles before ‘t-Service-r17’, or

-	[If the UE is configured with eDRX_IDLE cycle and with ‘t-Service’ [2] in the serving cell and with eDRX_IDLE cycle, and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [1] eDRX cycle before ‘t-Service’. ]

Otherwise the requirements defined for the configured DRX cycle length in Section 4.7A.2.1.1 shall apply.

The UE shall further meet the requirements in section 4.7A.2.1.1 during time period T0 after following occasions:

-	after the end of reception of latest paging message, or

-	from the moment UE has switched from RRC_CONNECTED state to RRC_IDLE state.

T0 = N*DRX cycle if the UE is not configured with eDRX_IDLE cycle and T0 = one eDRX IDLE cycle if the UE is configured with eDRX_IDLE cycle.

The relaxation factor N is given by Table 4.7A.2.1.1A-1 if the UE is not configured with eDRX_IDLE cycle and by Table 4.7A.2.1.1A-2 if the UE is configured with eDRX_IDLE cycle when the serving cell is operated by GSO satellie. The relaxation factor N is given by Table 4.7A.2.1.1A-3 if the UE is not configured with eDRX_IDLE cycle and by Table 4.7A.2.1.1A-4 if the UE is configured with eDRX_IDLE cycle when the serving cell is operated by NGSO satellite.

Table 4.7A.2.1.1A-1: The relaxation factor N for a UE not configured with eDRX IDLE cycle when the serving cell is operated by GSO satellite

| DRX cycle length [s] | Value |
| --- | --- |
| 0.32 | Min(n , 32) |
| 0.64 | Min(n , 16) |
| 1.28 | Min(n , 8) |
| 2.56 | Min(n , 4) |
| Note n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |

Table 4.7A.2.1.1A-2: The relaxation factor N for a UE configured with eDRX IDLE cycle when the serving cell is operated by GSO satellite

| DRX cycle length [s] | Value |  |  |
| --- | --- | --- | --- |
|  | 1.28 ≤ PTW length [s] < 2.56 | 2.56 ≤ PTW length [s] < 5.12 | 5.12 ≤ PTW length [s] |
| 0.32 | Min(n , 2) | Min(n , 4) | Min(n , 8) |
| 0.64 | 1 | Min(n , 2) | Min(n , 4) |
| 1.28 | N/A | 1 | Min(n , 2) |
| 2.56 | N/A | N/A | 1 |
| Note: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |  |  |

Editor’s note: FFS on Table 4.7A.2.1.1A-3 and Table 4.7A.2.1.1A-4 when cycle when the serving cell is operated by NGSO satellite.

Table 4.7A.2.1.1A-3: The relaxation factor N for a UE not configured with eDRX IDLE cycle when the serving cell is operated by NGSO satellite

| DRX cycle length [s] | Value |
| --- | --- |
| 0.32 | Min(n , 16) |
| 0.64 | Min(n , 8) |
| 1.28 | Min(n , 4) |
| 2.56 | Min(n , 2) |
| Note: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |

Table 4.7A.2.1.1A-4: The relaxation factor N for a UE configured with eDRX IDLE cycle when the serving cell is operated by NGSO satellite

| DRX cycle length [s] | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.28 ≤ PTW length [s] < 2.56 | 2.56 ≤ PTW length [s] < 5.12 | 5.12 ≤ PTW length [s] |  |
| 0.32 | 1 | Min(n , 2) | Min(n , 4) |
| 0.64 | 1 | 1 | Min(n , 2) |
| 1.28 | N/A | 1 | 1 |
| 2.56 | N/A | N/A | 1 |
| Note: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |  |  |

##### 4.7A.2.1.2	Void

##### 4.7A.2.1.3	Void

##### 4.7A.2.1.4	Void

##### 4.7A.2.1.5	Void

#### 4.7A.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage

##### 4.7A.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage

The requirements in this subclause apply if UE is in the enhanced coverage area of the serving cell served by a satellite access node. The UE is considered to be in enhanced coverage area of serving cell according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot of the serving cell defined in Annex B.1.3 for a corresponding Band.

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in [1] for the serving cell at least every DRX cycle.

The UE is allowed to perform RSRP measurements based on RSS signals provided UE is configured with rss-ConfigCarrierInfo [2] and following conditions are met:

-	Serving cell RSS are available within the paging MPDCCH narrowband for Nserv successive DRX cycles and the last subframe of the RSS occasion is in the window [n-5, n-1] where n is the first subframe of paging MPDCCH narrowband, and

-	RSS power offset (PRSS)with respect to CRS as defined in RSS-Config [2], where PRSS ≥ 0 dB, and

-	UE is not configured with eDRX_IDLE cycle, and DRX cycle length is 0.32s or 0.64s.

If UE performs RSRP measurement based on RSS for serving cell, it is not expected to perform RSRP measurement based on CRS on that serving cell.

The UE shall filter the RSRP and RSRQ measurements of the serving cell using at least 4 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

If the UE is not configured with eDRX_IDLE cycle and has evaluated according to Table 4.7A.2.2.1-1 in Nserv_EC consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

If the UE is configured with eDRX_IDLE cycle and has evaluated according to Table 4.7A.2.2.1-2 in Nserv_EC consecutive DRX cycles within a single PTW that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

Editor’s note: FFS on following:

[If the UE is configured with ‘t-Service’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s in Table 4.7A.2.1.1-1 starting from at least [4] DRX cycles before ‘t-Service-r17’.

If the UE is not configured with eDRX_IDLE cycle and configured with DRX cycle ≥ [1.28] s then the UE shall meet the requirements defined for DRX cycle of [320] ms in Table 4.7A.2.2.1-1during at least 2 configured DRX cycles immediately after ‘t-ServiceStart-r17’ [2].

If the UE is configured with eDRX_IDLE cycle then the UE shall meet the requirements defined for eDRX_IDLE cycle of 5.12 s in Table 4.7A.2.2.1-2during 2 eDRX_IDLE cycles immediately after ‘t-ServiceStart-r17’ [2] regardless of the configured eDRX_IDLE cycle.]

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T, the UE shall initiate cell selection procedures for the selected PLMN as defined in [1], where T=20 s if the UE is not configured with eDRX_IDLE cycle, and T=MAX(20 s, one eDRX_IDLE cycle) if the UE is configured with eDRX_IDLE cycle.

When the UE is provided with ‘t-serviceStart-r17’ and has discontinuous coverage capabilities, then after ‘t-service-r17’ is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage. Editor’s note: Definition of in coverage is FFS.

Table 4.7A.2.2.1-1: Nserv_EC

| DRX cycle length [s] | Nserv_EC [number of DRX cycles] |
| --- | --- |
| 0.32 | 8 |
| 0.64 | 8 |
| 1.28 | 4 |
| 2.56 | 4 |

Table 4.7A.2.2.1-2: Nserv_EC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Nserv_EC [number of DRX or eDRX cycles Note 3] |
| --- | --- | --- | --- |
| 5.12 | N/A | N/A | 4 |
| 10.24 ≤ eDRX_IDLE cycle length ≤ 2621.44 | 0.32 | ≥1.28 (1) | 4 |
|  | 0.64 | ≥2.56 (2) | 4 |
|  | 1.28 | ≥5.12 (4) | 4 |
|  | 2.56 | ≥10.24(8) | 4 |
| Note 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s, number of DRX cycles otherwise. |  |  |  |

Nserv_EC defined in Table 4.7A.2.2.1-3 applies if serving cell is measured based on RSS.

Table 4.7A.2.2.1-3: Nserv_EC

| DRX cycle length [s] | Nserv [number of DRX cycles] |
| --- | --- |
| 0.32 | 5 |
| 0.64 | 5 |
| 1.28 | N/A |
| 2.56 | N/A |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

##### 4.7A.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage

The UE which supports wakeUpSignal-r15 shall meet the requirement defined for the DRX cycle length of N*DRX_cycle in Section 4.7A.2.2.1, provided the following conditions are met:

-	WUS has been configured in the serving cell using WUS-Config-r15, and

-	The serving cell measurement relaxation is signalled by the network using num-DRX-CyclesRelaxed, and

-	Serving cell S criteria is met with at least 2 dB margin.

-	The relaxed monitoring criteria for neighbour cells in TS 36.304 [1] clause 5.2.4.12.1 is fulfilled,

Editor’s note: FFS on following additional conditions when the UE is configured with ‘t-Service’ [2] in the serving cell:

-	[If the UE is not configured with eDRX_IDLE cycle and if the UE is configured with ‘t-Service’ [2] in the serving cell and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [4] DRX cycles before ‘t-Service-r17’, or

-	If the UE is configured with eDRX_IDLE cycle and with ‘t-Service’ [2] in the serving cell and with eDRX_IDLE cycle, and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [1] eDRX cycle before ‘t-Service’.]

Otherwise the requirements defined for the configured DRX cycle length in Section 4.7A.2.2.1 shall apply.

The UE shall further meet the requirements in section 4.7A.2.2.1 during time period T0 after following occasions:

-	after the end of reception of latest paging message, or

-	from the moment UE has switched from RRC_CONNECTED state to RRC_IDLE state.

T0 = N*DRX cycle if the UE is not configured with eDRX_IDLE cycle and T0 = one eDRX IDLE cycle if the UE is configured with eDRX_IDLE cycle.

The relaxation factor N is given by Table 4.7A.2.2.1A-1 if the UE is not configured with eDRX_IDLE cycle and by Table 4.7A.2.2.1A-2 if the UE is configured with eDRX_IDLE cycle when the serving cell is operated by GSO satellie. The relaxation factor N is given by Table 4.7A.2.2.1A-3 if the UE is not configured with eDRX_IDLE cycle and by Table 4.7A.2.2.1A-4 if the UE is configured with eDRX_IDLE cycle when the serving cell is operated by NGSO satellite.

Table 4.7A.2.2.1A-1: The relaxation factor N for a UE not configured with eDRX IDLE cycle when the serving cell is operated by GSO satellite

| DRX cycle length [s] | Value |
| --- | --- |
| 0.32 | Min(n , 32) |
| 0.64 | Min(n , 16) |
| 1.28 | Min(n , 8) |
| 2.56 | Min(n , 4) |
| Note: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |

Table 4.7A.2.2.1A-2: The relaxation factor N for a UE configured with eDRX IDLE cycle when the serving cell is operated by GSO satellite

| DRX cycle length [s] | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.28 ≤ PTW length [s] < 2.56 | 2.56 ≤ PTW length [s] < 5.12 | 5.12 ≤ PTW length [s] < 10.24 | 10.24 ≤ PTW length [s] |
| 0.32 | 1 | Min(n , 2) | Min(n , 4) | Min(n , 8) |
| 0.64 | N/A | 1 | Min(n , 2) | Min(n , 4) |
| 1.28 | N/A | N/A | 1 | Min(n , 2) |
| 2.56 | N/A | N/A | N/A | 1 |
| Note: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |  |  |  |

Editor’s note: FFS on Table 4.7A.2.2.1A-3 and Table 4.7A.2.2.1A-4 when cycle when the serving cell is operated by NGSO satellite.

Table 4.7A.2.2.1A-3: The relaxation factor N for a UE not configured with eDRX IDLE cycle when the serving cell is operated by NGSO satellite

| DRX cycle length [s] | Value |
| --- | --- |
| 0.32 | Min(n , 16) |
| 0.64 | Min(n , 8) |
| 1.28 | Min(n , 4) |
| 2.56 | Min(n , 2) |
| Note: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |

Table 4.7A.2.2.1A-4: The relaxation factor N for a UE configured with eDRX IDLE cycle when the serving cell is operated by NGSO satellite

| DRX cycle length [s] | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.28 ≤ PTW length [s] < 2.56 | 2.56 ≤ PTW length [s] < 5.12 | 5.12 ≤ PTW length [s] |  |
| 0.32 | 1 | Min(n , 2) | Min(n , 4) |
| 0.64 | 1 | 1 | Min(n , 2) |
| 1.28 | N/A | 1 | 1 |
| 2.56 | N/A | N/A | 1 |
| Note: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |  |  |

##### 4.7A.2.2.2	Void

##### 4.7A.2.2.3	Void

##### 4.7A.2.2.4	Void

##### 4.7A.2.2.5	Void

#### 4.7A.2.3	WUS receptions for UE category M1

The requirements defined in clause 4.7.2.3 apply for this section.

### 4.7A.3	Channel quality report for UE Category M1 in idle mode for Satellite Access

The requirements in this clause shall apply for the UE category M1, as defined in TS 36.321 [17] section 5.25, supporting DL channel quality reporting in an area of the serving cell served by a satellite node.

The DL channel quality provides the serving eNB with information about

-	the minimum MPDCCH repetition level to satisfy the hypothetical MPDCCH block error rate of 1% with the parameters specified in Table 4.7.3-1 if the repetition level in DL quality report is larger than 1, or

-	the minimum MPDCCH aggregation level to satisfy the hypothetical MPDCCH block error rate of 1% with the parameters specified in Table 4.7.3-2 if the repetition level in DL quality report is 1 and UE is in normal coverage.

Table 4.7.3-1: MPDCCH transmission parameters for downlink quality reporting, repetition number being reported

| Attribute | Normal coverage | Enhanced coverage |
| --- | --- | --- |
| DCI format | 6-1A | 6-1B |
| Starting OFDM symbols | 4; Bandwidth = 1.4MHz |  |
| MPDCCH Aggregation level (ECCE) Note2 | 24 |  |
| M-PDCCH Transmission type | Distributed |  |

Table 4.7.3-2: MPDCCH transmission parameters for downlink quality reporting, aggregation level being reported

| Attribute | Normal coverage |
| --- | --- |
| DCI format | 6-1A |
| Starting OFDM symbols | 4; Bandwidth = 1.4MHz |
| MPDCCH repetition level Note1 | 1 |
| M-PDCCH Transmission type | Distributed |

When UE is in idle mode, the reported MPDCCH repetition level or aggregation level shall be derived from the channel quality measured in the period T in the narrowband where the random access response is transmitted, where

-	T is the period from the beginning of the random access response to the beginning of PUSCH for DL channel quality reporting.

The MPDCCH repetition level or aggregation level is chosen from the supported MPDCCH repetition levels and aggregation levels [3]. The report mapping is defined in section 9.1.21.22.

The UE shall satisfy the downlink channel quality measurement accuracy requirements as specified in section 9.1.21.23 if UE is in normal coverage or in section 9.1.21.24 if UE is in enhanced coverage.

### 4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access

#### 4.7A.4.1	Introduction

The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation for transmitting in uplink using preconfigured uplink resources (PUR) as specified in [TS 36.331].

#### 4.7A.4.2	Requirements on UE synchronization for transmission using PUR

The requirements in this clause are applicable for the UE in normal coverage or in enhanced coverage area of the serving cell served by a satellite access node.

The UE is allowed to transmit using the preconfigured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the PUR transmission. The UE determines the PUR transmission occasion according to the received PUR configuration [TS 36.331].

#### 4.7A.4.3	Requirements on TA validation for transmission using PUR

The UE is allowed to transmit using PUR provided that TA is valid.  [ FFS for the validation conditions]

The UE shall determine the uplink timing,  TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

<End of Change 1>

<Start of Change 2>

## 5.5A	E-UTRAN Handover for Cat-M1 UEs for Satellite Access

### 5.5A.1	Introduction

This section defines the E-UTRAN intra-frequency handover requirements and inter-frequency handover and conditional handover requirements for Cat-M1 UEs for Satellite Access as required by TS 36.300 [25].

### 5.5A.2	Requirements in CEModeA

#### 5.5A.2.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs

The requirements in this clause are applicable to FDD intra-frequency handovers and FDD inter-frequency handovers for a Cat-M1 FDD UE in CEModeA. The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target SAN cell during Dhandover, otherwise interruption time may be longer than the requirements in clause 5.5A.2.1.2.

##### 5.5A.2.1.1	Handover delay

Procedure delays for all procedures that can command a handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying handover the UE shall finish the transmission of all repetitions of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command,

Where:

-	Dhandover equals the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2] plus the interruption time stated in clause 5.5A.2.1.2.

##### 5.5A.2.1.2	Interruption time

The interruption time is the time between the end of the last TTI containing the RRC command on the old PDSCH and the moment the UE has transmitted all repetitions of PRACH in the target cell, excluding the RRC procedure delay. This requirement applies when UE is not required to perform any synchronisation procedure before transmitting on the new PRACH.

When intra-frequency handover or inter-frequency handover is commanded and the field sameSFN-Indication and mib-RepetitionStatus [2] are included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

When intra-frequency handover or inter-frequency handover is commanded and the field sameSFN-Indication or mib-RepetitionStatus [2] is not included in the handover command then UE the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TMIB + TIU + 20 ms

Where:

-	Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Otherwise, Tsearch shall be according to the non-DRX cell identification requirements specified in Clause 8.13A.2.1 for intra-frequency handover for a UE configured with CEModeA or Tsearch shall be according to the non-DRX cell identification requirements specified in Clause 8.13A.2.2 for inter-frequency handover for a UE configured with CEModeA. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	TMIB is the time required for acquiring the MIB and SIB information of the target cell.

-	TIU is the time required to complete the transmission of PRACH in the target cell. The actual value of TIU shall depend upon the uncertainity in acquiring the first available PRACH occasion based on the PRACH configuration used in the target cell and the PRACH coverage enhancement level used by the UE for sending the random access preamble to the target cell.

-	In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal or longer than the time duration required for the cell identification. Otherwise, it is unknown. For intra-frequency handover the time duration required for the cell identification is specified in relevant intra-frequency cell identification requirements as described in Clause 8.13A.2.1 for CEModeA. For inter-frequency handover the time duration required for the cell identification is specified in relevant inter-frequency cell identification requirements as described in Clause 8.13A.2.2 for CEModeA.

Editor’s note: FFS  Tsi-to-epoch to accommodate for the time needed between the UE acquires the NTN SIB of the target cell and the epoch time conveyed in this SIB is reached.

#### 5.5A.2.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs

The requirements defined in clause 5.5A.2.1 are applicable to FDD intra-frequency handovers and FDD inter-frequency handovers for a Cat-M1 HD-FDD UE in CEModeA.

#### 5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers for a Cat-M1 FDD UE in CEModeA. The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target SAN cell during Dhandover, otherwise the measurement time, preparation time and interruption time may be longer than the requirements in clause 5.5A.2.3.2, 5.5A.2.3.4 and 5.5A.2.3.4.

##### 5.5A.2.3.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Dhandover = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tinterrupt

Where:

-	TRRC is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

-	TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

-	Tmeasure is the measurements time stated in clause 5.5A.2.3.2.

-	TCHO_execution is the conditional execution preparation time in clause 5.5A.2.3.3.

-	Tinterrupt is the interruption time stated in clause 5.5A.2.3.4.

##### 5.5A.2.3.2	Measurement time

The measurement time delay Tmeasure is defined as the time period from the end of TEvent_DU until the UE begins the preparation time for handover execution.

The measurement event evaluation delay measured without Time To Trigger (TTT) and L3 filtering shall be less than T identify intra defined in clause 8.13A.2.1 for intra-frequency conditional handover or Tidentify inter defined in clause 8.13A.2.2 for inter-frequency conditional handover. When TTT or L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.13A.2.1 or T identify inter defined in clause 8.13A.2.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TMeasurement_Period, Intra or TMeasurement_Period, Inter provided the timing to that cell has not changed more than ± 50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

##### 5.5A.2.3.3	Preparation time

Preparation time is the time when the UE prepares the target cell for conditional handover. It begins after measurement time.

-	TCHO_execution is the time needed for preparing the conditional handover to the target cell.

-	TCHO_execution can be up to 10 ms.

##### 5.5A.2.3.4	Interruption time

The interruption time is the time between when the UE completes the preparation time TCHO_execution and the time when the UE starts the transmission of the PRACH to the target cell.

When intra-frequency conditional handover or inter-frequency conditional handover is commanded and the field sameSFN-Indication and mib-RepetitionStatus [2] are included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = TIU + 20 ms

When intra-frequency contional handover or inter-frequency conditional handover is commanded and the field sameSFN-Indication or mib-RepetitionStatus [2] is not included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = TMIB + TIU + 20 ms

Where:

-	TIU is the time required to complete the transmission of PRACH in the target cell. The actual value of TIU shall depend upon the uncertainity in acquiring the first available PRACH occasion based on the PRACH configuration used in the target cell and the PRACH coverage enhancement level used by the UE for sending the random access preamble to the target cell.

-	In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal or longer than the time duration required for the cell identification. Otherwise, it is unknown. For intra-frequency handover the time duration required for the cell identification is specified in relevant intra-frequency cell identification requirements as described in Clause 8.13A.2.1 for CEModeA. For inter-frequency handover the time duration required for the cell identification is specified in relevant inter-frequency cell identification requirements as described in Clause 8.13A.2.2 for CEModeA.

-	TMIB is the time required for acquiring the MIB and SIB information of the target cell.

#### 5.5A.2.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs

The requirements defined in clause 5.5A.2.3 are applicable to FDD intra-frequency handovers and FDD inter-frequency handovers for a Cat-M1 HD-FDD UE in CEModeA.

### 5.5A.3	Requirements in CEModeB

#### 5.5A.3.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs

The requirements in this clause are applicable to FDD intra-frequency handovers and FDD inter-frequency handover for a Cat-M1 FDD UE configured with CEModeB.

##### 5.5A.3.1.1	Handover delay

Procedure delays for all procedures that can command a handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying handover the UE shall finish the transmission of all repetitions of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command,

Where:

-	Dhandover equals the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2] plus the interruption time stated in clause 5.5A.3.1.2.

##### 5.5A.3.1.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. This requirement applies when UE is not required to perform any synchronisation procedure before transmitting on the new PRACH.

When intra-frequency handover or inter-frequency handover is commanded and the field sameSFN-Indication and mib-RepetitionStatus [2] are included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

When intra-frequency handover or inter-frequency handover is commanded and the field sameSFN-Indication or mib-RepetitionStatus [2] is not included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TMIB + TIU + 20 ms

Where:

-	Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Otherwise, Tsearch shall be according to the non-DRX cell identification requirements specified in Clause 8.13A.3.1 for intra-frequency handover for a UE configured with CEModeB or Tsearch shall be according to the non-DRX cell identification requirements specified in Clause 8.13A.3.2 for inter-frequency handover for a UE configured with CEModeB. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	TMIB is the time required for acquiring the MIB and SIB information of the target cell.

-	TIU is the time required to complete the transmission of PRACH in the target cell. The actual value of TIU shall depend upon the uncertainity in acquiring the first available PRACH occasion based on the PRACH configuration used in the target cell and the PRACH coverage enhancement level used by the UE for sending the random access preamble to the target cell.

-	In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal or longer than the time duration required for the cell identification. Otherwise, it is unknown. For intra-frequency handover the time duration required for the cell identification is specified in relevant intra-frequency cell identification requirements as described in Clause 8.13A.3.1 for CEModeB. For inter-frequency handover the time duration required for the cell identification is specified in relevant inter-frequency cell identification requirements as described in Clause 8.13A.3.2 for CEModeB.

Editor’s note: FFS  Tsi-to-epoch to accommodate for the time needed between the UE acquires the NTN SIB of the target cell and the epoch time conveyed in this SIB is reached

#### 5.5A.3.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs

The requirements defined in clause 5.5A.3.1 are applicable to FDD intra-frequency handovers and FDD inter-frequency handovers for a Cat-M1 HD-FDD UE configured with CEModeB.

#### 5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers for a Cat-M1 FDD UE in CEModeA.

##### 5.5A.3.3.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Dhandover = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tinterrupt

Where:

-	TRRC is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

-	TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

-	Tmeasure is the measurements time stated in clause 5.5A.3.3.2.

-	TCHO_execution is the conditional execution preparation time in clause 5.5A.3.3.3.

-	Tinterrupt is the interruption time stated in clause 5.5A.3.3.4.

##### 5.5A.3.3.2	Measurement time

The measurement time delay Tmeasure is defined as the time period from the end of TEvent_DU until the UE begins the preparation time for handover execution.

The measurement event evaluation delay measured without Time To Trigger (TTT) and L3 filtering shall be less than T identify intra defined in clause 8.13A.3.1 for intra-frequency conditional handover or Tidentify inter defined in clause 8.13A.3.2 for inter-frequency conditional handover. When TTT or L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.13A.3.1 or T identify inter defined in clause 8.13A.3.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TMeasurement_Period, Intra or TMeasurement_Period, Inter provided the timing to that cell has not changed more than ± 50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

##### 5.5A.3.3.3	Preparation time

Preparation time is the time when the UE prepares the target cell for conditional handover. It begins after measurement time.

TCHO_execution is the time needed for preparing the conditional handover to the target cell.

TCHO_execution can be up to 10 ms.

##### 5.5A.3.3.4	Interruption time

The interruption time is the time between when the UE completes the preparation time TCHO_execution and the time when the UE starts the transmission of the PRACH to the target cell.

When intra-frequency conditional handover or inter-frequency conditional handover is commanded and the field sameSFN-Indication and mib-RepetitionStatus [2] are included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = TIU + 20 ms

When intra-frequency contional handover or inter-frequency conditional handover is commanded and the field sameSFN-Indication or mib-RepetitionStatus [2] is not included in the handover command then the interruption time shall be less than Tinterrupt

Tinterrupt = TMIB + TIU + 20 ms

Where:

-	TIU is the time required to complete the transmission of PRACH in the target cell. The actual value of TIU shall depend upon the uncertainity in acquiring the first available PRACH occasion based on the PRACH configuration used in the target cell and the PRACH coverage enhancement level used by the UE for sending the random access preamble to the target cell.

-	In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal or longer than the time duration required for the cell identification. Otherwise, it is unknown. For intra-frequency handover the time duration required for the cell identification is specified in relevant intra-frequency cell identification requirements as described in Clause 8.13A.3.1 for CEModeA. For inter-frequency handover the time duration required for the cell identification is specified in relevant inter-frequency cell identification requirements as described in Clause 8.13A.3.2 for CEModeA

-	TMIB is the time required for acquiring the MIB and SIB information of the target cell.

#### 5.5A.3.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs

The requirements defined in clause 5.5A.3.4 are applicable to FDD intra-frequency handovers and FDD inter-frequency handovers for a Cat-M1 HD-FDD UE configured with CEModeB.

<End of Change 2>

<Start of Change 3>

## 6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access

### 6.7A.1	Introduction

### 6.7A.2	Void

TUE_re-establish_delay=50 ms+i=1NfreqKsatellite,i*Tsearch+TSI_EUTRA-M1-CEModeA+TPRACH

Tsi-to-epoch

TUE_re-establish_delay=50 ms+i=1NfreqKsatellite,i*Tsearch+TSIEUTRA-M1-CEModeB+TPRACH

Tsi-to-epoch

<End of Change 3>

<Start of Change 4>

## 6.8A	RRC Connection Release with Redirection for UE Category M1 for Satellite Access

### 6.8A.1	Introduction

### 6.8A.2	Void

<End of Change 4>

<Start of Change 5>

## 8.13A	Measurements for UE Category M1 for Satellite Access

### 8.13A.1	Introduction

The UE category M1 applicability of the requirements in subclause 8.13 is defined in Section 3.6.

This clause contains requirements on the UE regarding measurement reporting in RRC_CONNECTED state for UE Category M1 for Satellite Access. The requirements are specified for E-UTRA intra- and inter-frequency measurements. These measurements may be used by the E-UTRAN, e.g. for handover decisions. The measurement quantities are defined in TS 36.214 [4], the measurement model is defined in TS 36.302 [22] and measurement accuracies are specified in clause 9. Control of measurement reporting is specified in TS 36.331 [2].

The UE shall meet the requirements in Section 8.13A, provided:

-	the UE does not require measurement gaps for the corresponding measurements, or

-	the UE requires measurement gaps for the corresponding measurements and is configured with the measurement gap pattern Id 0 or 1 and is not configured with any measurement gap pattern from Table 8.1.2.1-3.

When the UE is provided with IDC solution, the UE shall also perform RRM measurements and meet the corresponding requirements in clause 8.

### 8.13A.2	Void

### 8.13A.3	Void

<End of Change 5>