3GPP TSG-RAN WG4 Meeting #106	R4-2301988

Athens, Greece, 27 February – 3 March, 2023

Title: 	Reply LS on support of per FR PRS gap

Source: 	Huawei, HiSilicon

Agenda item:	11.1.3

Document for:	Approval

Introduction

RAN4 received LS [1] from RAN2#120 related to per-FR gap for PRS measurement. RAN2 asks RAN4 to provide feedback whether RAN4 supports PRS to be associated with per FR measurement gap in case of concurrent gaps.

In Rel-17, it was allowed to configure the perFR gap for PRS measurements. In the TS 38.306, the UE capability is also specified to indicate whether the UE supports two independent measurement gap configurations for FR1 and FR2 for PRS measurement. However, in the current version TS 38.331, the gapAssociationPRS IE can only be configured for per UE measurement gap. During RAN2 discussion on the above issue, there is no consensus whether the PRS measurement can be associated with per FR measurement gap when concurrent gaps are configured, as companies have different understanding whether RAN4 supports so. Therefore RAN2 kindly requests RAN4 to provide feedback whether RAN4 supports PRS to be associated with per FR measurement gap in case of concurrent gaps.

In this paper we will provide our views on the question in the LS and a draft reply.

Discussion

As mentioned in [1], RAN4 has introduced support of per-FR gap for PRS measurement in Rel-17 ePOS WI. The discussion was based on the Rel-16 gap scenario, i.e. UE is configured with at most one per-FR gap for FR1 and at most one per-FR gap for FR2. We understand in Rel-16 gap scenario, one or both of the per-FR gaps can be used for PRS measurement with the assumption that UE is not required to process more than one PFL at a time.

When con-MG is considered, UE can be configured with gap combinations as defined in Table 9.1.8-1 of 38.133, as copied below.

Based on con-MG agreements,

- For combination ID#3/4/5, PRS measurement will be performed in the per-UE gap.

- For combination ID#2, PRS measurement will be performed in one of the per-UE gap that is configured with IE gapAssociationPRS

The question in [1] is whether PRS measurement can be performed in any of the per-FR gaps for the gap combinations with more than one per-FR gaps in one FR, i.e. combination ID#0/1/6/7.

Table 9.1.8-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap patterns and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap | Per-UE measurement gap |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| 6 | 2 | 0 | 0 |
| 7 | 0 | 2 | 0 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. |  |  |  |

In our view, the answer is yes, provided that all PRS measurement in one FR are associated to a single gap in that FR. For example, with combination ID#0, one of the FR1 gaps and the FR2 gap can be configured with IE gapAssociationPRS. From PRS measurement perceptive, it is same as the Rel-16 gap scenario above.

If such association is not supported, it means UE cannot support PRS measurement with combination ID#0/1/6/7, and NW has to configure a per-UE gap with combination ID#3/4/5, so the benefits of per-FR gap for PRS measurement cannot be achieved with con-MGs.

Proposal: Support PRS to be associated with per FR measurement gap in case of concurrent gaps. All PRS measurements in one FR are associated to a single per FR gap for that FR.

Conclusions

In this paper we provided our views on the question in the LS [1].

Proposal: Support PRS to be associated with per FR measurement gap in case of concurrent gaps. All PRS measurements in one FR are associated to a single per FR gap for that FR.

A draft reply LS is provided in the Annex.

Reference

- R2-2213350, LS on support of per FR PRS gap, RAN2

Annex: draft Reply LS on support of per FR PRS gap

3GPP TSG-RAN WG4 Meeting #106-e R4-22xxxxx Athens, Greece, 27 February – 3 March, 2023 Title: Reply LS on support of per FR PRS gap Response to: R2-2213350 Release: Release 17 Work Item: NR_MG_enh-Core Source: RAN4 To: RAN2 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 thanks RAN2 for the questions in LS R2-2213350. Based on discussions in RAN4#106, RAN4 would like to confirm that RAN4 supports PRS to be associated with per FR measurement gap in case of concurrent gaps. In addition, RAN4 agreed that all PRS measurements in one FR are associated to a single per FR gap for that FR. RAN4 respectfully asks RAN2 to take the above information into account. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #106-bis-e Apr 17 – Apr 26, 2023 Online RAN WG4 Meeting #107 May 22 – May 26, 2023 Incheon, South Korea