3GPP TSG-RAN WG4 Meeting #106                                R4-2302093

Athens, Greece, 27 February– 3 March, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to introduce ATG UE Tx requirements

Agenda Item:	9.13.2.2

Document for:	Approval

Introduction

In this paper, we provide the following text proposals to capture some ATG UE Tx requirements referring to the WF [1].

- References

[1] R4-2220543, WF on ATG UE RF requirements, Huawei

- Text proposal

- <<Start of Change for TR 38.876>>

- 7.1.2 Tx requirements

- 7.1.2.1 Frequency error

The doppler frequency for ATG UE can be determined by,

fD =  cosθ*vc*f_c

Where,

- fD is Doppler frequency, f_c is the carrier frequency, which is set as 5GHz for n79.

- v is the speed of the aircraft which is 1200km/h.

- θ is the elevation angle between UE and gNB.

Since the potential maximum cell range is up to 200-300km [x] and the normal commercial airplane altitude is 10km, the elevation angle is only about 2~3 degrees when UE doing initial access at cell edge. Then the maximum DL doppler frequency is approaching 5.5kHz and the UL doppler frequency is approaching 11kHz if without any compensation which may cause not negligible impact for link performance and access successful rate. ATG UE needs to perform frequency compensation. The frequency accuracy requirement will be written assuming Doppler frequency pre-compensation.

The ATG BS location information could be broadcasted for UE to do frequency compensation if necessary. [UE shall rely on the ATG BS location broadcasted by the IE EphemerisInfo in NTN SIB 19 in 38.331 if pre-compensation is based on SIB19]. Where, Ephemeris may be expressed in format of position and velocity state vector.

- 7.1.2.2 MOP requirements

- 7.1.2.3 MPR/AMPR requirements

- 7.1.2.4 Configured transmitted power

- 7.1.2.5 Minimum output power

- 7.1.2.6 Transmit OFF power

- 7.1.2.7 Transmit ON/OFF time mask

- 7.1.2.8 Power control

- 7.1.2.9 Transmit modulation quality

- 7.1.2.10 Occupied bandwidth

The occupied bandwidth requirements specified in TS 38.101-1 can be reused for ATG UE.

- 7.1.2.11 SEM requirements

- 7.1.2.12 ACLR requirements

- 7.1.2.13 Spurious emission

The general spurious emission requirements specified in TS 38.101-1 can be reused for ATG UE.

- 7.1.2.14 Spurious emissions for UE co-existence

- 7.1.2.15 Transmit intermodulation

- <<End of Change>>