3GPP TSG-RAN WG4 Meeting #106                                R4-2302096

Athens, Greece, 27 February– 3 March, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to add some coexistence assumption and methodology

Agenda Item:	9.13.1.1

Document for:	Approval

Introduction

In this paper, we provide the following text proposals to capture some coexistence assumption and methodology [1].

- References

[1] R4-2220543, WF on ATG UE RF requirements, Huawei

- Text proposal

- <<Start of Change for TR 38.876>>

- 6.2.4 ACLR and ACS modelling

For DL it seems reasonable from the perspective of simulating worst case scenarios that we assume BS ACLR is modelled as flat in space, and the UE ACS can be modelled flat in space.

If this assumption is for DL, then the similar assumption could be made for the UL.

Therefore, it is assumed that both ACLR ( or the adjacent channel interference) and ACS are flat in both space and frequency. The ACIR model can be express as:

(Assuming ACLR, ACS and ACIR to be linear).

The ACLR/ACS requirements for TN are defined as below.

Table 6.2.4-1: ACLR/ACS requirements for TN

| NR TN system |  |  |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | ACS | 33 |

- <<Next of Change>>

- 6.2.6.3 ATG UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied for TN with following parameters.

where:

-	Pmax is ATG UE maximum output power (TRP) dBm,

-	Rmin is ATG UE minimum output power (TRP) dBm,

-	CLx-ile and γ are set as following:

-	CLx-ile = 10*log10(Pmax) – (SNRtarget + (-174+F+10*log(B)) )

-	SNRtarget is the targeted UL SNR (dB).

-    F is BS noise figure (dB).

-    B is UL transmission BW (Hz)

-	γ = 1 for uplink scenario.

The specific parameters are assumed as below in table 6.2.6.3.

Table 6.2.6.3-1 LOS probability

| UE UL power control parameters | 2GHz | 4GHz |
| --- | --- | --- |
| Target SNR | 15dB | 15dB |
| Pmax (TRP) | 40dBm | 23dBm for per polarization |
| BW | 20MHz | 100MHz |
| Rmin | -27dBm | -20dBm |
| NF for ATG BS | 5dB | 5dB |
| CLx_ile | 121dB | 97 |

- <<Next of Change>>

- 6.3 Co-existence simulation methodology

Adopt following simulation steps.

-	Step 1: Generate aggressor and victim networks.

-		One ATG site with one sector is dropped referring to clause 6.2.1.3

- 	Deployment of terrestrial network (19 cells with wraparound) refers to Table 6.2.1.2-1

- 	The relationship between TN and ATG can refer to clause 6.2.1.1

-	Step2: UE associations

-	TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

-	Deployment of ATG UE refers to clause 6.2.1.3.

-	Step 3: Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS/ATG BS-UE. This is done for both victim and aggressor networks.

-	Step 4: Throughput is computed in the victim systems without considering ACI as below:

ThputNO ACIbpshz=fSINRICI=fSN+IICI,

where:			IICI is the inter-cell interference.

-	Step 5: Throughput is computed considering ACI as below:

ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI,

where:			IACI is the adjacent channel interference.

-	Step 6: RF parameters are determined based on the degradation cause by ACI as below:

LossACI=1-ThputACIThputSINGLE

- <<End of Change>>