3GPP TSG-RAN WG4 Meeting #106	R4-2302131

Athens, Greece, Feb 27- Mar 3, 2023

Agenda Item:	11.1.3

Source: 	Ericsson

Title:	Reply LS on support of per-FR ePos gap

Document for:	Discussion

# Introduction

RAN2 has sent a LS to check whether RAN4 supports per-FR Pos gap if UE supports Con-MGs [1].  In this contribution, we will share our views on per-FR Pos gaps and prepare the draft of reply LS.

# Discussion

In our understanding, RAN4 has a common understanding that any enhancement of Pos gap is out of scope for concurrent gaps discussion in Rel-17.

RAN4 #101 meeting Issue 2-1-2: Whether PRS measurement for positioning can only be exclusively associated with only one of the instance of multiple gaps at least for R17 - Agreement - PRS measurement for positioning including all positioning frequency layers is associated with only one of the concurrent gaps - It is up to network whether to associate a concurrent gap only to PRS measurement RAN4 #102 meeting Issue 2-3-4: Number of configurable priority levels, if Option 1 in Issue 2-3-3 is agreed - Agreement from GTW - Only two levels are needed in the NR_MG_enh WI. However, considering forward compatibility on inter-working with other features (e.g., MUSIM, NTN, Positioning), RAN4 recommends 5 levels. RAN4 kindly requests that at least two priority levels are supported in Rel-17 and leaves the decision to support a higher number of priority levels to RAN2

Proposal 1: In Rel-17, the per-FR Pos gap is out of scope for concurrent gaps.

- Summary

In this contribution, we have discussed the Pos gap’s LS. Based on the discussions, we have made the following proposals:

Proposal 1: In Rel-17, the per-FR Pos gap is out of scope for concurrent gaps.

- References

- R4-2213350, “LS on support of per FR PRS gap”, Huawei

- Annex: draft LS response on support of per-FR POS gap

3GPP TSG-RAN WG4 Meeting #106-e R4-22xxxxx Athens, Greece, Feb 27- Mar 3, 2023 Title: Reply LS on support of per FR PRS gap Response to: R2-2213350 Release: Release 17 Work Item: support of per FR PRS gap Source: RAN4 To: RAN2 Cc: Contact Person: Name: Zhixun Tang #### E-mail Address: zhixun.tang@ericsson.com Attachments: - 1. Overall Description: RAN4 thanks the LS from RAN2 on support of per-FR PRS gap. During this RAN4 meeting, RAN4 further discussed the questions and achieved the following conclusions: [RAN4 Response]: From RAN4’s perspective, RAN4 doesn’t support PRS to be associated with per FR measurement gap for concurrent gaps. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: TSG-RAN4 #106-bis-e 17 Apr. – 26 Apr. 2023 Electronic Meeting TSG-RAN4 #107 May 22 – May 26, 2023 Incheon, Korea