3GPP TSG-RAN WG4 Meeting # 106	R4-2302171

# Athens, Greece, Februray 27th – March 3rd, 2023

Title: 	Updated simulation results on Adaptation Absolute Physical Layer requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.18.2

Document for:	Discussion

Background

In Rel-17, companies submitted the ATP simulation results which is well aligned. As per [1], it is agreed to submit simulation results if updated in this meeting.

- Companies are encouraged to provide (potentially updated) simulation results for next meeting for alignment. - Alignment shall be done based on all provided results for metrics as did in Rel-17 SI, i.e. including throughput v.s. SNR, BLER, median CQI and median RI with SNR range [0:2:20] for FR1 and [0:2:16] for FR2-1.

In this meeting, we provide our updated simulation results

Simulation results

Simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions Table

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

FR1 FDD

In this section we provide our simulation results for FR1 FDD.

Table 2.1-1: FR1 FDD 2Rx results (Throughput, CQI, RI, BLER statistics)

| SNR | Throughput | CQI | RI | BLER |
| --- | --- | --- | --- | --- |
| 0 | 3.37 | 3 | 1 | 0.03 |
| 2 | 4.81 | 3 | 1 | 0.04 |
| 4 | 6.45 | 4 | 1 | 0.05 |
| 6 | 8.56 | 5 | 1 | 0.04 |
| 8 | 11.07 | 6 | 1 | 0.03 |
| 10 | 13.53 | 7 | 1 | 0.04 |
| 12 | 16.10 | 7 | 1 | 0.05 |
| 14 | 19.00 | 8 | 1 | 0.05 |
| 16 | 21.47 | 9 | 2 | 0.08 |
| 18 | 24.59 | 9 | 2 | 0.08 |
| 20 | 26.95 | 9 | 2 | 0.12 |

Table 2.1-2: FR1 FDD 2Rx results summary (Tput% vs Target SNR)

| Tput | SNR |
| --- | --- |
| 10% | 4.0 |
| 15% | 6.9 |
| 20% | 9.5 |
| 25% | 12.0 |
| 30% | 14.3 |
| 35% | 16.7 |
| 40% | 19.0 |

Table 2.1-3: FR1 FDD 4Rx results (Throughput, CQI, RI, BLER statistics)

| SNR | Throughput | CQI | RI | BLER |
| --- | --- | --- | --- | --- |
| 0 | 5.12 | 4 | 1 | 0.12 |
| 2 | 7.43 | 4 | 1 | 0.07 |
| 4 | 9.14 | 4 | 2 | 0.08 |
| 6 | 13.23 | 5 | 2 | 0.04 |
| 8 | 17.29 | 5 | 2 | 0.02 |
| 10 | 20.88 | 6 | 2 | 0.02 |
| 12 | 24.45 | 7 | 2 | 0.04 |
| 14 | 29.50 | 8 | 2 | 0.03 |
| 16 | 33.85 | 9 | 2 | 0.03 |
| 18 | 37.94 | 10 | 2 | 0.04 |
| 20 | 42.42 | 11 | 2 | 0.02 |

Table 2.1-4: FR1 FDD 4Rx results summary (Tput% vs Target SNR)

| Tput | SNR |
| --- | --- |
| 10% | 1.2 |
| 15% | 4.3 |
| 20% | 5.8 |
| 25% | 7.4 |
| 30% | 9.2 |
| 35% | 11.0 |
| 40% | 12.5 |
| 45% | 13.8 |
| 50% | 15.3 |
| 55% | 16.8 |
| 60% | 18.3 |
| 65% | 19.8 |

FR1 TDD

In this section we provide our simulation results for FR1 TDD.

Table 2.2-1: FR1 TDD 2Rx results (Throughput, CQI, RI, BLER statistics)

| SNR | Throughput | CQI | RI | BLER |
| --- | --- | --- | --- | --- |
| 0 | 10.81 | 3 | 1 | 0.02 |
| 2 | 13.92 | 3 | 1 | 0.05 |
| 4 | 20.05 | 4 | 1 | 0.06 |
| 6 | 26.35 | 5 | 1 | 0.07 |
| 8 | 32.51 | 6 | 1 | 0.06 |
| 10 | 41.24 | 7 | 1 | 0.05 |
| 12 | 47.75 | 7 | 1 | 0.09 |
| 14 | 57.42 | 7 | 2 | 0.07 |
| 16 | 64.54 | 8 | 2 | 0.09 |
| 18 | 72.34 | 8 | 2 | 0.14 |
| 20 | 74.62 | 9 | 2 | 0.21 |

Table 2.2-2: FR1 TDD 2Rx results summary (Tput% vs Target SNR)

| Tput | SNR |
| --- | --- |
| 10% | 4.2 |
| 15% | 7.5 |
| 20% | 10.1 |
| 25% | 12.9 |
| 30% | 15.4 |
| 35% | 18.2 |

Table 2.2-3: FR1 TDD 4Rx results (Throughput, CQI, RI, BLER statistics)

| SNR | Throughput | CQI | RI | BLER |
| --- | --- | --- | --- | --- |
| 0 | 16.77 | 3 | 1 | 0.15 |
| 2 | 23.69 | 4 | 1 | 0.16 |
| 4 | 30.12 | 4 | 2 | 0.10 |
| 6 | 44.99 | 4 | 2 | 0.06 |
| 8 | 61.05 | 5 | 2 | 0.05 |
| 10 | 70.79 | 6 | 2 | 0.04 |
| 12 | 84.50 | 7 | 2 | 0.05 |
| 14 | 100.18 | 8 | 2 | 0.0+ |
| 16 | 114.72 | 9 | 2 | 0.05 |
| 18 | 124.33 | 10 | 2 | 0.09 |
| 20 | 130.99 | 11 | 2 | 0.14 |

Table 2.2-4: FR1 TDD 4Rx results summary (Tput% vs Target SNR)

| Tput | SNR |
| --- | --- |
| 10% | 1.1 |
| 15% | 4.1 |
| 20% | 5.5 |
| 25% | 6.9 |
| 30% | 8.2 |
| 35% | 10.3 |
| 40% | 11.8 |
| 45% | 13.1 |
| 50% | 14.5 |
| 55% | 15.9 |
| 60% | 18.0 |

FR2 TDD

In this section we provide our simulation results for FR2 TDD.

Table 2.3-1: FR2 TDD 2Rx results (Throughput, CQI, RI, BLER statistics)

| SNR | Throughput | CQI | RI | BLER |
| --- | --- | --- | --- | --- |
| 0 | 19.71 | 5 | 1 | 0.03 |
| 2 | 26.00 | 6 | 1 | 0.04 |
| 4 | 34.40 | 7 | 1 | 0.02 |
| 6 | 45.15 | 8 | 1 | 0.02 |
| 8 | 55.60 | 9 | 1 | 0.03 |
| 10 | 66.67 | 10 | 1 | 0.04 |
| 12 | 82.10 | 11 | 1 | 0.03 |
| 14 | 97.15 | 9 | 2 | 0.02 |
| 16 | 110.39 | 9 | 2 | 0.04 |

Table 2.3-2: FR2 TDD 2Rx results summary (Tput% vs Target SNR)

| Tput | SNR |
| --- | --- |
| 10% | 1.5 |
| 15% | 4.4 |
| 20% | 6.7 |
| 25% | 9.0 |
| 30% | 10.8 |
| 35% | 12.4 |
| 40% | 14.0 |
| 45% | 15.9 |

Conclusion

In this paper we provide our updated link level simulation results for ATP requirements

Reference

[1]   Way forward on physical layer throughput requirements. Intel.