3GPP TSG-RAN WG4 Meeting #106	R4-2302332

Athens, Greece, Feb 27 – Mar 3, 2023

Agenda item:	11.1.1.2

Source: 	Qualcomm Incorporated

Title: 	On the applicability of timing error margins for TEGs

Document for:	Approval

# Discussion

RAN2 has sent a Reply LS to RAN4 asking some questions about timing error margins [1]. The LS was discussed in RAN4#105 but several of the questions in the LS remain unanswered [4, 5]. Below we provide our view on the questions from RAN2 and related issues.

Question 1: Will there also be definition of applicable values of timing error margin for Tx TEG and/or for RxTx TEG, similar to that for Rx TEG?

RAN4 reached an initial agreement regarding applicability of timing error margins for RxTx TEGs [2]. The agreement contains FFS statements that need to be addressed. For now, RAN4 can inform RAN2 of the current agreement and that RAN4 is still discussing this issue.

Issue 2-1-2 Candidate timing error margin for RxTx TEG?

Agreements:

- (16 values): 1/2 Tc, 1 Tc, 2 Tc, 4 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 64 Tc, 80 Tc, 96 Tc, 128 Tc.

- The applicable timing error margin values that can be selected by the UE are the pre-defined values that are not larger than the sum of twice the Rel-16 group delay margin (dependent on PRS/SRS BW) and frequency drift margin

  - FFS on the frequency drift margin

  - FFS on “sum of twice the Rel-16 group delay margin and frequency drift margin”

For Tx TEGs, our view is that no applicability rule for timing error margin values is needed. Some companies proposed defining an applicability rule similar to the one defined for Rx TEGs [5]. However, there are no group delay margins associated with uplink transmissions. The group delay margins defined by RAN4 are associated with DL PRS-based measurements.

Proposal 1: RAN4 will not define an applicability rule for timing error margin values associated with Tx TEGs.

For RxTx TEGs, RAN4 needs to define an applicability rule for timing error margins that applies to new relative measurement accuracy requirements for UE Rx-Tx time difference that RAN4 agreed to introduce in Rel-17 [2]. This issue was discussed in RAN4#105 and there was good support for the proposal below. The frequency drift margin can be set to zero since clock drift should not be a significant source of error for UE Rx-Tx in the scenarios covered by Rel-17 requirements.

Proposal 2a: The applicable timing error margin values for a RxTx TEG that can be selected by the UE are the pre-defined values that are not larger than the sum of the Rel-16 group delay margins for the individual measurements and the frequency drift margin.

Observation 1: For Rel-16 UE Rx-Tx absolute measurement accuracy, no frequency drift margin was added.

Proposal 2b: For Rel-17 UE Rx-Tx relative measurement accuracy, the frequency drift margin is zero.

Question 2: Is the “applicability of timing error margin of Rx TEG” as included in the RAN4 LS (R4-2214493) supposed to be specified in LPP?

In our view, it is not necessary to include the above applicability statement in the LPP specification. Instead, it would be preferrable to add references to the associated measurement accuracy requirements in 38.133.

Observation 2: It is not necessary to capture the applicability of timing error margin values for Rx TEGs and RxTx TEGs in the LPP specification.

Proposal 3: Recommend to RAN2 to add references in LPP to the measurement accuracy requirements associated with TEGs in 38.133, clauses 10.1.23 (RSTD) and 10.1.25 (UE RxTx).

Question 3: Does the applicability of timing error margin of Rx TEG in the LS (R4-2214493) apply for UE Rx-Tx timing difference?

To address question 3, we refer to the prior RAN4 agreements shown below [2, 3]. In our view, the applicability of Rx TEG timing error margins defined by RAN4 (third agreement below) only applies in the context of the RSTD measurement accuracy requirements with TEG that RAN4 agreed to define in Rel-17 (first and second agreements below).

Issue 2-1-6 Accuracy requirements needed to be defined related to TEG?

Agreements:

- Define absolute RSTD measurement accuracy requirements when the measurements of reference cell and neighbor cell are within the same Rx TEG.

- When defining the absolute accuracy requirements of RSTD, the simulation results of RSTD in R16 can be reused.

- Discuss whether to define relative UE Rx-Tx accuracy requirements with timing error for the case where two measurements are in same RxTx TEG.

Issue 2-2-1 RSTD measurement accuracy requirements with TEG?

Agreements:

- For RSTD measurements where the reference cell and neighbour cell TOAs belong to the same Rx TEG, absolute measurement accuracy requirements are defined as the sum of the baseband accuracy derived from simulations and the Rx TEG timing error margin.

Issue 2-1-1 Applicability of timing error margin of Rx TEG?

Agreements:

- For Rx TEG, the applicable timing error margin values that can be selected by the UE are the pre-defined values that are not larger than the sum of the Rel-16 group delay margin (dependent on PRS/SRS BW) and frequency drift margin.

RAN4 has also agreed to define relative accuracy requirements for UE Rx-Tx for measurements associated with the same RxTx TEG. There is no agreement in RAN4 to introduce new requirements for UE Rx-Tx when measurements are reported with Rx TEGs.

Issue 2-2-2 Whether to define UE Rx-Tx accuracy and test case related to TEG?

Agreements:

- Define relative UE Rx-Tx accuracy requirements and corresponding test cases for the case where two measurements are in same RxTx TEG based on side condition with one cell is -6dB the other is -13dB by using the absolute RSTD simulation results from Rel-16.

#### Issue 2-2-2 Whether to define UE Rx-Tx accuracy and test case related to TEG?

Agreements:

- Define relative UE Rx-Tx accuracy requirements and corresponding test cases for the case where two measurements are in same RxTx TEG based on side condition with one cell is -6dB the other is -13dB by using the absolute RSTD simulation results from Rel-16.

Proposal 4: The applicability of timing error margin of Rx TEG in the LS (R4-2214493) does not apply to UE Rx-Tx timing difference measurements.

Proposal 5: Clarify the prior RAN4 agreement on the applicability of timing error margin values for Rx TEGs

- For RSTD measurements where the reference cell and neighbour cell TOAs belong to the same Rx TEG, the applicable timing error margin values that can be selected by the UE are the pre-defined values that are not larger than the sum of the Rel-16 group delay margin (dependent on PRS/SRS BW) and frequency drift margin.

# Conclusions

Proposal 1: RAN4 will not define an applicability rule for timing error margin values associated with Tx TEGs.

Proposal 2a: The applicable timing error margin values for a RxTx TEG that can be selected by the UE are the pre-defined values that are not larger than the sum of the Rel-16 group delay margins for the individual measurements and the frequency drift margin.

Observation 1: For Rel-16 UE Rx-Tx absolute measurement accuracy, no frequency drift margin was added.

Proposal 2b: For Rel-17 UE Rx-Tx relative measurement accuracy, the frequency drift margin is zero.

Observation 2: It is not necessary to capture the applicability of timing error margin values for Rx TEGs and RxTx TEGs in the LPP specification.

Proposal 3: Recommend to RAN2 to add references in LPP to the measurement accuracy requirements associated with TEGs in 38.133, clauses 10.1.23 (RSTD) and 10.1.25 (UE RxTx).

Proposal 4: The applicability of timing error margin of Rx TEG in the LS (R4-2214493) does not apply to UE Rx-Tx timing difference measurements.

Proposal 5: Clarify the prior RAN4 agreement on the applicability of timing error margin values for Rx TEGs

- For RSTD measurements where the reference cell and neighbour cell TOAs belong to the same Rx TEG, the applicable timing error margin values that can be selected by the UE are the pre-defined values that are not larger than the sum of the Rel-16 group delay margin (dependent on PRS/SRS BW) and frequency drift margin.

# References

[1] R2-2210977, Reply LS on applicability of timing error margin of Rx TEG, RAN2#119bis-e

[2] R4-2214492, WF on NR Positioning Enhancements (Part 2), RAN4#104-e

[3] R4-2210602, WF on NR Positioning Enhancements (Part 2), RAN4#103-e

[4] R4-2220729, Reply LS on applicability of timing error margin of Rx TEG, RAN4#105

[5] R4-2220080, Topic summary for [105][234] Reply_LS_2, RAN4#105