3GPP TSG-RAN WG4 Meeting #106	R4-2302364

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	9.22.3.1

Source:	MediaTek Inc.

Title:	Discussion on DL interruption for Tx switching across 3&4 bands

Document for:	Discussion

# Introduction

In the last meeting, the RRM requirements for Multi-carrier enhancements WI were discussed and several agreements have been achieved as captured in the WF [1]. In this paper we further discuss the remaining issues in DL interruption for Tx switching across 3/4 bands and provide our view.

# Discussion

In the last meeting, RAN4 agreed to use RTD=6us [1] for non-collocated inter-band scenario for Tx switching with 2 TAGs. Therefore, the DL interruption length for R18 Tx switching across 3/4 bands with 2 TAGs can be calculated using the same principle used in R16/R17 as shown below:

|  | NR Slot length (ms) | Uplink Tx switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 8 |
| 2 | 0.25 | 4 | 10 | 14 |

As it can be noticed from above table, only one value (in red) differs from R16/R17 DL interruption length. However, in the last meeting, the table shown below was agreed as captured in the WF [1]:

<Agreement>: Issue 1-3: DL interruption length for 2TAGs Agreements on GTW: - Use the following DL interruption length (in symbols) for R18 Tx switching across 3/4 bands with 2 TAGs for RTD = 6us NR Slot length (ms); Uplink Tx switching period. 35us; 140us; 210us. 0; 1; 2; 3; 4. 1; 0.5; 3; 6; [7 or 8]. 2; 0.25; 4; 10; 14.

The tentative value between bracket indicates whether to reuse the same value used in R16/R17 (i.e., 7) or to update it to 8 symbols. In our view, and for sake of simplifying the requirements, it is fine to keep this value same as the legacy R16/R17, which means R16/R17 DL interruption length can be reused for R18 Tx switching across 3/4 bands with 2 TAGs.

Proposal 1: DL interruption length in R16/R17 can be reused for R18 Tx switching across 3/4 bands with 2 TAGs.

# Summary

In this contribution, the following proposal is concluded:

Proposal 1: DL interruption length in R16/R17 can be reused for R18 Tx switching across 3/4 bands with 2 TAGs.

# References

- R4-2220417, WF on RRM requirements for NR Multi-carrier enhancements, Huawei, HiSilicon.