3GPP TSG-RAN WG4 Meeting #106	R4-2302391

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	9.13.4.1

Source:	Ericsson

Title:	Discussions on general ATG RRM requirements

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we revisit the general open issues identified in [1] and provide our updated view on topic.

# 2	Discussion

## Frequency bands

The following three frequency bands are agreed to be specified for ATG according to the approved WID [2]:

- Band n1

- Band n78

- Band n79

It is noted that these bands already exist in the NR operating band groups defined in Table 3.5.2-1 in TS 38.133 as shown below:

Table 1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL |  | NR CCA10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation | Operating bands |
| A | NR_FDD_FR1_A | n1, n18, n24, n70, n744, n91, n92, n93, n94, n100 | NR_TDD_FR1_A | n34, n389, n39, n40, n50, n51, n53, n101 | NR_SDL_FR1_A | n67, n75, n76 | NR_CCA_FR1_A | - |
| B | NR_FDD_FR1_B | n65, n66, n743 | NR_TDD_FR1_B | n387 | NR_SDL_FR1_B | - | NR_CCA_FR1_B | - |
| C | NR_FDD_FR1_C | n30 | NR_TDD_FR1_C | n48, n771, n78, n79 | NR_SDL_FR1_C | - | NR_CCA_FR1_C | - |
| D | NR_FDD_FR1_D | n28 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - | NR_CCA_FR1_D | - |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41, n90 | NR_SDL_FR1_E | - | NR_CCA_FR1_E | - |
| F | NR_FDD_FR1_F | n266 | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - | NR_CCA_FR1_F | - |
| G | NR_FDD_FR1_G | n3, n8, n12, n13, n14, n20, n71, n85, n10511 | NR_TDD_FR1_G | n104 | NR_SDL_FR1_G | n29 | NR_CCA_FR1_G | - |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - | NR_CCA_FR1_H | - |
| I | NR_FDD_FR1_I | - | NR_TDD_FR1_I |  | NR_SDL_FR1_I | - | NR_CCA_FR1_I | n46 |
| J | NR_FDD_FR1_J | - | NR_TDD_FR1_J | n478, n96 | NR_SDL_FR1_J | - | NR_CCA_FR1_J | n96 |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. NOTE 6: The minimum Io condition is reduced by 0.5 dB when the carrier frequency of the assigned NR channel bandwidth is within 865-894 MHz. NOTE 7: When this band is only used for V2X SL service, the band is exclusively used for NR V2X in particular regions. NOTE 8: This band is unlicensed band used for V2X service. There is no expected network deployment in this band. NOTE 9: When this band is only used for WAN service. NOTE 10: Operating bands where operation on carrier frequencies with CCA is supported. NOTE 11: The minimum Io condition is reduced by 0.5 dB when the downlink channel overlap the 612-617 MHz frequency range and the channel bandwidth is 5 MHz. |  |  |  |  |  |  |  |  |

RAN4 needs to discuss whether new frequency band grouping table is needed for ATG. In our opinion, the existing tables with same band notation can be reused provided the UE requirements on the REFSENS do not change for the ATG bands. Otherwise, new ATG specific tables need to be introduced. Since RF group has not yet agreed on the REFSENS requirements for the ATG bands, it is proposed to monitor the RF agreement and further revisit the RRM discussions on need for separate frequency band grouping table for ATG.

- Observation #1: Frequency band grouping table is sorted based on the REFSENS requirements.

- Observation #2: RF group has not agreed on the REFSENS requirements for ATG bands.

Proposal 1	RAN4 to revisit the discussions on need for separate frequency band grouping table after REFSENS has been agreed for ATG bands.

## Specification impact

RAN4 has made good progress in the following ATG RRM topics:

- Radio link monitoring

- Link recovery

- Active BWP switching

- L1-RSRP measurements

- Active TCI state switching

- Pathloss reference signal switching

It is now time to work on the CR to capture those agreements. Our view is to define the ATG specific requirements in separate sections since there are still important differences between the legacy NR requirements and ATG specific requirements although some requirements are identical. For instance, the legacy requirements contain CA/DC specific requirements in same sections and inter-RAT requirements which are not applicable to ATG. In addition, defining the ATG specific requirements in separate sections facilities specification readability as well as further enhancements to ATG.

Proposal 2	ATG specific RRM requirements are defined in separate sections in TS 38.133.

## Text proposal for TR

Text proposal to TR 38.876 was discussed in the last meeting and companies had different views on the need to capture RRM part in the TR. Our view is to avoid duplicating the RRM requirements from TS 38.133 to the TR, but instead to add a high-level description to the ATG RRM requirements introduced in TS 38.133 and further include reference to TS 38.133. This way of working is less prone to mistakes and inconsistencies between the TR and TS. A skeleton for the RRM part of TR is provided below which is to be updated based on progress of the work.

Proposal 3	RAN4 to adopt the following the skeleton for TR 38.876 for RRM section.

# 8 RRM requirements ## 8.1 Applicability of ATG RRM requirements <To describe the scenario and conditions under the ATG RRM requirements apply.> ## 8.2 Types of requirements defined <To list and refer to the type of ATG RRM requirements that are defined in TS 38.133.>

# Conclusion

In this contribution we have discussed the general aspects of ATG requirements based on the issues discussed at last meeting. We have made the following observations:

- Observation #1: Frequency band grouping table is sorted based on the REFSENS requirements.

- Observation #2: RF group has not agreed on the REFSENS requirements for ATG bands.

Proposal 1	RAN4 to revisit the discussions on need for separate frequency band grouping table after REFSENS has been agreed for ATG bands.

Proposal 2	ATG specific RRM requirements are defined in separate sections in TS 38.133.

Proposal 3	RAN4 to adopt the following the skeleton for TR 38.876 for RRM section.

# 8 RRM requirements ## 8.1 Applicability of ATG RRM requirements <To describe the scenario and conditions under the ATG RRM requirements apply.> ## 8.2 Types of requirements defined <To list and refer to the type of ATG RRM requirements that are defined in TS 38.133.>

# References

R4-2220361, “WF on NR ATG RRM requirements”, CMCC

RP-221369, “Revised WID on Air-to-ground network for NR”, CMCC