3GPP TSG-RAN WG4 Meeting # 106	R4-2302487

# Athens, Greece, February 27 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0350 | rev | - | Current version: | 15.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to clarify and align random precoder configuration for PDSCH/PDCCH requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2023-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Precoder configuration for PDCCH, PDSCH, PDSCH DMRS is not clearly captured in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify precoder configuration with number of Tx = 1; Clarify random precoder behavior in time and frequency |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Precoder configuration for PDCCH, PDSCH, PDSCH DMRS will not be clearly captured in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | #### 5.2, 5.3, 5.5A.1 7.2, 7.3. 7.5A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.2	PDSCH demodulation requirements

The parameters specified in Table 5.2-1 are valid for all PDSCH tests unless otherwise stated.

Table 5.2-1: Common test parameters

-----------------Start of Change 1---------------------

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| [… omitting unchanged cells …] |  |  |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH | Symbols | 0, 1 |
|  | Number of PRBs in CORESET |  | Table 5.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
|  | CCE-to-REG mapping type |  | Non-interleaved |
|  | DCI format |  | 1_1 |
|  | TCI state |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I; Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1, i2 combination; |
| [… omitting unchanged cells …] |  |  |  |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1:Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1, i2 combination |

-----------------End of Change 1---------------------

## 5.3	PDCCH demodulation requirements

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The parameters specified in Table 5.3-1 are valid for all PDCCH tests unless otherwise stated.

-----------------Start of Change 2---------------------

Table 5.3-1: Common test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| [… omitting unchanged cells …] |  |  |
| PDCCH Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1, i2 combination |
| [… omitting unchanged cells …] |  |  |

----------------- End of Change 2 ---------------------

## 5.5A	Sustained downlink data rate provided by lower layers

### 5.5A.1	FR1 CA requirements

[…]

----------------- Start of Change 3 ---------------------

Table 5.5A-1: Common test parameters for FDD and TDD component carriers

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| [… omitting unchanged cells …] |  |  |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH |  | Symbols #0 |
|  | Number of PRBs in CORESET |  | Table 5.5A-4 |
|  | Number of PDCCH candidates and aggregation levels |  | 1/AL 1 for 30 kHz / 5 MHz 1/AL4 for 15 kHz / 5 MHz, 30 kHz / 10 MHz and 30 kHz / 15 MHz 1/AL 8 for other combinations |
|  | CCE-to-REG mapping type |  | Non-interleaved |
|  | DCI format |  | 1_1 |
|  | TCI State |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For Number of Tx = 2: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder index 0 and 2 For Number of Tx= 4: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2} |
| [… omitting unchanged cells …] |  |  |  |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1, i2 combination; |
| [… omitting unchanged cells …] |  |  |  |

-----------------End of Change 3---------------------

## 7.2	PDSCH demodulation requirements

The parameters specified in Table 7.2-1 are valid for all PDSCH demodulation tests unless otherwise stated.

Table 7.2-1: Common Test Parameters

-----------------Start of Change 4---------------------

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| [… omitting unchanged cells …] |  |  |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH |  | 0 |
|  | Number of PRBs in CORESET |  | Table 7.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
|  | CCE-to-REG mapping type |  | Non-interleaved |
|  | DCI format |  | 1_1 |
|  | TCI state |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1, i2 combination |
| [… omitting unchanged cells …] |  |  |  |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection with Wideband size and updated per slot, with equal probability of each applicable i1, i2 combination |

-----------------End of Change 4---------------------

## 7.3	PDCCH demodulation requirements

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The parameters specified in Table 7.3-1 are valid for all PDCCH tests unless otherwise stated.

-----------------Start of Change 5---------------------

Table 7.3-1: Common test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| [… omitting unchanged cells …] |  |  |
| PDCCH & PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1, i2 combination |
| [… omitting unchanged cells …] |  |  |

-----------------End of Change 6---------------------

## 7.5A	Sustained downlink data rate provided by lower layers

### 7.5A.1	FR2 CA requirements

[…]

-----------------Start of Change 6---------------------

Table 7.5A.1-1: Test parameters for FR2 TDD

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| [… omitting unchanged cells …] |  |  |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH |  | Symbols #0 |
|  | Number of PRBs in CORESET |  | Table 7.5A.1-2 |
|  | Number of PDCCH candidates and aggregation levels |  | 1/8 |
|  | CCE-to-REG mapping type |  | Non-interleaved |
|  | DCI format |  | 1-1 |
|  | TCI State |  | TCI state #1 |
|  | PDCCH &PDCCH DMRS Precoding configuration |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder index 0 and 2 |
| [… omitting unchanged cells …] |  |  |  |