3GPP TSG-RAN WG4 Meeting # 106											                  R4-2302878

Athens, Greece, 27th Feb – 3rd March 2023

Agenda item:			9.25.6

Source:	Moderator (Samsung)

Title:	WF for above 10GHz NTN-TN co-existence study

Document for:	Approval

Introduction

This document provides the way forward on co-existence study NR NTN enhancement work item in Rel-18.

Topic #1: Coexistence scenario

Agreement

Following agreements have been made:

- Study NTN-TN coexistence by assuming a reference frequency of 17 GHz for NTN DL cases and 27 GHz NTN UL cases, as well as the consideration of ACLR and ACS assumptions as following:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |

Furthermore, for the assumption on TN ACLR/ACS for co-existence simulation, the values for 17GHz in below table are considered as starting point for co-existence simulation purpose yet other options not precluded.

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | [19] | [25] | [23.8] | [18.2] |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

- RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands with the following figures:

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

Open issues

N/A

Topic #2: Network layout model

Agreement

Following agreements have been made:

- For TN Urban area deployment, 20% active ratio can be applied to urban areas.

- Further discuss whether NTN UEs could be dropped within TN cluster taking into account relative discussion of Issue 3-3 Number of active UE (UL).

Open issues

Issue 2-1:  Network and UE deployment

- Proposals

  - Option 1(Ericsson, CATT): Agree on the deployment an observation rules as listed in Annex 1 (when NTN UEs are dropped at TN cluster edges) and Annex 2 (when NTN UEs are dropped in TN).

  - Option 2(ZTE): Based on Option 1, for NTN UE deployment, it can be dropped within TN clusters based on the deployment scenario, and NTN UE could be placed within TN cluster based on the deployment scenario.

  - Option 3(Qualcomm): To drop VSAT inside the TN cluster and to study ESIM on the ground e.g., on top of train that also to be dropped in the TN cluster because it can be considered as worst case compared to Aeronautical or Maritime ESIM.

| No. | Combination | Aggressor | Victim | Contribution | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | NTN UE ACLR (NTN UE to BS) | Worst case should be when distance is very small between VSAT or ESIM and the BS. Propose to drop VSAT inside the TN cluster Propose to study ESIM on the ground e.g., on top of train and to be dropped in the TN cluster because it can be considered as worst case compared to Aeronautical or Maritim ESIM |
| 2 | TN with NTN | TN UL | NTN UL | NTN SAN ACS (TN UE to Sat) | Propose to use same assumption of activity factor as in [2] with 20% for Urban as it will be the worst case |
| 3 | TN with NTN | NTN UL | TN DL | NTN UE ACLR (NTN UE to TN UE) | Propose to use same assumption as case 1, the deployment of VSAT/ESIM should be in close distances with TN UEs |
| 4 | TN with NTN | TN DL | NTN UL | NTN SAN ACS (BS to Sat) | Propose to use same assumption as in case 2 of activity factor with 20% for Urban Noting that the footprint of GEO beam at that frequency is smaller compared to S-band, so the number of active clusters will be much lower compared to before as in [2] |
| 5 | TN with NTN | TN DL | NTN DL | NTN UE ACS (BS to NTN UE) | Same as above |
| 6 | TN with NTN | NTN DL | TN DL | NTN SAN ACLR (Sat to TN UE) | Same as above |
| 7 | TN with NTN | NTN DL | TN UL | NTN SAN ACLR (Sat to BS) | Same as above |
| 8 | TN with NTN | TN UL | NTN DL | NTN UE ACS (TN UE to NTN UE) | Same as above |

Topic #3: System parameters

Agreement

Following agreements have been made

- For calibration only, the NTN GEO and LEO SAN Noise Figure is set as 5.9dB.

- NTN DL parameters in the table below is used with the circular aperture type antenna pattern defined in Section 6.4.1 of TR 38.811 for co-ex study as the starting point. Simulation with phased array antenna type is not excluded.

Ka-Band DownLink (i.e. ~20 GHz for DL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 5,0 | 0,5 | 0,5 |
| Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |
| Satellite Tx max Gain (dBi) | 58,5 | 38,5 | 38,5 |

- For calibration, the number of active NTN UE (UL) is 10 UEs with divided RBs per UE by channel bandwidth.

- Following assumptions of NTN UE height are used as the starting points

| NTN UE scenario | Fixed VSAT (on roof) | M-ESIM | A-ESIM | L-ESIM |
| --- | --- | --- | --- | --- |
| Altitude | 22.5m | 22.5m | 3-14km | 1.5m |

- For calibration, NTN UE antenna elevation angle is 90 degree.

- TN parameters of Dense Urban Micro base station will be considered with second priority and the density is yet to be discussed.

- Both parabolic antenna and phased array antenna for NTN SAN and UE are considered as the starting point of co-existence simulation.

- Parameters of above mentioned two types of antennas will be defined seperately.

- To reduce the workload of calibration, consider parabolic antenna for GEO and phased array antenna for LEO.

- Adopt the following TN BS antenna parameters:

- Adopt 90 degree for 0.5 wavelengths Horizontal/Vertical element spacing.

- Adopt 5.5dBi for 90-degree horizontal/vertical 3dB beamwidth.

|  | Macro urban |
| --- | --- |
| Base Station Antenna Characteristics |  |
| Antenna pattern | TR 38.803 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5  dv = 0.5  |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Mechanical downtilt (degrees) | 10 |

- To update the UE maximum output power from 23dBm to UE peak EIRP as 22.4dBm for both Urban and Dense Urban

- To assume the TN ISD with 300m for the 17 GHz frequency range.

Open issues

Issue 3-5: Horizontal boresight and Vertical tilt assumptions of NTN UEs

- FFS following options in next meeting:

  - Option 1 (Samsung):

|  | GEO | LEO |
| --- | --- | --- |
| Antenna vertical tilt | 20 degree above horizontal | 30 degree above horizontal |
| Antenna horizontal boresight | Following Serving satellite or random between (0-360) | Following Serving satellite or random between (0-360) |

  - Option 2 (ZTE):

      - Fixed tilt values is not needed.

      - FFS for A-ESIM in taking-off and landing period, as well as portable stations.

      - prefer to go with option 2 with the assumption of 2/3-Axis Stabilized + Auto Skew operation platform with assistance information Built in Gyro or GPS information

  - Option 3 (CATT): fixed tilt values is not needed.

  - Option 4 (Qualcomm): To study different elevation angles and to consider the worst-case assumptions for the NTN UE (VSAT) vertical tilt.

Issue 3-6: NTN VSAT devices antenna pointing accuracy assumption

- FFS following options in next meeting

  - Option 1(Ericsson): RAN4 should define an antenna pointing accuracy assumption when considering NTN VSAT devices.

  - Option 2 : No

Issue 3-7: Methodology of simulate NTN VSAT (fixed) devices

- FFS following options in next meeting

  - Option 1(Ericsson): RAN4 should further discuss how to simulate NTN VSAT (fixed) devices

  - Option 2: Other

Issue 3-8: Parameters for Ka-Band UpLink (i.e. ~27 GHz for UL) for different satellite orbits

- FFS following options in next meeting, noting that these parameters are to be used in co-existence simulation.

  - Option 1 (Thales): The following set of SAN parameters are proposed for Ka-Band in UL (Updated Set-1 from TR 38.821)

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 3,33 | 0,33 | 0,33 |
| G/T max (dB/K) | 30,4 | 10,4 | 10,4 |
| Satellite RX max Gain (dBi) | 58,5 | 38,5 | 38,5 |
| Earth temperature (K) | 290 | 290 | 290 |
| Satcom Repeater Noise Figure (dB) | 3,5 | 3,5 | 3,5 |

  - Option 2: other

Issue 3-11: Ka-Band SAN Antenna pattern

- FFS following options:

  - Option1 & 2

| Options | Equations |
| --- | --- |
| Option 1(Ericssion) | Fθ=2 J1uu |
| Option 2(Thales) | Fθ=232 J1uu+4 J2uu2 |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

  - Option 3(Huawei):

| SAN phased array antenna Tx characteristics |  |  |
| --- | --- | --- |
| 1 | Ka band SAN phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 2) | 4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Row × Column)(Note 4) | 441 elements (21*21) |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 10dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 54 degrees |
| 1.13 | normal direction | Toward Z- axis |
| Note 1: Void Note 2: The element gain in row 1.2 includes the loss given in row 1.9. Note 3: The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element). Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements. |  |  |

| SAN phased array antenna Rx characteristics |  |
| --- | --- |
| Rx antenna gain | Same as Tx antenna gain |
| Noise figure | 9 |
| Rx Feeder loss | 2dB |
| Max G/T figure of merit | -5 (dB/K) (Note) |
| NOTE: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB |  |

Issue 3-12: Parabolic Antenna for NTN UE

- FFS following options:

Moderator’s note: New proposals have been added in corresponding legacy Options, using color marks indicating contributors of those elements.

| Characteristics | Option 1 (Ericsson, Samsung, ZTE, CATT) | Option 2 (Thales) |  |
| --- | --- | --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |  |  |
| Polarisation | Circular | Circular |  |
| Efficiency |  | UL 60%, DL 57% |  |
| Rx Antenna gain | 39.7 dBi | 39 dBi |  |
| Noise figure | 1.2 dB | Option 2-1: | Option 2-2: |
|  |  | 1.2 dB | 2.1 dB (equivalent) |
| Rx Feeder loss | -0.5dB | -0.5 dB | -0.5 dB |
| Antenna temperature | 150 K | 40 K (Note: T_sky + T_ground) | N/A |
| Sky temperature | N/A | 30 K | N/A |
| Ground temperature | N/A | 10 K | N/A |
| G/T figure of merit | 15.35 (dB/K) (Note 2) | 16.5 dB/K | N/A |
| Tx transmit power | 2 W (33 dBm) | 2W (3dBW) |  |
| Tx antenna gain | 43.2 dBi | 42.9 dBi |  |
| Output loss | N/A | -1.0 |  |
| EIRP | 45.2 dBW | 44.9 dBm |  |
| UE height | FFS |  |  |
|  | NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna NOTE 2: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB |  |  |

  - Option 3(Huawei): noise figure value of NTN UE to 9dB

Issue 3-13: Phased Array Antenna for NTN UE

- FFS following options:

  - Option 1-3(Samsung, CATT, ZTE):

| Characteristics | ESIM (TR 38.821) | ESIM (TR 38.821),CATT | ESIM (ITU) |
| --- | --- | --- | --- |
| Antenna type and configuration | Directional (M,N,P,Mg,Ng) = (128/64,128/32,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=65/90 deg) | Directional (M,N,P,Mg,Ng) = (128,128,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=90 deg) | Antenna pattern: S.580-6 |
| Polarisation | Linear: +/-45°X-pol | Linear: +/-45°X-pol |  |
| Element gain including loss | 5.5dBi | 3 dBi per element |  |
| Rx Antenna gain | 45.3dBi/ 3dBi per elemant | 45dBi(per polarization) | 45 dBi |
| Antenna temperature | 150 K | 150 K |  |
| Noise figure | 1.2 dB/ 11/13dB | 1.2 dB/ 11/13dB |  |
| Tx transmit power | Transmit power density（dBW/Hz）-46.3/ 10*log10(64*32)+7 dBm) | 10*log10(128*128)+7 dBm | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain | 45.3dBi (per polarization)/ 3 dBi per element | 45dBi(per polarization) | 45 dBi |

  - Option 4 (Thales):

- RAN4 shall use for coexistence studies the NTN UE antenna pattern as described by the following relation:

Fu=2J1uu

where:

- J1x  is the Bessel function of first type and first order with argument x,

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ),

- D : antenna diameter,

- : wavelength.

The normalised antenna pattern, expressed in decibels, is given by the following relation:

FudB=10 logFu2

- With respect to NTN UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-6:

The following reference radiation patterns should be adopted for angles between the direction considered and the axis of the main beam for frequencies in the range from 2 to 31 GHz:

G    32  –  25 log 		dBi		for  min          48°

  –10				dBi		for    48°        180°

where:

min    1° or 100 /D degrees, whichever is the greater, for D/ ≥ 50.

min =  2° or 114 (D/)–1.09 degrees, whichever is the greater, for D/ < 50.

- For secondary NTN UE lobes, RAN4 shall use ITU-R S.465-6 limit as reference for coexistence studies instead of complete antenna pattern with all the side lobes (which would require intensive simulations):

  - Option 3 (Ericsson): Fixed VSAT parameters apply to mobile VSAT as well.

  - Option 4 (Huawei):

| UE phased array antenna Tx characteristics |  |  |
| --- | --- | --- |
| 1 | Ka band UE phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 2) | 5.2 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90° for H 90° for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Row × Column)(Note 4) | 40*40 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 8 dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward Z+ axis |
| Note 1: Void - Note 2: The element gain in row 1.2 includes the loss given in row 1.9. - Note 3: The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element). - Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements. |  |  |

A separate antenna is used for Rx:

| UE phased array antenna Rx characteristics |  |
| --- | --- |
| Rx antenna gain | Same as Tx antenna gain |
| Noise figure | 9dB |
| Rx Feeder loss | 2dB |
| Antenna temperature | 150 K |
| Max G/T figure of merit | 1.8 dB/K (NOTE) |
| Typical UE height | 1m |
| NOTE: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB |  |

Topic #4: Evaluation methodology

Agreement

N/A

Open issues

Issue 4-1:  Propagation model between NTN UE and TN UE

- FFS following options:

  - Option 1 (ZTE):

- Propagation model between NTN UE (on ground) and TN UE should reference to the following propagation model,

- UE-to-UE: UMi (h_BS=1.5 m ~ 22.5 m)

+ penetration loss see TR 38.803

- Propagation model between A-ESIM and TN UE, use the NTN channel model

- Propagation model between M-ESIM and TN UE

  - Option 2 (Huawei): to explicitly draft propagation model between NTN UE and TN UE as below

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified formatand and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

  - Option 3 (CATT):

- Propagation model between NTN UE (on ground) and TN UE should reference to the following propagation model,

- - TN UE-to- TN BS: UMi in TR 38.901 with h_BS=hNTN,UE, where TN BS needs to be replaced by NTN UE

- + Penetration loss see TR 38.803

  - Option 4 (Qualcomm): For the propagation model, to use same assumptions as in 38.821 and consider the atmospheric losses and the scintillation losses.

Reference

[1] R4-2303758, “Topic summary for [106][313] NR_NTN_enh_Part2”, Samsung

- Annex 1 Issue 2-1 Option 1 consideration of Network and UE deployment – NTN UE outside TN cluster

Moderator’s Note: Case 1 to 6 are identical with the deployment for S-Band study, except the removal of the Note in Case 6. Case 7 & 8 are newly proposed.

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | All active TN clusters which has the NTN UE(s) at its edge. |

| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | Consider an active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 3 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values) NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam |  |

| 4 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |
| --- | --- | --- | --- | --- | --- | --- |

| 5 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 6 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

- Annex 2 Issue 2-1 Option 1 consideration of Network and UE deployment – NTN UE inside TN cluster

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped in TN clusters | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Only the active TN clusters which contain the NTN UE(s). |

| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped in TN clusters | Consider an active rate of 20% for TN. | Only the active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 3 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | All in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，interested companies can bring analysis and results for other values) NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam |  |

| 4 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped in TN clusters | TN randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) . |
| --- | --- | --- | --- | --- | --- | --- |

| 5 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which contain NTN UE(s) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |

| 6 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE NTN UEs dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped in TN clusters | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |

| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |