3GPP TSG-RAN WG4 #106			    R4-2302910

Athens, February 27th – March 3rd, 2022

Agenda item:	9.32.2

Source: 	Qualcomm Incorporated

Title: 	WF for Rel-18 mIAB RF requirements impact and co-existence study

Document for:	Approval

# Introduction

This document provides way-forward on mobile IAB (mIAB) based on the outcomes of “Topic summary for [106][315] NR_mobile_IAB_RF”.

# RF core requirements

### mIAB output power and power control

Agreement:

- FFS if output power and power control requirements for Rel-18 mIAB could be the same as for UE.

mIAB FR1 bands

Agreement:

- Consider developing mIAB for bands in addition to n41, n77, n78, n79 based primarily on operator request.

# Coexistence study

Agreement:

- FFS the difference between Rel-18 mIAB and Rel-16 IAB coexistence study in terms of deployment, network layout and coexistence scenarios.

- To list preliminary list of parameters (network layout, deployment, antenna parameters that might be different between Rel-18 mIAB and previous Rel-16 IAB coexistence study.

MT/DU/RAN carrier and frequency configurations

Agreement:

- At least from co-existence study aspect, RAN4 focus on adjacent channel within same band.

mIAB-MT and DU TDM

Agreement:

- For R18 mIAB, RAN4 only consider TDM for MT/DU operation.

Simulation assumptions:

Agreement:

- Network layout:

  - Heterogeneous scenario (layout 1).

  - homogeneous scenario (layout2).

  - other layout options are not precluded.

- FFS on the below listed simulation parameters:

  - Inter-BS distance.

  - Minimum distance between moving IAB Node belonging to two cells.

  - Minimum distance between BS and UE.

  - Proposal for additional system parameters is shown in Table 2.

- For the simulation parameters not listed in Annex below, Rel-16 IAB coexisting simulation can be utilized for preliminary studies.

- Other parameters for coexisting are not precluded and new parameter values could be further discussed.

# Annex

In this Annex, the simulation assumption [2] is listed in Table 1 and Table 2.

- Table 1: layouts summary with change proposals

| Parameters | Layout 1 Heterogeneous scenario (dense urban) | Homogeneous scenario Layout 2 (urban micro) |
| --- | --- | --- |
| Layout | Two layers Macro layer: Hex. Grid (all macro BSs are IAB-donors) 19 sites Micro layer: 2 micro/pico BSs per macro BS 1. Random drop (All micro/pico BSs are all outdoor and are IAB-nodes) Victim network 1. Macro layer: Hex grid 3 sector coordinated layout (0% grid shift) – Rel.15 legacy network 2. Same as aggressor | Two layers: Micro layer: Hex tri-sectorial (micro BS as IAB-donors. 19 sites Second micro layer: 1 micro/pico BSs per micro BS 1. Random drop (All micro/pico BSs are all outdoor and are IAB-nodes) Victim network 1. Micro layer: Hex tri-sectorial – grid shift 0% |
| Inter-BS distance | Macro layer: 200m FR2, 500m FR1 | Inter-BS distance: 200m |
| Minimum distance between moving IAB Node belonging to two cells | 5m | 5m |
| Minimum distance between BS and UE | 35 m along the ground for macro Base Stations 5 m along the ground for micro Base Stations 2 m along the ground for pico Base Stations | 5 m along the ground for micro Base Stations 2 m along the ground for pico Base Stations |

Table 2 : System Parameter

| Parameters |  |
| --- | --- |
| Duplex mode | TDD |
| Frequency range | FR1: 4.9GHz – FR2: 30GHz |
| Beamforming | FR1: Yes – FR2: Yes |
| Simulation bandwidth | 100MHz for FR1 200MHz for FR2 |
| Number of UEs in the network | FR2: 1 active UE/sector FR1: 3 active UEs/sector |
| gNB Tx power | 33 dBm for FR2 macro and micro 46 dBm for FR1 |
| IAB node Tx power | TBD for MT and DU link |
| gNB antenna height | 25m for macro cells and 10m for micro cells |
| IAB node antenna height | 4 m for macro cells and 4m for micro cells |
| gNB receiver noise figure | 10dB for FR2 5dB for FR1 |
| IAB node receiver noise figure | 10dB for FR2 5dB for FR1 |
| UE Tx power (dBm) | FR2: TBD FR1: 23dBm (conducted) |
| UE noise figure (dB) | 10 |

# References

- R4-2303760, “Topic summary for [106][315] NR_mobile_IAB_RF”, Qualcomm Incorporated

- R4-2302403, Coexisting study simulation assumptions for mobile IAB, Ericsson