3GPP TSG-RAN WG4 Meeting #106	R4-2302933

Athens, Greece, 27 February ‒ 3 March 2023

Title: 	WF for SAN demodulation requirements for IoT over NTN

Agenda Item:	10.5.8

Source: 	Ericsson

Document for:	Approval

# Topic #1: Work plan

## Sub-topic 1-1: SAN demod performance requirements

Agreement:

| 3GPP RAN4 meeting | Work content |
| --- | --- |
| 3GPP RAN4 #106 meeting (Feb, 2023) | - Approve work plan. - Agree on the scope of SAN requirements for NB-IoT and Cat-M1. - Agree on simulation assumptions. - Decide the CR work split. |
| 3GPP RAN4 #106-bis-e meeting (April, 2023) | - Update simulation assumptions if necessary. - Collect the simulation results. - Review the draft CR for TS36.108 and TS36.181. |
| 3GPP RAN4 #107 meeting (May, 2023) | - Collect the simulation results. - Agree with the CR for TS36.108 and TS36.181. |

# Topic #2: General

## Sub-topic 2-1: Scenario

Agreement: Deployment mode for NB-IoT

  - Only standalone deployment considered for NB-IoT demodulation requirement. The requirement can be applicable for other mode for BS demodulation requirements

Agreement: Duplex mode for eMTC/NB-IoT

  - Only consider FDD

    - For Cat-M1 UE, define SAN demodulation requirements only for FDD

    - For Cat-NB1/NB2 UEs, define SAN demodulation requirements only for FDD.

Agreement: UL segmented transmission with multiple segments

  - No PUSCH/NPUSCH/PRACH/NPRACH requirement for Uplink segmented transmission with multiple segments. Only define requirements with one segment.

Agreement: Assumption of pre-compensation gap for UL segmented transmission

  - Not to consider pre-compensation gap for simulation.

    - Interested companies can bring simulation results for pre-compensation gap enabled case.

    - See the detailed simulation assumption in Topic #3 and #4.

## Sub-topic 2-2: Channel model

Agreement: Additional Doppler shift offset and transmit timing offset

  - When using segments size (Nsegmentprecompensation) greater than 1, for NPRACH, NPUSCH and NPUCCH, create additional frequency doppler shift offset for every subsequent repetition after the first in a segment.

  - Evaluate if an offset in transmit timing needs to be applied to the subsequent repetitions in a segment as well

  - For additional Doppler shift offset modeling

    - Option 1: gradual offset accumulation is considered for all Tx duration period in one segment.

    - Option 2: Only consider the largest accumulated offset for all Tx duration period in one segment.

  - For additional Doppler offset applied physical channel

    - Option 1: only for NPUSCH format 1 and PUSCH

    - Option 2: for NPUSCH, PUSCH, PUCCH, NPRACH, and PRACH

  - For additional transmit timing offset modeling

    - Option 1: gradual offset accumulation is considered for all Tx duration period in one segment.

    - Option 2: Only consider the largest accumulated offset for all Tx duration period in one segment.

  - For additional transmit timing offset applied physical channel

    - Option 1: only for NPRACH and PRACH

    - Option 2: for NPUSCH, PUSCH, PUCCH, NPRACH, and PRACH

Figure 1	Example of additional Doppler shift offset modeling (For information)

Agreement: Channel model for eMTC SAN demodulation requirements

  - For initial simulation purpose, below channel model considered

    - NTN-TDLA100-X,		X = [5, 200] Hz

    - NTN-TDLC5-Y,		Y = [5, 30] Hz, Rice factor=8.05dB

Agreement: Channel model for NB-IoT SAN demodulation requirements

  - For initial simulation purpose, below channel model considered

    - NTN-TDLA100-X,		X = [1, 200] Hz

    - NTN-TDLC5-Y,		Y = [1, 30] Hz, Rice factor=8.05dB

## Sub-topic 2-3: Demodulation requirement scope

Agreement: Antenna branch for SAN demodulation

  - Define both 1Rx and 2Rx requirements for all SAN demodulation requirements for NB-IoT/eMTC.

Agreement: Scope of eMTC SAN demodulation requirements

  - PUSCH CE mode A

  - PUSCH CE mode B

  - PUCCH DTX to ACK performance requirement

  - PUCCH format 1a

  - PRACH false alarm probability

  - PRACH preamble detection with frequency hopping off

Agreement: Channel bandwidth for eMTC SAN demodulation requirements

  - 1.4MHz only

Agreement: Scope of NB-IoT SAN demodulation requirements

  - NPUSCH format 1

  - NPUSCH format 2 DTX to ACK performance requirement

  - NPUSCH format 2

  - NPRACH false alarm

  - NPRACH miss detection

Agreement: Channel bandwidth for NB-IoT SAN demodulation requirements

  - 200kHz only

# Topic #3: SAN demodulation requirement for eMTC

## Sub-topic 3-1: PUSCH requirements

Way forward: Simulation assumptions for PUSCH CE mode A (for initial simulation purpose)

  - Assume TxDuration is same as the number of repetitions

| Parameters | CE Mode A |
| --- | --- |
| Max number of HARQ transmissions | 4 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions | 8 |
|  | FFS: 1 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-X, X = [5, 200] Hz Option 2: NTN-TDLC5-Y, Y = [5, 30] Hz |
| FRC | TS36.104 A3-2 |
| System BW | 1.4MHz only |
| Doppler shift offset | FFS |
| Transmit timing offset | FFS |
| Fraction of maximum throughput | 70% |

Way forward: Simulation assumption for PUSCH CE mode B (for initial simulation purpose)

  - Assume TxDuration is same as the number of repetitions

| Parameters | CE Mode B |
| --- | --- |
| Max number of HARQ transmissions | 2 |
| RV sequences | 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Number of PUSCH repetitions | 256 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | NTN-TDLA100-X, X = [5, 200] Hz |
| FRC | TS36.104 A3-1 |
| System BW | 1.4MHz only |
| Doppler shift offset | FFS |
| Transmit timing offset | FFS |
| Fraction of maximum throughput | 70% |

## Sub-topic 3-2: PUCCH requirements

Way forward: Simulation assumption for PUCCH format 1a (for initial simulation purpose)

  - The ACK missed detection probability shall not exceed 1%

  - Assume TxDuration is same as the number of repetitions

  - RAN4 will set the same propagation channel for PUCCH format 1a and PRACH fading scenario.

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-X, X = [5, 200] Hz Option 2: NTN-TDLC5-Y, Y = [5, 30] Hz |
| System BW | 1.4MHz |
| Fraction of maximum throughput | 70% |

## Sub-topic 3-3: PRACH requirements

Way forward: Simulation assumptions for PRACH requirements (for initial simulation purpose)

  - The false alarm probability shall be less than or equal to 0.1%.

  - The probability of detection shall be equal to or exceed 99%.

  - Frequency hopping: OFF

  - Assume TxDuration is same as the number of repetitions

  - RAN4 will set the same propagation channel for PUCCH format 1a and PRACH fading scenario.

| Number of Tx antennas | Number of Rx antennas | Propagation channel | Frequency offset | Number of repetitions | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | Option 1: NTN-TDLA100-X, X = [5, 200] Hz Option 2: NTN-TDLC5-Y, Y = [5, 30] Hz | 200Hz + X (X: See Sub-topic 2-2 Additional Doppler shift offset) | 8 | YES | YES |  |  |
|  |  |  |  | 16 |  |  | YES | YES |

# Topic #4: SAN demodulation requirement for NB-IoT

## Sub-topic 4-1: NPUSCH format 1 requirements

Way forward: Simulation assumptions for NPUSCH format 1 requirements (for initial simulation purpose)

  - FFS for Doppler shift offset and Transmit timing offset

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions | FRC | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 3.75KHz | 1 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | TS36.104 A16-1 | 16 | 70% |
| 1 | 1 and 2 | 15KHz | 12 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | TS36.104 A16-5 | 16 | 70% |

## Sub-topic 4-2: NPUSCH format 2 requirements

Way forward: Simulation assumptions for NPUSCH format 2 requirements (for initial simulation purpose)

  - The DTX to ACK probability, i.e., the probability that ACK is detected when nothing is sent per NPUSCH format 2 transmission, shall not exceed 1% per NPUSCH format 2 transmission.

  - The ACK missed detection probability shall not exceed 1%.

  - Assume TxDuration is same as the number of repetitions.

  - RAN4 will set the same propagation channel for NPUCCH format 2 and NPRACH fading scenario.

| Number of TX antennas | Number of RX antennas | Propagation conditions | Number of allocated subcarriers | Subcarrier spacing | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 1 | 3.75KHz | 16 |
| 1 | 1 and 2 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 1 | 15KHz | 16 |

## Sub-topic 4-3: NPRACH requirements

Way forward: Simulation assumptions for NPRACH demodulation requirements (for initial simulation purpose)

  - The false alarm probability shall be less than or equal to 0.1%.

  - The probability of detection shall be equal to or exceed 99%.

  - Assume TxDuration is same as the number of repetitions

  - RAN4 will set the same propagation channel for NPUCCH format 2 and NPRACH fading scenario.

| Number of Tx | Number of Rx | Repetition number | Propagation channel and correlation | Frequency offset | PF0 | PF1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 8 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 200Hz + X (X: See sub-topic 2-2 Additional Doppler shift offset) | YES | YES |
| 1 | 1 and 2 | 32 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 200Hz + X (X: See sub-topic 2-2 Additional Doppler shift offset) | YES | YES |

# Topic #5: CR work split

  - TS 36.108: Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception

| Clause number | Clause name | Company name |
| --- | --- | --- |
| 8.1 | Conducted performance requirements - General | Ericsson |
| 8.2 | Performance requirements for PUSCH |  |
| 8.3 | Performance requirements for PUCCH |  |
| 8.4 | Performance requirements for PRACH |  |
| 8.5 | Performance requirements for NPUSCH format 1 | Samsung |
| 8.6 | Performance requirements for NPUSCH format 2 |  |
| 8.7 | Performance requirements for NPRACH |  |
| 11.1 | Radiated performance requirements - General | FFS |
| 11.2 | Performance requirements for PUSCH |  |
| 11.3 | Performance requirements for PUCCH |  |
| 11.4 | Performance requirements for PRACH |  |
| 11.5 | Performance requirements for NPUSCH format 1 | FFS |
| 11.6 | Performance requirements for NPUSCH format 2 |  |
| 11.7 | Performance requirements for NPRACH |  |
| A.5 | Fixed Reference Channels for PUSCH transmission | Ericsson |
| A.6 | PRACH Test preambles | Ericsson |
| A.7 | Fixed Reference Channels for NPUSCH format 1 transmission | Samsung |

  - TS 36.181: Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node conformance testing

| Clause number | Clause name | Company name |
| --- | --- | --- |
| TBD | Conducted performance requirements - General | Nokia |
| TBD | Performance requirements for PUSCH |  |
| TBD | Performance requirements for PUCCH |  |
| TBD | Performance requirements for PRACH |  |
| TBD | Performance requirements for NPUSCH format 1 | Huawei |
| TBD | Performance requirements for NPUSCH format 2 |  |
| TBD | Performance requirements for NPRACH |  |
| TBD | Radiated performance requirements - General | FFS |
| TBD | Performance requirements for PUSCH |  |
| TBD | Performance requirements for PUCCH |  |
| TBD | Performance requirements for PRACH |  |
| TBD | Performance requirements for NPUSCH format 1 | FFS |
| TBD | Performance requirements for NPUSCH format 2 |  |
| TBD | Performance requirements for NPRACH |  |
| TBD | Fixed Reference Channels for PUSCH transmission | Nokia |
| TBD | PRACH Test preambles | Nokia |
| TBD | Fixed Reference Channels for NPUSCH format 1 transmission | Huawei |
| TBD | Measurement of Performance Requirements | Huawei |