3GPP TSG-RAN WG4 Meeting #xxx	R4-2302942

Athens, Greece, 27 February -03 March, 2023

Title: 	WF for 8Rx UE performance requirements

Agenda Item:	9.6.7

Source: 	Huawei, HiSilicon

Document for:	Approval

1 General

Issue 1-1: Whether to consider CA requirements

- Focus on single CC performance requirements definition now

- Discuss CA scenario after RF session agrees to define any CA band combination(s) for 8Rx

Issue 1-2: Duplex mode

- Focus on TDD and then define requirements for FDD when RF session has conclusion for FDD

Issue 1-3: EVM for PDSCH and SDR simulation

- Tx EVM = 6% for up to 64QAM

- Tx EVM = 3% for 256QAM

- Tx EVM = 2.5% for 1024QAM

Issue 1-4: Whether to define PDCCH requirements

- FFS whether to consider PDCCH requirements

Issue 1-5: PDCCH simulation assumptions (If agreed)

- Option 1

  - FR1 TDD, 40 MHz/30 kHz

| Test num | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNR (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 102 | 1 | 2 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |
| 2 | 3 | 2 | 102 | 1 | 4 | TDLA30-10 | 1x8 ULA Low | 1_1 | 53 | New |
| 3 | 3 | 6 | 90 | 1 | 8 | TDLA30-10 | 2x8 ULA Low | 1_1 | 53 | New |
| 4 | NA | 6 | 48 | 2 | 16 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |

| Parameter | Unit | 1 Tx Antenna |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |
| CCE to REG mapping type |  | Test 4: Non-interleaved Other tests: interleaved | Interleaved |  |
| Interleaver size |  | 3 |  |  |
| REG bundle size |  | 2 | 6 |  |
| Shift Index |  | 0 |  |  |

- Other options are not precluded

  - Interested companies can bring some typical scenarios that PDCCH decoding is the bottleneck of PDSCH performance for 8Rx UE in next meeting.

Issue 1-6: OTA testability

Only define conducted performance requirements for 8Rx

Issue 1-7: Applicability rules

- Rank 2: [Existing corresponding rank 2 test cases can be skipped with test applicable rule if new test case with rank 2 verified]

- Rank 4: Existing rank 4 test cases can be skipped with test applicable rule if new test case with rank 4 verified

- Consider following description as start point

- Case1: with both 2Rx and 4Rx supported RF bands - Case2: without 2Rx supported RF bands but with 4Rx supported RF bands - Case3: with 2Rx supported RF bands but without 4Rx supported RF bands - Case4: without both 2Rx and 4Rx supported RF bands, i.e. with only 8Rx supported RF bands Test applicability rules Table Antenna port# in the supported RF bands; 2Rx test; 4Rx test; 8Rx test. Case1; ×; √; √. Case2; ×; √; √. Case3; √; ×; √. Case4; ×; √; √. For PDSCH requirements: - Case1&Case2: For 8RX capable UEs support only 4Rx bands or both 2RX and 4RX bands, all single carrier test cases specified in 5.2.3.2 with 4Rx are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied. - Case3: For 8RX capable UEs support only 2Rx bands, all single carrier test cases specified in 5.2.2.2 with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied. - Case4: For 8RX capable UEs without support of any 4Rx and 2Rx bands, all single carrier tests specified in 5.2.3.2 with 4Rx except cases listed in Table 5.2.3.1.1-7 (Enhanced Receiver Type 1), 5.2.3.2.16 (MMSE-IRC with inter cell interference), 5.2.3.2.17(MMSE-IRC with intra cell inter user interference), 5.2.3.2.18(CRS-IM with scenario1) and 5.2.3.2.19 (CRS-IM with scenario2) are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. The SNR requirements should be applied with [1.5] dB less than the number specified for 4Rx tests. For PDCCH requirements: - Case1&Case2:For 8RX capable UEs support only 4Rx bands or both 2RX and 4RX bands, all single carrier test cases specified in 5.3.3.1 with 4Rx are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied. - Case3:For 8RX capable UEs support only 2Rx bands, all single carrier test cases specified in 5.3.2.2 except 5.3.2.2.4 (RedCap) with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied. - Case4: For 8RX capable UEs without support of any 4Rx and 2Rx bands, all single carrier test cases specified in 5.3.3.1 with 4Rx are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. Same requirements specified with 4Rx should be applied. For SDR, CSI requirements - For 8RX capable UEs, only apply 8RX tests and no need to apply 2RX and 4RX test.

2 PDSCH requirements

Issue 2-1: MIMO layers

- Rank 2, 4 and 8

Issue 2-2: Antenna configuration

- Rank 2: 2T8R

- Rank 4: 4T8R

- Rank 8: 8T8R

Issue 2-3: Antenna correlation

- Rank 2: ULA Medium B

- Rank 4: ULA Low

- Rank 8: ULA Low

Issue 2-4: Propagation conditions

- Rank 2: TDLC300-100 ,TDLA30-10

- Rank 4 and 8: TDLA30-10

- FFS on consideration of beamforming model on top of TDL channel model for two MCS configuration

Issue 2-5: MCS configuration

- The following MCS can be used starting point;

Rank 2:

- up to 256QAM: [MCS26] (table 2); (Low correlation) (Only forTDLA30-10)

- MCS16 (Medium B);

- MCS19(Medium B)

- MCS23(Low)

Rank 4:

- up to 64QAM: MCS26;

- MCS17;

Rank 8:

- up to 64QAM: MCS20;

- MCS13;

Issue 2-6: Two MCS configuration for Rank>4

- Option 1: Configure two MCSs for Rank>4 and discuss the spatial channel model to be used to guarantee different precoding gain for each layer

- Option 2: Only consider single MCS

Issue 2-7: Overhead

- Overhead = 0

Issue 2-8: Mapping Type

- PDSCH mapping Type A

- FFS PDSCH mapping Type B

Issue 2-9: TRS configuration

- Configure TRS symbols in symbol#{5, 9}

Issue 2-10: NZP-CSI-RS configuration

- NZP CSI-RS configuration of row 6 for 8 layers.

Issue 2-11: Whether consider scenarios with configuration of NZP CSI-RS overlapping with PDSCH for CSI-RS ports larger than 4Tx

- Option 1: Yes, also define ‘Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH’ for 8Rx UE similar as 5.2.2.1.2, 5.2.3.1.2 in 38.101-4.

- Option 2: No

Issue 2-12: Special slot

- Not schedule PDSCH in special slot for 8 layers cases

Issue 2-13: N1 and N2 configuration for 8Tx case

- (N1,N2)=(4,1)

Issue 2-14: Test metric

- Only 70% of maximum TP

Issue 2-14: Simulation assumptions

- Companies are encouraged to bring simulation results based on assumptions listed in Table2-1 and 2-2

Table 2-1: Common simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | 30/40 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |

3 SDR requirements

Issue 3-1: Whether to configure additional DMRS symbol

- Yes

Issue 3-2: Feasibility of 64QAM

- It is feasible to define SDR test for 8 MIMO layer for 64QAM

  - Companies can bring the simulation results in next meeting for the maximum achievable MCS

Issue 3-3: Feasibility of 256QAM

- Evaluate SDR test cases for 2, 4 and 8 MIMO layers for 8Rx; FFS 6 MIMO layers

  - 2 and 4 MIMO layers: Use the existing MCS value defined in Table 5.5A-5 of TS 38.101-4

  - 8 MIMO layers: FFS the max achievable MCS

Issue 3-4: Feasibility of 1024QAM

- FFS whether it is feasible to consider 1024QAM

- If it is feasible to introduce requirements for 1024QAM

  - FFS the maximum achievable MIMO layers: Rank 2 or Rank 4

  - FFS the maximum achievable MCS

Issue 3-5: Simulation assumptions

- Companies are encouraged to bring simulation results based on assumptions listed in Table 3-1.

- The results are formulated as maximum achievable MCS for each combination of modulation and MIMO layers

Table 3-1: Simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 for Rank <= 4 2 for Rank > 4 |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1005} for 6 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2, 4 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |

4 CSI requirements

Issue 4-1: Simulation assumptions for CQI requirmeents

- Antenna configuration: XP 4T8R

- (N1,N2)=(2,1); (O1,O2)=(4,1)

- ReportQuantity: CRI/Ri/i1/CQI

- Two-one-TypeI-SinglePanel-Restriction: 0b00000001

- TypeI-SinglePanel-codebookSubsetRestriction-i2: 0x0000000000000001

Issue 4-2: Summary of simulation assumptions for CQI requirements

- Companies are encouraged to bring simulation results based on assumptions listed in Table 4-1

Table 4-1: Simulation assumptions for CQI test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-i1/CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | The codebook is restricted to i1,1=0, i1,2=0,i1.3=0 and i2=0 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |

Issue 4-3: SNR for CQI requirements

- Option 1: [2, 3] dB and [8, 9]dB

- Option 2: [4, 5] dB and [10, 11]dB

- Option 3: [1, 2] dB and [7, 8] dB

- Other options are not precluded

Issue 4-4: Whether to define PMI requirements

- No

Issue 4-6: Whether to define RI requirements

- No