3GPP TSG-RAN WG4 Meeting #106										    			       R4-2302949

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:			9.19.4

Source:	Samsung

Title:	Ad-hoc minutes for SBFD feasibility study

Document for:	Approval

# Introduction

For Rel-18 SI for study on evolution of NR duplex operation, there are two threads [310][311] organized for the relevant discussion on RAN4#106, and one ad-hoc meeting is planned for further discussion after 1st round online treatment.

# Discussion

## Reply LS to R1-2212963

### Response to Agreement-1 in R1-2212963

[Moderator] The following Agreement-1 is contained in RAN1 LS.

Agreement-1 Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise - σz,n2=PblockerICSBS×NDLRB - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2 - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm, - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1, - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB. - NDLRB is the total number of DL RBs in the DL subbands, - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received. - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g., ICSBS= ICS1 Pblocker<P1 ICS2, P1< Pblcoker<P2ICS3 Pblocker>P2 - Note: Pblocker can be reported by companies

Drafted reply LS from CATT:

[RAN4 confirms RAN1 modelling of inter-site inter-gNB co-channel inter-subband CLI. Regarding the value of the blocker interference power, the impact of interference at the receiver should be considered based on two factors; blocking and leakage/selectivity. An RX blocking model is currently being discussed in RAN4. The blocking model should be applied taking into account the total linear sum of all received power, including wanted signal, self-interference, inter-gNB interference and inter-sector interference.] The impact of interference at the receiver should be considered based on two factors; blocking and leakage/selectivity. An RX blocking model is provided below. The blocking model should be applied taking into account the total linear sum of all received power, including wanted signal, self-interference, inter-gNB interference and inter-sector interference. Usually the blocking effect cannot be determined from the interference contribution of one individual gNB, but from the total input power received in the operating band of the victim gNB (which includes the wanted signals and the interference received from all the aggressor gNBs (i.e. inter-site, co-site inter-sector interference and self-interference)) <Blocking model> Regarding the selectivity for intra gNB interference, currently, RAN4 assumes ACLR performance for Tx sub-band signal and ACS performance for Rx sub-band, which means sub-band selectivity are assumed for gNB Tx/Rx. Besides, RAN4 will further study the possibility of improved receiver impairment performance compared to gNB ACS. So the interference falling into the victim gNB as a result of leakage + selectivity should be calculated according to ACIR value. The impact of both the blocking model (based on all received power) and the leakage/selectivity interference from the interference sources should be combined.

- Discussion

  - QC: Several things to be responded: confirm model, and the view on the three step model.

  - Spark: The draft reply don’t match the above equation.

  - Ericsson: Need the block model based on the total power.

  - CATT: for inter-site, we don’t need to consider the blocking since the impact is minimal. Blocking model is good for RF analysis but is not that important to RAN1 simulation.

- Agreement:

- From RAN4 perspective, the following model is provided for simulation purpose:

  - RAN4 can confirm RAN1 can assume ICS_BS (in channel selectivity) is given by the value of gNB ACS.

  - The noise figure model is provided as below:

    - X-axis: Total received power is the linear sum of all received power, including wanted signal, self-interference, inter-gNB interference and inter-sector interference.

    - Y-axis: noise figure

    - The values of A, B, C and D:

      - A = -43dBm

      - B = -25dBm

      - C = 5dB

      - D = 14dB

    - If the total received power is larger than B:

      - The receiver will be blocked.

  - FFS Receiver ICS model if the total received power larger than [a]dBm.

    - The value of a =

      - Option 1: [-44]dBm for the simulation baseline for legacy macro-BS with certain improvements

      - FFS other options with further improvements.

    - FFS how to model Y value(s).

    - RAN4 will continue discuss this RX blocking model based on the input from companies.

  - FFS RAN1 simulation shall need to include the blocking model

    - FFS based on the blocking model from QC, Nokia and Ericsson.

### Response to Agreement-2 in R1-2212963

[Moderator] The following Agreement-2 is contained in RAN1 LS.

Agreement-2 For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows. Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB - Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB. - Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale). - NDLRB is the total number of DL RBs in the DL subbands. - Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x. - αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI. - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment. - Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived. - Send LS to RAN4 confirming the model and asking the value ranges for spatial isolation, and values of 10*log10ACLRBS and 10*log10ACSBS.

Drafted reply LS from CATT:

[RAN4 confirms RAN1 the co-site inter-sector co-channel inter-subband CLI modelling carried out in RAN1. RAN4 has considered so far baseline values for ACLR and ACS (based on TS 38.104) to be reused for inter-subband co-channel CLI modelling. Additionally, RAN4 has not yet precluded possible improvements on receiver performance compared to baseline gNB ACS. The ACLR/ACS values for FR1 and FR2 are shown in the table below. Range; ACLR [dB]; ACS [dB]. FR-1; 45; 46. FR-2; 28; 24. There’re still some open issues for the RF analysis for the co-site inter-sector co-channel inter-subband CLI modelling in RAN4. Regarding spatial isolation values, the following values have been proposed in RAN4: - FR1: 62-93dB with 75dB being typical values. - FR2: 75-98dB with 88dB being typical values. Some companies have proposed that isolating materials could be added between sectors to increase the isolation. RAN4 has not yet discussed the details of what kind of materials and the building practice or whether such approaches can be applied to outdoor sites. RAN4 suggests RAN1 to take into account these initial values. More exact values will be provided once the analysis has progressed.] When considering co-site interference, it is important to bear in mind that the blocking model should be applied considering the total contribution to interference power (i.e., self-interference, inter-gNB interference and inter-sector interference) and the wanted signalThe blocking issue should be considered and the contribution to interference power is not complete either. There’re still some open issues for the RF analysis for the co-site inter-sector co-channel inter-subband CLI modelling in RAN4. Analysis has been provided of TX panel – RX sub-array interference levels assuming no additional measures such as isolating materials between sectors. The range of values is as follows: FR1: 62-93dB with 75dB being typical FR2: 75-98dB with 88dB being typical The range arises due to the variation of isolation with beam steering directions and sub-arrays. In addition, some companies have proposed that isolating materials could be added between sectors to increase the isolation. RAN4 has not yet discussed the details of what kind of materials and the building practice or whether such approaches can be applied to outdoor sites. If such materials are used, the range of inter-sector isolation valus may be as follows: FR1: 90-100dB RAN4 suggests RAN1 to take into account these initial values. More exact values will be provided once the analysis has progressed.

- Discussion

  - ZTE: isolation simulation right now for macro, but not for local and medium range BS.

- Agreement:

  - RAN4 confirms RAN1 the co-site inter-sector co-channel inter-subband CLI modelling carried out in RAN1, except:

    - The blocking model shall be used in the simulation.

  - Regarding spatial isolation values, the following values have been proposed for macro BS in RAN4:

    - FR1: 62-93dB with 75dB being typical values.

    - FR2: 75-98dB with 88dB being typical values.

    - Some companies have proposed that isolating materials could be added between sectors to increase the isolation. RAN4 has not yet discussed the details of what kind of materials and the building practice or whether such approaches can be applied to outdoor sites. Further improvement over the spatial isolation is FFS.

  - Additionally, RAN4 has not yet precluded possible improvements on receiver performance compared to baseline gNB ACS. The ACLR/ACS values for FR1 and FR2 are shown in the table below.

| Range | ACLR [dB] | ACS [dB] |
| --- | --- | --- |
| FR-1 | 45 | 46 |
| FR-2 | 28 | 24 |

### Response to Agreement-3 in R1-2212963

[Moderator] The following Agreement-3 is contained in RAN1 LS.

Agreement-3 For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 is used as starting point. - Option 1: RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point. - Send LS to RAN4 to ask them whether it can be modelled as an equivalent frequency flat model (e.g., IBEUE,ave) based on RAN4 IBE requirement, and if possible, what is the value of IBEUE,ave

Drafted reply LS from CATT:

Answer to the question 3: RAN4 has agreed that IBE-based model as mentioned in R4-2220243, please RAN1 use that model in the simulation.

- Discussion

  - TBA

- Agreement:

  - RAN4 has agreed that IBE-based model as mentioned in R4-2220243, and RAN4 has not reached the agreement for an equivalent frequency flat model. The IBE-based model is suggested in the RAN1 simulation.

## BS Aspects: Co-channel co-site gNB-gNB CLI model

#### Issue 1-5-4: Analysis Framework

- Proposals/Observations:

  - Proposal 1 (Qualcomm, Nokia, Samsung): For co-channel co-site inter-sector inter-gNB CLI, RAN4 to reuse the self-interference analysis framework with revisited mitigation capabilities if found necessary.

  - Proposal 1a (Samsung): Based on the analysis framework for self-interference, the below table is proposed for analysis for co-channel co-site inter-sector gNB-gNB CLI modeling, with different fields highlighted as below:

- Table 4: Analysis framework for  co-channel co-site inter-sector gNB-gNB CLI

| FR1 (or FR2-1) |  |  |  | Company-A |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | xxx dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | xxx dBc |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | xxx dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | xxx dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | xxx dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | xxx dBc |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  |
|  |  |  | Rx IM3 contribution (dBm) |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | xxx dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | xxx dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | xxx dBc |
| Noise floor ⑩dBm |  |  |  | xxx dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | xxx dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | xxx dBc |
| SBFD configuration |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

- Discussion

  - TBA

- Agreement:

  - For co-channel co-site inter-sector inter-gNB CLI, RAN4 to reuse the self-interference analysis framework with revisited mitigation capabilities if found necessary:

    - FFS how much desense because of co-site inter-sector CLI

      - FFS 1dB desense (in additional to the self-interference) can be used as starting point for further study.

      - FFS the desense value contains the interference from both neighboring sectors or from only one neighboring sector.

## BS Aspects: Co-channel inter-site gNB-gNB CLI model

#### Issue 1-6-1: RX selectivity level

- Proposals/Observations for improved receiver impairment performance compared to gNB ACS:

  - Observation 1 (CMCC): commercial gNB could at least achieve 50dB Rx selectivity.

  - Proposal 1 (CMCC): it’s suggested to at least use 50dB for inter-site Rx selectivity.

  - Proposal 2 (QC): RAN4 to adopt baseline ACS requirements given in TS 38.104, which are 46 dB and 24 dB for FR1 and FR2, respectively for all base station classes (i.e., wide area, medium range, and local area gNBs).

- Discussion

  - TBA

- Agreement:

  - TBA

#### Issue 1-6-2: How to determine the enhancement over baseline (ACLR and ACS)

- Proposals/Observations:

  - Proposal 1 (QC): For co-channel inter-site inter-gNB CLI, RAN4 to reuse existing BS ACLR and ACS to model inter-gNB CLI in the feasibility study as well as the adjacent channel coexistence study.

  - Proposal 2 (QC): RAN4 to investigate the applicability of RAN4 baseline ACLR and ACS for the inter-subband leakage and inter-subband selectivity depending on the findings in the adjacent channel coexistence work.

- Discussion

  - TBA

- Agreement:

  - TBA

## (If time allow) BS Aspect: Residual Self-Interference Cancellation

#### Issue 1-4-1: Assumption for input power metric to LNA

- Proposals:

  - Proposal 1 (Nokia): Use RMS average power for the saturation, non-linearity, and AGC models in the feasibility study and system level simulations. In aspects requiring more precise understanding of the signal peaks, a reasonable RX PAPR (at least 10 dB) should be added to the RMS power.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-4-2: Analysis on LNA non-linearity and blocking level

- Observation and proposals on LNA non-linearity:

  - Observation 1 (Samsung): Simply following existing RAN4 requirement to derive the required LNA IIP3 metrics and accordingly the feasibility conclusion of SBFD is very pessimistic assumption.

  - Observation 2 (Intel): The value of Rx IIP3 seen at the LNA input comes from the numerous trade-offs between gain, noise and IIP3 for all the Rx components that make up the Rx chain.  In order to achieve a feasible Rx IIP3, some re-design of the Rx chain to enable higher IIP3 may be required.

  - Proposal 1 (Qualcomm): FR1 BS IIP3 model: For FR1 BS IM3 model, RAN4 to adopt a Pin-dependent (average total input power) piecewise linear model as shown in the Figure above to characterizes IIP3. The proposed model captures IM3 contributions and AGC impact on IIP3 which can be utilized by RAN4 to progress the SBFD feasibility work.

  - Observation 3 (Qualcomm): FR1 BS IM3: The IM3 contributions are not significant when the total input power signal + jammer is lower than -52 dBm.

  - Observation 4 (CATT): If IM3 contribution is -108dBm for WA BS SBFD Rx noise, IIP3 should be at least-10.5 dBm in whole operation temperature range, which is very challenging.

  - Observation 5 (Ericsson) A reasonable RF filtering solution prior to the LNA is not feasible

- Observation and proposals on blocking level:

  - Proposal 1 (ZTE): to consider in-band blocking level as baseline for gNB receiver saturation, non-linearity and higher capability should depend on vendors’ declaration.

  - Proposal 2 (Qualcomm): FR1 BS maximum blocking level: We are OK with -43 dBm+

- Discussion

  - TBA

- Agreement:

  - TBA

#### Issue 1-4-3: Analysis on AGC and NF model

- Proposals from Qualcomm:

  - Proposal 1 (Qualcomm): FR1 BS NF model: For FR1 BS NF, RAN4 to adopt a noise figure model (shown below) that is dependent on the total average (not peak as in [1]) input power (signal + jammer) with a piecewise linear model. The NF is 5 dB below -52 dBm, and 25.2 dB above -21 dBm. In between there is a linear slope.

  - Observation 1 (Qualcomm): FR1 BS NF: For FR1 BS NF, beyond the total input power value of -52 dBm, the AGC impacts are observed and lead to increase in the BS noise figure.

  - Proposal 2 (Qualcomm): FR2 BS NF:  For FR2-1 BS NF = 5 dB up to -52 dBm Pin, and then a sloped section rising 1 dB/dB with Pin > -52 dBm.

  - Observation 2 (CATT): AGC is not need to be analysed for BS Rx path because ADC dynamic is not a problem.

  - Proposal 3 (Nokia): The AGC model captures impacts of receiver non-linearity, gain control, and ADC dynamic range, and the following parameters are used for wide area base stations:

| Snf | Small signal noise figure | 5 | dB |
| --- | --- | --- | --- |
| a | Input power threshold 1 (RMS) | -44 | dBm |
| b | Input power threshold 2 (RMS) | -38.5 | dBm |
| SL1 | Noise figure slope 1 | 0.364 |  |
| SL2 | Noise figure slope 2 | 0.778 |  |

  - Ericsson: Adopt the piecewise linear model of figure/table 1 for FR1.

| Parameter | Value |
| --- | --- |
| Noise figure (F) | 5 dB |
| Total input average power threshold 1 (P1) | -57.9 dBm |
| Total input average power threshold 2 (P2) | -41.7 dBm |
| Sensitivity degradation slope 1 (k1) | 0.4 dB/dBm |
| Sensitivity degradation slope 2 (k2) | 3.0 dB/dBm |
| Total input average power blocking limit (P3) | -32.5 dBm |

- Discussion

  - TBA

- Agreement:

  - TBA

#### Issue 1-4-4: Analysis on other distortions (Phase noise, ADC quantization noise, Residual sideband, ADC distortions)

- Observations:

  - Observation 1 (Qualcomm): For FR1 BS, other distortions such as ADC quantization noise and distortions were considered in our simulation and measurements, and it was observed that ADC performance is not limiting. Similarly, phase noise and residual sideband are not significant contributors

  - Observation 2 (Samsung): According to calculation presented in Table 5, the requested ADC dynamic range is still within the range of commercialized available component.

  - Observation 3 (CATT): If Rx blocking level is assumed to be the current WA BS requirement, ADC dynamic range can cover the whole signal level including wanted signal and interference signals.

- Discussion

  - TBA

- Agreement:

  - TBA

#### Issue 1-4-5: Interference from co-channel jammer for FR2

- Observation from Qualcomm:

  - Proposal FR2 BS interference model with co-channel jammer: FR2 BS interference can be modelled as a fixed level of interference 34 dB below the total input power.

- Discussion

  - TBA

- Agreement:

  - TBA

# Appendix for Moderator Summary for Reply LS to R1-2212963

### Sub-topic 3-1: Response to Agreement-1 in R1-2212963

[Moderator] The following Agreement-1 is contained in RAN1 LS.

Agreement-1 Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise - σz,n2=PblockerICSBS×NDLRB - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2 - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm, - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1, - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB. - NDLRB is the total number of DL RBs in the DL subbands, - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received. - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g., ICSBS= ICS1 Pblocker<P1 ICS2, P1< Pblcoker<P2ICS3 Pblocker>P2 - Note: Pblocker can be reported by companies

#### Issue 3-1-1: General modeling:

- Proposals on UL reception impairment due to receiver selectivity consists of a combination of linear and non linear effects:

  - Proposal 1 (Nokia):

    - RAN4 to communicate to RAN1 that UL reception impairment due to receiver selectivity consists of a combination of linear and non linear effects:

      - Linear effect is primarily due to crosstalk effect between subcarriers which is dependent on the filtering and on how much OFDM symbol orthogonality there is between the aggressor/interfering and the wanted signal. Note that this interference effect is dependent on the frequency separation between aggressor and victim subcarriers.

      - Non-linear effect is due to receiver densensitization caused by blocking in the receiver.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 3-1-2: Confirmation on gNB ACS value used for ICS

- Proposals on the confirmation of RAN1 understanding for the modelling of inter-site gNB-gNB co-channel inter-subband CLI

  - Option 1 (CATT, vivo): RAN1 ICS understanding is not correct.

  - Option 1a (CATT): Further clarification: RAN4 assumes ACLR performance for Tx sub-band signal and ACS performance for Rx sub-band that sub-band filters are assumed for Tx/Rx, which is different with ICS performance where there’s no in channel filter. So the interference falling into the victim gNB should be calculated according to ACIR value.

  - Option 2 (Samsung, Qualcomm): Confirm gNB ACS as co-channel RX modeling, based on the following agreement:

Agreement: - For co-channel inter-site gNB-gNB CLI modeling, gNB ACS shall be used as baseline for system level simulation and feasibility study. - Further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works.

  - Option 3 (Nokia): RAN1 model in agreement-1 above could be applied as a simplification in the absence of a frequency-dependent model.

    - RAN4 to inform RAN1 that the impairment due to selectivity is dependent on the total power and the bandwidth occupied by the aggressor signal, as well as the position of the DL and UL subbands (e.g. DUD vs DU).

  - Option 4 (Huawei): For FR1 WA, the achievable equivalent selectivity is much higher than 45 dB.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 3-1-3: In-channel selectivity and noise modeling depending on blocker interference

- Proposals on whether ICSBS can be modelled depending on the value of the blocker interference:

  - Option 1 (CATT): No blocking issue for inter-site SBFD gNBs.

  - Option 2 (vivo, Samsung): The model is not dependent on P_blocker.

  - Option 3 (Huawei): For FR1 WA, ICS_BS can be modelled as below with ICS_1  and P_1/P_2 reported by companies

ICSBS= ICS1,            Pblocker≤P1 ICS1-2Pblocker-P1,   P1< Pblcoker≤P2Saturated,             Pblocker>P2

  - Option 4 (Nokia): RAN4 to communicate to RAN1 that the gNB UL reception impairments due to receiver selectivity and receiver blocking are not dependent on the total power received from one specific aggressor gNB (referred to as P_blocker in the RAN1 agreement), but on the total power passing the front-end analogue filter of the gNB receiver, including gNB-gNB co-channel and adjacent channel interference, self-interference, legacy co-channel and adjacent-channel UL interference, UL desired signal transmissions, as well as the signals from other networks in the same operating band.

- Proposals on the level of blocker(s) interference to be considered:

  - Option 1 (Nokia, Ericsson): Non-linear part depends on total power due to self-interference, inter-site and inter-cell interference

- Proposals/observation noise modeling based on input total power:

  - Proposal 1 (Ericsson): Adopt the piecewise linear model of figure/table 1 for FR1.

| Parameter | Value |
| --- | --- |
| Noise figure (F) | 5 dB |
| Total input average power threshold 1 (P1) | -57.9 dBm |
| Total input average power threshold 2 (P2) | -41.7 dBm |
| Sensitivity degradation slope 1 (k1) | 0.4 dB/dBm |
| Sensitivity degradation slope 2 (k2) | 3.0 dB/dBm |
| Total input average power blocking limit (P3) | -32.5 dBm |

  - Proposal 2 (Nokia): the NF increases as a function of total received RMS input power in the gNB receiver P_blocker^((j)), e.g. using a piece-wise linear approximation with the parameters (a, b, SL1, SL2):

| Snf | Small signal noise figure | 5 | dB |
| --- | --- | --- | --- |
| a | Input power threshold 1 (RMS) | -44 | dBm |
| b | Input power threshold 2 (RMS) | -38.5 | dBm |
| SL1 | Noise figure slope 1 | 0.364 |  |
| SL2 | Noise figure slope 2 | 0.778 |  |

  - Observation 1 (Ericsson): If 1dB desense is assumed to model self-interference, then the self-interference power input to the model should be the value assumed to get 1dB desense.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 3-2: Response to Agreement-2 in R1-2212963

[Moderator] The following Agreement-2 is contained in RAN1 LS.

Agreement-2 For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows. Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB - Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB. - Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale). - NDLRB is the total number of DL RBs in the DL subbands. - Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x. - αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI. - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment. - Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived. - Send LS to RAN4 confirming the model and asking the value ranges for spatial isolation, and values of 10*log10ACLRBS and 10*log10ACSBS.

#### Issue 3-2-1: Range of spatial isolation for co-site inter-sector CLI modelling

- General proposal:

  - Proposal 1 (Ericsson): When replying on the value range for spatial isolation, capture also the assumptions on inter-sector distance and antenna structure for each art of the range.

  - Proposal 2 (Huawei): The values of ACLR_BS and ACS_BS for co-channel inter-sub-band should be the same for all the cases, e.g. co-site inter-sector and inter site gNB-gNB.

- Proposals for FR1:

  - Option 1 (Samsung): the value range of spatial isolation in the range can be in the range of [90,100] dB for FR1, by having the methods e.g., installing EM conjugated structure between sectors, larger horizontal distance, or vertical antenna arrangement, or different boresight angle directions, or different electrical tilts or combination thereof.

  - Option 2 (Ericsson): FR1 range of inter-sector TX panel – RX sub-array isolation is 75-90dB, with 75dB being typical, based on a separation of sectors of 400mm horizontal and 300mm vertical.

  - Option 3 (Nokia): For FR1 sectorized deployments, a range between 55-70 dB can be assumed as the starting point.

  - Option 4 (Huawei): For co-site inter-sector case better spatial isolation than RSI case is achievable.

- Proposals for FR2:

  - Option 1 (Ericsson): FR2 range of inter-sector TX panel – RX sub-array isolation is 75-98dB, with 88dB being typical based on an edge to edge separation of 400mm (horizontal and vertical).

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 3-2-2: TX leakage and RX impairment modeling for co-site inter-sector CLI

- Proposals on gNB ACLR and gNB ACS for TX leakage and RX impairment in the CLI modelling:

  - Option 1 (CATT): RAN1 consideration for the co-site inter-sector co-channel inter-subband CLI modelling is not complete. The blocking issue should be considered and the contribution to interference power is not complete either. RAN1 can use the following assumption and update when RAN4 updates the evaluation results.

    - SBFD co-site co-channel interference consideration

      - Using following assumption for calibration purpose:

      - For co-site self-interference scenario, it is assumed the interference level from gNB self-interference is: Noise floor – 6dB.

      - For co-site inter-sector scenario, it is assumed the interference level from co-site inter-sector gNBs is: Noise floor + X dB

        - For medium and local BS: X = -6 dB;

        - For wide-area BS: X = -6 dB.

        - Note 1: this is the sum of all inter-sector gNB-gNB CLI per site.

        - Note 2: Final co-existence study simulation set-up need to be aligned with the conclusion on co-site inter-sector interference modelling and isolation.

        - Note 3: for FR1 wide-area, this means the inter-sector isolation should be not less than [144dB]

  - Option 2 (vivo, Samsung, Qualcomm): Confirm the reuse of gNB ACLR/ACS.

  - Option 2a (Nokia): Confirm the reuse of gNB ACLR/ACS as starting point, e.g. 45 dB and 46 dB respectively for FR1.

    - On the definition of α_(co-site) , RAN4 to inform RAN1 that the impairment due to selectivity is dependent on the total power as well as the size and the arrangement of the DL and UL subbands (e.g. DUD vs DU), while current agreement from RAN1 seems to assume that both DL and UL subband are of equal size.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 3-3: Response to Agreement-3 in R1-2212963

[Moderator] The following Agreement-3 is contained in RAN1 LS.

Agreement-3 For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 is used as starting point. - Option 1: RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point. - Send LS to RAN4 to ask them whether it can be modelled as an equivalent frequency flat model (e.g., IBEUE,ave) based on RAN4 IBE requirement, and if possible, what is the value of IBEUE,ave

#### Issue 3-3-1: Tx leakage model of UE-UE co-channel inter-subband CLI modelling

- Proposals

  - Option 1 (CATT, Samsung, vivo, Apple, Qualcomm, Huawei): RAN4 confirm RAN1’s Option 1 (IBE model) aligned with RAN4 agreement, already contained in RAN4 LS R4-2220243.

  - Option 1a (Huawei): In addition to Option 1, below information can be further replied with:

    - the IQ image falls into the UL sub-band for DUD configuration hence IQ image also can be ignored.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

# Appendix for RSIC summary

#### Issue 1-2-2: RSIC capability for FR1 WA-BS

- Proposals/Observations:

Table FR1 RSIC budget calculation Summary for WA-BS

| FR1 |  |  |  | Qualcomm | Intel | Samsung | Nokia | Ericsson | Huawei |  | Kumu |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Wide Area BS | Wide Area BS | Wide Area BS | Wide Area BS | Wide Area BS example 1 | Wide Area BS example 2 | Wide Area BS (High) | Wide Area BS (Mean) |
| BS TX Power dBPTX = ① dBm |  |  |  | [49] dBm | 49 dBm | 49 dBm | 54 dBm | 53 dBm | 47 | 53 | 53 dBm | 53 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 | 45 | 45 dB | 45 |
|  |  | Frequency isolation techniques used |  | DPD or sub-band filtering |  | DPD utilized | Digital filtering or windowing to clean UL sub-band; DPD to suppress PA distortion | Digital filtering, CFR, DPD | DPD | DPD |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 70 dBc | 80 dBc | 65 dBc | 70 dBc | 80 | 80 | 65 dB | 65 |
|  |  | Spatial isolation techniques used |  | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation |  | TX/RX panel separation and RF barrier structure | Spatial separation between TX/RX panels; EM shielding structures between TX/RX panels | A combination of spatial isolation, chokes, absorption, mushroom EBG. Note that 65dB is an “average” and the exact value depends on TX and RX beam steering direcitons, varying between 55dB and 80dB. | spatial separation between TX/RX panel | spatial separation between TX panel to single RX |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 15dBc | 0 dBc | 5 dBc | 5-10 dBc | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. | 10 （Beam nulling /isolation in TX sub-band） | 10 （Beam nulling /isolation in TX sub-band） | 17.87 dB | 21.3 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  | Limited, ~0dB | 1 dB maximum | 3-5dB, depending on TX beam direction | Less than 0.5 dB loss | Less than 0.5 dB loss |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -91 dBm (=①-②-③-④) | = 49 – 45 -70 = -66dBm | -81 dBm Note: provided by ①-②-③-④ dBm | -62 dBm/20 MHz | -72 dBm | =-94 | -88 | -57 dBm ①-②-③ Tx beam nulling is not helping in Rx subband because of random phases in non-linearirites from DPD | -57 dBm ①-②-③ Tx beam nulling is not helping in Rx subband because of random phases in non-linearirites from DPD |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  |  | 0 dB | 25 dBc | 0 dBc | 0 dBc | N/A | N/A | 15.39 dB | 16.6 |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  |  | 11 dB | 0dBc | 0 dBc | 0 dBc | N/A | 10 | 26.03 dB | 26.2 |
|  |  | RF IC techniques and other tech. (before LNA) |  |  |  | subband filtering | None apply due to feasibility concerns | Analogue interference cancellation incurs RX sensitivity loss due to insertion and also severe limitations on sub-band pre-coding and multi-carrier. Also, high routing complexity with large number of TX and RX. Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Insertion loss would degrade sensitivity. | N/A | Analog filter is put after LNA |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | - | 1dB | TBA | N/A dBc | >=5dBc if e.g. filtering or analogue IC would be applied. | N/A |  | 0.2 dB (assuming 15 dB Rx coupler) | 0.2 |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | x dBc (①-③-④-⑤) | = 49-70-11 = -32dBm | -61 dBm Note: provided by ①-③-⑨-⑤dBm | -21 dBm to -16 dBm depending on TX beam | -27 dBm | -43 | -37 (Equivalent to -47 when 10 dB filtering is counted.) | <-45.26 dBm | -49.9 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 15dBc | 45 dBc | 40 dBc | 0 dBc | xxx dBc | 80 | digital filtering | 0 dB | 0 |
|  |  | Frequency isolation techniques |  | Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done). |  | Filtering | None apply due to feasibility concerns | The RX input level is -27 dBm, and hence the receiver is blocked; no possibility for interference mitigation as part of the digital receive combining algorithms. Filtering after the LNA reduces insertion loss but does not suppress the RX level sufficiently prior to the filter. | digital filtering | digital filtering |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | Not a significant contributor on the gNb Rx capability. When the total input power (Pin) (signal + jammer) is lower than -52 dBm, IM3 contribution is not significant (see Section 3.12 for more details). | -15 dBm | -20dBm | -10 dBm at maximum sensitivity; +10 dBm at maximum linearity (at NF penalty) | -32dBm (Minimum for RAN4 requirement) -22dBm (Realistic for AAS) -10dBm (optimistic for AAS) | -10 | -10 | -20 | -20 |
|  |  |  | Rx IM3 contribution (dBm) |  | -66 dBm | -143dBm | -87 dBm/20 MHz to -74 dBm/20 MHz depending on the TX beam Assuming sufficient isolation at a given TX power, IM3 contribution should be minor in the input range of interest, if using front-end design with suitable gain control elements close to the antenna (NF penalty) | Note: ADC will be overloaded unless AGC is used (which would significantly increase noise figure) or sufficient filtering prior to ADC. Even without ADC overload: -17 dBm (RAN4 minimum receiver) -37 dBm (Realistic) -61 dBm (Optimistic) | -109 | -121 | -95.78 | -109.7 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Increase in Noise figure when total input power (Pin) exceeds -52 dBm. Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model as show Section 3.1.1. |  | N/A | At an RX input signal level of -21 dBm to -16 dBm, the NF would increase to 22 dB to 27 dB. NF penalty is due to gain control at input levels above -45 dBm RMS; phase noise is minor at 3.5 GHz | ADC will be overloaded unless AGC used (which would significantly increase noise figure) or sufficient filtering prior to ADC. | ADC noise: -109 reciprocal phase noise mixing:-112 | ADC noise: -113 reciprocal phase noise mixing:-116 | 5 dB (noise figure) | 5 |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | - (①-③-④-⑤-⑥) | =-66dBm + (-32-45) = -65.6 dBm | -101 dBm Note: provided by ①-③-⑨-⑤-⑥dBm | TBD dBm due to spillover in FFT, if lacking sufficient digital filtering (the self-interference signal in the example is -21 dBm to -16 dBm) | Receiver saturated | =-105 | -111 | -95.78 dBm | -109.7 |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | - | 0 dBc | 5 dBc | 0 dBc | RX processing does not mitigate saturation | 10 | 10 | 12.28 dB | 17.5 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | - | 0 dBc | Limited, ~0dB | 0 dBc; should not assume further UL beamforming loss to maintain any UL gains | Receiver saturated | Less than 0.5 dB loss | Less than 0.5 dB loss | TBD (?) | TBD |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc | 25 dBc | 20 dBc | 0 dBc | Digital IC not possible due to receiver saturation and would anyhow be highly complex due to large number of TX/RX for wide area. | 15 | 15 | 0 dB | 0 |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 155 dBc (②+③+④+⑦) | =-65.6dBm – 25dB = -90.6 dBm, (49- -90.6dBm) = 139.6 dB | 150.0 dBc | 115 dBc to 120 dBc for TX sub-band 110 dBc for RX sub-band | Transmitter: 125 dB Receiver: N/A due to receiver saturation | 150 | 155 | 148.31 dB | 153.7 |
| Noise floor ⑩dBm |  |  |  | -96 dBm/20 MHz @ 5dB noise figure | xxx dBm/CBW | -95dBm/20MHz | -96 dBm/CBW (20 MHz) | -96 dBm/CBW | -96 dBm/20 MHz | -96 dBm/20 MHz | -89 dBm/100MHz | -89 dBm/100MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -102 dBm | xxx dBm | -101 dBm | -102 dBm | -102 dBm | -102 dBm | -102 dBm | -95 dBm | -95 |
| Required RSIC budget (①-⑪dBc) |  |  |  | 151 dBc | xxx dBc | 150 dBc | 156 dBc | 155 dBc | 149 | 155 | 148 dB | 148 |
| SBFD configuration |  |  |  | DUD |  | DUD(40-20-40MHz) | DUD (40/20/40 MHz) | See Annex. 40-20-40 MHz | [40, 20, 40] | [40, 20, 40] |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRBs |  | 5 PRB | 5 RB (1.8 MHz) | See Annex. 5 PRB. | Existing SU | Existing SU+ additionally few RBs |  |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |  | 20MHz |  | >300 MHz |  |  |  |  |
| Others |  |  |  |  |  |  |  |  |  |  |  |  |

#### Issue 1-2-3: RSIC capability for FR1 MR-BS

- Proposals/Observations:

Table FR1 RSIC budget calculation Summary for MR-BS

| FR1 |  |  |  | Samsung | ZTE | Qualcomm | Ericsson |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Medium Range BS | Medium Range BS | Medium Range BS | Medium Range (3GPP minimum requirements) | Medium range (Realistic) | Medium Range (Optimistic RX) | Medium Range (Realistic, lower power) |
| BS TX Power dBPTX = ① dBm |  |  |  | 38 dBm | 31dBm for 100MHz DL and 30dBm for 80MHz DL | 38 dBm | 38 dBm | 38 dBm | 38 dBm | 35 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45dBc+10*log10(80/20)=51dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized | DPD | DPD or sub-band filtering | Digital filtering, CFR, DPD |  |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 50-60 | 80 dBc | 65-70 dBc | 65-70 dBc | 65-70 dBc | 65-70 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | Spatial separation between TX/RX | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation | A combination of spatial isolation, chokes, absorption, mushroom EBG. Note that 65dB is an “average” and the exact value depends on TX and RX beam steering directions, varying between 55dB and 80dB. |  |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5 dBc | N/A | 10 dBc | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. |  |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB | N/A | - | 3-5dB, depending on TX beam direction |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -92 dBm Note: provided by ①-②-③-④ dBm | 30dBm-51(ACLR)-50(antenna isolation)-30(digital cancellation/sub-band filtering)-10*log10(20*10) =-124.01dBm/100kHz 30dBm-51(ACLR)-60(antenna isolation)-20(digital cancellation/sub-band filtering)-10*log10(20*10) =-124.01dBm/100kHz Note 1: the digital IC and sub-band filter in the transmitter could achieve the same purpose for suppress the transmitter leakage into its receiver Note 2: with increasing antenna isolation, then sub-band filter attenuation or digital IC could be reduced as well. | -97 dBm (=①-②-③-④) | -87 dBm | -87 dBm | - 87 dBm | -92 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 25 dBc | N/A |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0dBc | 30dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | subband filtering | analog filter for Rx sub-band |  | Analogue interference cancellation incurs RX sensitivity loss due to insertion and also severe limitations on sub-band pre-coding and multi-carrier. Also, high routing complexity with large number of TX and RX. Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Insertion loss would degrade sensitivity. |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | TBA |  |  | >=5dBc if e.g. filtering or analogue IC would be applied. |  |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -72 dBm Note: provided by ①-③-⑨-⑤dBm | 30dBm-50(antenna isolation)-30 (sub-band filter) = -50dBm equal to 50dBm for ACS requirement and -38dBm for IBB requirements of 20MHz reception |  | -42 dBm | -42 dBm | -42 dBm | -47 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 25 dBc | 46dBc for ACS requirement | 15dBc | xxx dBc |  |  |  |
|  |  | Frequency isolation techniques |  | Filtering | 30dBc for analog filter of Rx receiver | Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done). | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. For RX, the 3rd column represents improved receiver linearity in the analogue domain. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. This assumption should be reviewed by RAN1. |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm | TBD | Not a significant contributor on the gNb Rx capability. When the total input power (Pin) (signal + jammer) is lower than -52 dBm, IM3 contribution is not significant (see Section 3.12 for more details). | -27.6 dBm | -17.6 dBm | -13 dBm | -17.6 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -176dBm | =3*Pinf-2*IIP3 |  | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |  | Increase in Noise figure when total input power (Pin) exceeds -52 dBm. Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model as show Section 3.1.1. | No significant issues for medium range BS power level other than mentioned above. Phase noise reciprocal mixing is not significant for this frequency range and power levels. |  |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -97 dBm Note: provided by ①-③-⑨-⑤-⑥dBm | 30dBm-50(antenna isolation)-30(sub-band filter)-46dB (ACS)-10*log10(20*10) = -118.01dBm/100kHz 31dBm-60(antenna isolation)-30(sub-band filter)-46dB (ACS)-10*log10(20*10) = -128.01dBm/100kHz | - (①-③-④-⑤-⑥) | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 dBc | N/A | - | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | N/A | - | TBC dBc |  |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 20 dBc | 20 dBc | 10 dBc | 10-15 dBc (Transmitter) |  |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 148.8 dBc | [120.9dBss] | 145 dBc (②+③+④+⑦) | 109 dBc | 128 dBc | 135 dBc | 135 dBc |
| Noise floor ⑩dBm |  |  |  | -90dBm/20MHz |  | -91 dBm/20 MHz @ 10 dB NF | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -96 dBm | -174dBm/Hz+10*log10(100*10^3)+10dB-6dB= -120dBm/100kHz | -97 dBm | -96 dBm | -96 dBm | -96 dBm | -96 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 134 dBc | 30dBm-10*log10(80*10)-(-120dBm/100kHz) = 120.9dB for single antenna 36dBm-10*log10(80*10)-(-120dBm/100kHz) = 126.9dB for four antenna | 135 dBc | 134 dBc | 134 dBc | 134 dBc | 131 dBc |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) | 80MHz DL, 20MHz | DUD | 40-20-40, see annex |  |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB | Left up to the implementation. This highly depend on how to achieve the ACS performance and Q factor of analog filter if needed | 5 PRBs | 5 PRB, see annex |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | 20MHz | 80MHz DL and 20MHz UL | 100MHz | <300MHz |  |  |  |
| Others |  |  |  |  |  |  | Preliminary estimates due to lack of final conclusion on RX ACS and scattering effects. |  |  |  |

#### Issue 1-2-4: RSIC capability for FR1 LA-BS

- Proposals/Observations:

Table FR1 RSIC budget calculation Summary for LA-BS

| FR1 |  |  |  | Samsung | ZTE | Ericsson (preliminary) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Local Area BS | Local Area BS | Local Area BS (3GPP minimum) | Local Area BS (Realistic RX) |
| BS TX Power dBPTX = ① dBm |  |  |  | 24 dBm | 24 for 100MHz DL and 23.0dBm for SBFD DL | 24 dBm | 24 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45dBc+10*log10(80/20)=51dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized | DPD | Digital filtering, CFR, DPD | Digital filtering, CFR, DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 50 | 70 dBc | 70 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | Spatial separation between TX/RX | Physical distance, isolation structures | Physical distance, isolation structures |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5 dBc | N/A | 0 dBc | 0 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB |  | TX beam nulling not assumed due to array size | TX beam nulling not assumed due to array size |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -106 dBm Note: provided by ①-②-③-④ dBm | 23dBm-51(ACLR)-50(antenna isolation)-20(digital cancellation)-10*log10(20*10) =-121.01/100kHz | -91 dBm | -91 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 25 dBc | N/A | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0dBc | N/A | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | subband filtering | N/A | Analogue IC could be considered for this case, but is restrictive on pre-coding and multi-carrier. Digital IC has instead been assumed. |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | TBA |  | 0 dBc | 0 dBc |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -86 dBm Note: provided by ①-③-⑨-⑤dBm | 23dBm-50(antenna isolation)=-27dBm Which is a bit higher than in-band blocking requirement of LA -35dBm | -46 dBm | -46 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 20 dBc | 46dBc | xxx dBc | xxx dBc |
|  |  | Frequency isolation techniques |  | N/A | N/A | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. This assumption should be reviewed by RAN1. |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm | -2dBm | -24.6 dBm | -14 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -218dBm | 3*(-35)-2(-2)-10*log10(20*10)=-124.01dBm/100kHz | -88.8 dBm | -110 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |  | No other significant impacts other than those mentioned above |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -106 dBm Note: provided by ①-③-⑨-⑤-⑥dBm | 23dBm-50(antenna isolation)-46dB (ACS)-10*log10(20*10)-20= -116.01dBm/100kHz | -88.8 dBm | -110 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 dBc | N/A | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | N/A | TBC dBc |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 20 dBc | 20dBc | 15 dBc on transmitter | 15 dBc on transmitter |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 147.0 dBc | 110.9dB | 112 dBc | 124 dBc |
| Noise floor ⑩dBm |  |  |  | -87dBm/20MHz |  | -87 dBm/CBW | -87 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93dBm | -174dBm/Hz+10*log10(100*10^3)+13dB-6dB= -117dBm/100kHz | -93 dBm | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 117 dBc | 23dBm-10*log10(80*10)-(-117dBm/100kHz) = 110.9dB for single antenna 29dBm-10*log10(80*10)-(-117dBm/100kHz) = 116.9dB for four antenna | 117 dBc | 117 dBc |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) | 80MHz DL, 20MHz | 40-20-40, see annex |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB | Left up to the implementation. This highly depend on how to achieve the ACS performance and Q factor of analog filter if needed | 5 PRB, see annex |  |
| bandwidth over which suppression is achieved |  |  |  | 20MHz | 80MHz DL and 20MHz UL | <300MHz |  |
| Others |  |  |  |  |  | Preliminary estimates due to lack of final conclusion on RX ACS and scattering effects. |  |

#### Issue 1-2-5: RSIC capability for FR2 BS

- Proposals/Observations:

Table: FR2-1 RSIC budget calculation Summary

| FR2-1 |  |  |  | Qualcomm | Samsung | China Telecom |  | Ericsson |  |  | Huawei |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | FR2-1 BS | Case I | Case II | 40 dBm TRP | 30 dBm TRP | 24 dBm TRP | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm | 30 dBm | 30 dBm@FR2 | 30 dBm@FR2 | 40 dBm | 30 dBm | 24 dBm | 35 dBm/200MHz |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc |
|  |  | Frequency isolation techniques used |  | DPD or sub-band filtering | Without DPD | Filtering, CFR | N\A | Filtering, CFR |  |  | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 85-95 dBc | 87 dBc | 80 dBc | 80 dBc | 80 dBc | 80 dBc | 80 dBc | 85~95 dBc |
|  |  | Spatial isolation techniques used |  | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation | TX/RX panel separation and RF barrier structure | A combination of spatial isolation, chokes, absorption, mushroom EBG. | spatial separation between TX/RX panel; | A combination of spatial isolation, chokes, absorption, mushroom EBG. |  |  | spatial separation between TX/RX panel |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5-10 dBc | 10 dBc | 0dBc | 5 dBc | 5 dBc | 5 dBc | 5 dBc | 10 dBc （Beam nulling /isolation in TX sub-band） |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | - | Limited, ~0dB | <1.5dB | TBC | TBC | TBC | TBC | Less than 0.5 dB loss |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -88 dBm (=①-②-③-④) | -95 dBm Note: provided by ①-②-③-④ dBm | −100dBm | -73 dBm | -73 dBm | -83 dBm | -89 dBm | -94~-104 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | - | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | - | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | - | Not applicable |  |  |  |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | - | No impact | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | - (①-③-④-⑤) | -67 dBm Note: provided by ①-③-⑨-⑤dBm | −70dBm | -45 dBm | -45 dBm | -55 dBm | -61 dBc | -60 ~ -70 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 15 dBc | 24 dBc |  |  |  |  |  |  |
|  |  | Frequency isolation techniques |  | Filtering | Filtering |  |  |  |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | Similar conclusion as FR1 (i.e., IIP3 and IM3 are not dominant) following the same analysis that was conducted for FR1 (Section 3.1.2). | -35 dBm | -35 dBm | -35 dBm | -35 dBm | -35 dBm | -35 dBm |  |
|  |  |  | Rx IM3 contribution (dBm) |  | -65 dBm | - 95 dBm | -113 dBm | -65 dBm | - 95 dBm | -113 dBm | negligible |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Noise figure degradation for FR2 is given in Section 3.2. | Reciprocal phase noise mixing will add noise at around -95dBm. | Reciprocal phase noise mixing will add noise at around -105dBm | Reciprocal phase noise mixing will add noise at around -110dBm | Reciprocal phase noise mixing will add noise at around -95dBm. | Reciprocal phase noise mixing will add noise at around -105dBm | Reciprocal phase noise mixing will add noise at around -110dBm | negligible |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | - (①-③-④-⑤-⑥) | -91 dBm Note: provided by ①-③-⑨-⑤-⑥dBm | −90dBm | -65 dBm | -65 dBm | -94.6 dBm | -108 dBm | negligible |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | - | 10 dBc | 0dBc | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  | 10 dB （Beam nulling /isolation in RX sub-band） |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | - | Limited, ~0dB | <1dB | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  | Less than 0.5 dB loss |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10 dBc | 15 dBc | 0 dBc | 10 dBc | 10 dBc | 10 dBc | 10 dBc |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 128 dBc (②+③+④+⑦) | 134.5 dBc | 120 dBc | 130 dBc | 105 dBc | 120,7dBc | 122.5 dBc | 129~139 dBc |
| Noise floor ⑩dBm |  |  |  | -88 dBm/40 MHz @ 10dB noise figure | -83 dBm/100MHz | −83dBm/100MHz | −83dBm/100MHz | -87 dBm/CBW | -87 dBm/CBW | -87 dBm/CBW | -88 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -94 dBm | -89 dBm | −89 dBm | −89 dBm | -93 dBm | -93 dBm | -93 dBm | -94 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 124 dBc | 119 dBc | 119 dBc | 119 dBc | 133 dBc | 123 dBc | 117 dBc | 129 dBc |
| SBFD configuration |  |  |  | DUD | DU (100MHz-100MHz) |  |  | 75-50-75 (See Annex) |  |  | 75-50-75 (See Annex) |
| Guardband assumption (if exist) |  |  |  | 5 PRBs | 5PRB |  |  | 3 RB (See Annex) |  |  | 3 RB (See Annex) |
| bandwidth over which suppression is achieved |  |  |  | 200MHz | 100MHz | 100MHz | 100MHz | Several GHz |  |  | 200MHz |
| Others |  |  |  |  |  |  |  |  |  |  |  |