3GPP TSG-RAN WG4 Meeting # 106                                                                R4-2302969			R4-22xxxxx

# Athens, Greece, February 27 – March 3, 2023

Title: 	WF on BS RF requirement impact for SBFD operation

Source: 	CATT

Agenda item:	9.19.4

Document for:	Approval

- Background

The SBFD BS RF requirements impact has been discussed for several meetings.

There were some agreements in RAN4#104b and RAN4#105. The status before this meeting is summarized as below.

Agreements:

- OTA sensitivity:

  - New requirements are needed

- The existing RF requirements with respect to wanted signal as below are still applicable for gNB capable of SBFD:

| Conducted RF requirement | Radiated RF requirement |
| --- | --- |
| BS output power Output power dynamics Transmit ON/OFF power Transmitted signal quality Occupied bandwidth Dynamic range | Radiated transmit power OTA base station output power OTA output power dynamics OTA transmitted signal quality OTA occupied bandwidth OTA dynamic range |

- The receiver out-of-band blocking and receiver spurious emission requirement in TS38.104 are still applicable for gNB capable of SBFD.

  - FFS the wanted signal could be REFSENS+7dB instead of REFSENS+6dB.

- ACLR and ACS are pending on RAN4 adjacent channel co-existence study

- Other requirements not precluded

No agreements for the following:

- In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity:

  - Option 1: No such requirement needed. RAN4 consider the SBFD performance requirement for receiver sensitivity with the simultaneous TX in the SBFD time slot, in which the in-channel adjacent subblock leakage ratio and in-channel adjacent subblock blocking requirements can be guaranteed implicitly.

  - Option 2: New requirements are needed for In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking + Adjacent subband selectivity.

- ACLR, ACS, in-band blocking, intermodulation:

This WF summarizes the progress of this meeting and the status until this meeting.

- Agreements in this meeting

The following requirements may be impacted,

- Transmitter intermodulation

- Out of band blocking

The following requirements are not applicable,

- Transmit ON/OFF power requirement within SBFD time slot

- OTA receiver spurious emissions

- The summary for BS RF requirement impact for SBFD capable gNB

The existing RF requirements with respect to wanted signal as below are still applicable within SBFD time slot:

| Conducted RF requirement | Radiated RF requirement |
| --- | --- |
| BS output power Output power dynamics Transmitted signal quality Occupied bandwidth Dynamic range | Radiated transmit power OTA base station output power OTA output power dynamics OTA transmitted signal quality OTA occupied bandwidth OTA dynamic range |

The following requirements are new requirements,

- OTA sensitivity within SBFD time slot

- Other requirements not precluded

The following requirements may be impacted,

- Transmitter intermodulation

- Out of band blocking

The following requirements are not applicable,

- Transmit ON/OFF power requirement within SBFD time slot

- OTA receiver spurious emissions

- Reference

[1] R4-2217464, WF on SBFD feasibility study and RF impact: BS aspect

[2] R4-2220244, WF for the feasibility from BS aspect

[3] R4-2303755, Topic summary for [106][310] FS_NR_duplex_evo_Part1