3GPP TSG-RAN WG4 Meeting #106	R4-2302971

Athens, Greece, February 27th ‒ March 3rd, 2023

Title: 	WF on BS demodulation requirements for FR2-2

Agenda Item:	6.2.7

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Approval

# Topic #1: General issues for BS demod requirements

### Sub-topic 1-1 General issues

Use of Rel-15 rule for performance requirements derivation

<Agreement>

- The Rel-15 rule for performance requirement derivation can be used as starting point for the valid test cases.

LOS component modelling for TDL-D

<Agreement>

- Adopt UE demodulation model for TDL-D LOS component.

MCS and number Tx/Rx branches for 16 QAM PUSCH requirements

<Agreement>

- For 1x2 Low, use TDLA30-650 for 100MHz CBW, TDLA10-650 for 400MHz CBW.

- For 2x2 Low, use TDLA30-200 for 100MHz CBW, TDLA10-200 for 400MHz CBW

  - Consider MCS16 for the requirements

  - Update requirements on CRs with

    - TDLA30-650 - 120 kHz and 100 MHz– add requirements in []

    - TDLA10-650 - 120 kHz and 400 MHz – add requirements in []

    - TDLA10-650 - 480 kHz and 400 MHz – add requirements in []

  - Bring simulation results next meeting for

    - MCS16

      - TDLA30-200 - 120 kHz and 100 MHz

      - TDLA10-200 - 120 kHz and 400 MHz

      - TDLA10-200 - 480 kHz and 400 MHz

    - MCS12

      - TDLA30-650 - 120 kHz and 100 MHz

      - TDLA10-650 - 120 kHz and 400 MHz

      - TDLA10-650 - 480 kHz and 400 MHz

  - Decide between MCS12 and MCS16 for 2x2 requirements based on the results provided

MCS and number Tx/Rx branches for 64 QAM PUSCH requirements

<Agreement>

- For 1x2 Low, introduce MCS 20 for PUSCH requiremets

- No requirements for MCS18 with 2T2R.