3GPP TSG-RAN WG4 Meeting #106 	R4-2302972

# Athens, Greece, 27th-February - 3rd March 2023

Agenda item:			6.2.6.1

Source:	Qualcomm

Title:	Topic summary for [106][322] NR_exto71GHz_Demod_Part2

Document for:	Information

# Way Forward

PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=20RBs;

- Agreement: Option 1: CORESET BW=18PRB, Duration = 2Sym, AL=4 (Baseline, Nokia);

Test configuration for Single Carrier and FR1 + FR2-2 CA test with reduced RB allocation (120kHz, MCS 17, 32RBs).:

- Agreement: Do not introduce Test with reduced RB allocation for single Carrier or CA;

Moderator’s note: The introduced requirements for 120kHz/100MHz/MCS 17 are above the assumed maximum testable SNR (see previous WF R4-2220187).

Whether to add the applicability rules that FR2-2 UE demodulation and CSI requirements only applies for the region and country that LBT is not mandatory

- Agreement: No applicable rules needed for both BS and UE performance requirements.

PDCCH Requirements Table for FR1+FR2-2 CA

- Agreement:

  - 100MHz/120KHz:

    - PDCCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration.

  - 400MHz/480KHz

    - PDCCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration.

CQI Configuration for High SNR test point

- Agreement:

  - Use [7,8] dB SNR for CQI reporting requirements;

CQI Requirements Table for FR1+FR2-2 CA

- Agreement:

  - 100MHz/120KHz: CQI requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is 66 RBs;