3GPP TSG-RAN WG4 Meeting #106	     											         R4-2302980

Athens, Greece, February 27th – March 3rd, 2023                       (revised from R4-2302978)

Agenda item:			9.25.6

Source:	Moderator (THALES)

Title:	WF for system parameters of above 10 GHz NTN band

Document for:	Agreement

# Introduction

This Way Forward considers remaining issues from R4-2303757, Topic summary for [106][312] NR_NTN_enh_Part1, with the following list of topics/sub-topics/issues (presented in R4-2303757):

- Topic #1: System parameters

* Include band definition

  - Sub-topic 1-1: Operating band definition and related parameters.

    - Issue 1-1-1: Above 10 GHz NTN band definition

    - Issue 1-1-2: Channel BW

    - Issue 1-1-3: SCS

    - Issue 1-1-4: Spectral Utilization

    - Issue 1-1-5: FR2-0(-NTN)/FR2-1-NTN sub-frequency range specification

    - Issue 1-1-6: Tx-Rx frequency separation

  - Sub-topic 1-2: Channel raster and synchronization raster

    - Issue 1-2-1: NR-ARFCN

    - Issue 1-2-2: GSCN

    - Issue 1-2-3: LS to RAN1

    - Issue 1-2-4: LS to RAN1 & RAN2

  - Sub-topic 1-3: Deployment, Regulatory and Protection Requirements

    - Issue 1-3-1: Protection requirements

    - Issue 1-3-2: Additional protection requirements

    - Issue 1-3-3: Deployment and Power Class

    - Issue 1-3-4: Deployment and Power Control

    - Issue 1-3-5: Deployment and beam pointing report

- Topic #2: SAN RF requirements

  - Sub-topic 2-1: General discussions

    - Issue 2-1-1: OBUE requirement for SAN above 10 GHz

    - Issue 2-1-2: SAN power flux density requirements

  - Sub-topic 2-2: SAN Conducted Requirements

    - Issue 2-2-1: Impact of insertion loss and link budget

    - Issue 2-2-2: SAN Tx Requirements

    - Issue 2-2-3: SAN Rx Requirements

  - Sub-topic 2-3: SAN OTA Requirements

    - Issue 2-3-1: SAN OTA Tx Requirements

    - Issue 2-3-2: SAN OTA Rx Requirements

- Topic #3: NR NTN enhancement workplan

  - Sub-topic 3-1 Coverage enhancement section from Revised Work plan (R4-2300973)

  - Sub-topic 3-2 NR-NTN deployment in above 10 GHz bands section from Revised Work plan (R4-2300973)

  - Sub-topic 3-3 Network verified UE location section from Revised Work plan (R4-2300973)

  - Sub-topic 3-4 NTN-TN and NTN-NTN mobility and service continuity enhancements section from Revised Work plan (R4-2300973)

  - Sub-topic 3-5 Entire Revised Rel-18 NR-NTN-enh work item Work plan (R4-2300973)

In this Way Forward we discuss only remaining issues from Topic#1 from Topic summary for [106][312] NR_NTN_enh_Part1. For SAN RF requirements please check R4-2302877 (WF for SAN RF requirements on above 10 GHz).

# Agreements

Issue 1-1-1: Above 10 GHz NTN band definition (updated during ad-hoc meeting 18h00-19h00 on 4th Day)

Agreement:

The following bands and frequency ranges were agreed, without square bracket.  Note that the DL lower frequency range was modified from 17.7 to 17.3 to reflect regional regulations.

| NTN operating band | UL Earth-to-Space | DL Space-to-Earth |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz |

The notes were then treated individually:

Note 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01.

Note 2: This band is applicable in the USA subject to FCC 47 CFR part 25.

Note 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202).

Issue 1-1-2: Channel BW

Agreement:

  - Option 1: Not to define channel bandwidth beyond 400MHz for Ka-band.

This was agreeable since there was already a prior agreement to this from RAN4 #105.

Issue 1-1-3: SCS

Agreement:

  - Option 1: Wait for RRM conclusion on the feasibility of using sub-carrier spacing values 60 and 120 kHz for NTN above 10 GHz.

This was agreeable. There was no objection to this option.

Issue 1-1-4: Spectral Utilization

Agreement:

  - Option 1: to use the same spectral utilization requirement of FR2 TN system for NTN Ka-band as starting point.

This was agreeable due to previous agreement from RAN4 #105.  “Take the same spectral utilization requirement of FR2 TN system as starting point pending on further confirmation after co-existence study.”

Issue 1-1-5: FR2-0(-NTN)/FR2-1-NTN sub-frequency range specification

- Proposals

  - Option 1: Introduce extension of the lower bound of the FR2 frequency range to cover both DL and UL of NTN bands defined under the Ka band. (P1& P2/R4-2302468)

NOTE: This FR2 extension does not preclude its further extensions.

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-0 | [17700] MHz – 24250 MHz |
|  | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

  - Option 2: FR2-1-NTN is agreed for the FR name only in NTN specifications. TN specifications are not touched at current stage. (P1/R4-2300563)

Table 1: Definition of frequency ranges for NTN

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2-1-NTN | [17700]-20200 MHz, [27500]-30000 MHz |

  - Option 3: Consider one single range [17700]-30000 MHz (as for FR1 410 MHz – 7125 MHz).

- Agreement:

  - Update with 17.3 GHz lower range instead of 17.7 GHz.

  - Further discuss if introducing FR2-0 or FR2-1-NTN with respect to definition of NTN (which is FDD).

- The FR would be captured in NTN specifications, in order to not impact terrestrial definition FR1 and FR2 for TN specifications.

- At least 2 options are considered for the corresponding frequency ranges:

  - 17300-24250 MHz as extension of the existing FR2 range:

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-0 | 17300 MHz – 24250 MHz |
|  | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

  - 17300-30000 MHz;

  - Other options not precluded.

Issue 1-1-6: Tx-Rx frequency separation

- Proposals

  - Option 1: Flexible Tx-Rx frequency separation can be used for Ka band NTN FDD operation.  (P2/R4-2300563, O1/R4-2301744)

- Recommended WF

  - FFS

Issue 1-2-1: NR-ARFCN

- Proposals

  - Option 1: Use following NR-ARFCN in the Table 2.3-1/2.3-2 for NTN Ka-band. (P3/R4-2301744)

Table 2.3-1: NR-ARFCN parameters for the global frequency raster [TS 38.101-5]

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 17700 – 20200 | 15 | 17700 | 1580000 | 1580000 – 1746664 |

Table 2.3-2. NR-ARFCN for NTN Ka-band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| [n512] | 60 | 1580000 – <4> – 1746664 |
|  | 120 | 1580000 – <8> – 1746664 |

- Recommended WF

  - Update Option 1 with 17.3 GHz lower range.

  - To update Option 1 with respect to FDD UL and DL ranges, since no single NR-ARFCN.

  - Add information for n511 and 510 bands.

Issue 1-2-2: GSCN

- Proposals

  - Option 1: Use following GSCN in the Table 2.3-3/2.3-4 for NTN Ka-band MSS-band. (P4/R4-2301744)

Table 2.3-3: GSCN parameters for the global frequency raster

| 17700 – 20200 MHz | 3000MHz MHz + N *1.44 MHz N = 10209:11944 | 7499 + N | 17708 – 19443 |
| --- | --- | --- | --- |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

Table 2.3-4. GSCN for NTN Ka-band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n512] | 120 kHz | Case D | 17724- <12> - 19428 |
|  | 240 kHz | Case E | 17736 - <24> - 19416 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. NOTE 2: Step size 12 equal to 17.28MHz by 1.44MHz*12 and step size 24 equals to 34.56MHz by 1.44MHz*24. |  |  |  |

- Recommended WF

  - Update Option 1 with 17.3 GHz lower range.

  - Add information for n511 and 510 bands.

Issue 1-3-1: Protection requirements

- Proposals

  - Option 1: Use NTN UE location in 3D space and guard bands. The use or avoidance of specific spectrum sub-channels that overlap with FS services can be controlled on a per-UE basis by the Network resource allocation mechanisms taking into account whether the NTN UE is subject to coordinated or uncoordinated operation, NTN UE location in 3D space and any guard band that may be required. (P5/R4-2302643)

  - Option 2: it’s necessary to design some technical mechanisms to guarantee the regulatory compliance for satellite communication, e.g. fixed vs moveable, GSO vs NGSO and other aspects. (P1/R4-2302107)

  - Option 3: Focus on general requirements first before tackling the band specific requirements. (P2/R4-2302711)

- Recommended WF

  - FFS

Issue 1-3-2: Additional protection requirements

- Proposals

  - Option 1: RAN4 should specify an optional method for the NTN UE or the Network to disable NTN UE UL transmission if and when required. (P4/R4-2302643)

- Recommended WF

  - FFS

Issue 1-3-3: Deployment and Power Class

- Proposals

  - Option 1: NTN UE power classes can be used to differentiate different types of NTN UE depending on the deployment scenario (e.g. land, vessel, aircraft, etc). (P6/R4-2302643)

- Recommended WF

  - FFS

Issue 1-3-4: Deployment and Power Control

- Proposals

  - Option 1: For each NTN UE power class, the Network and/or NTN UE should be able to further apply UL power control to the NTN UE transmission during operation to comply with any situational requirements. (P7/R4-2302643)

NOTE: factors such as, but not limited to, whether the NTN UE is subject to coordinated or uncoordinated operation, NTN UE location in 3D space and beam pointing can be taken into account. FFS whether specification work is needed or if it can be left to implementation.

- Recommended WF:

  - FFS, further details would be needed on behaviour and expected features.

Issue 1-3-5: Deployment and beam pointing report

- Proposals

  - Option 1: RAN4 should specify an optional method for NTN UE to monitor and report beam pointing towards the satellite. (P3/R4-2302643)

- Recommended WF

  - FFS, the contribution refers to “satellite beam tracking” or “satellite tracking” of the UE.