3GPP TSG-RAN WG4 Meeting # 106 										   R4-2303145

# Athens, February 27th – March 3rd, 2023

Title:	WF on Rel-17 FeMIMO RRM core requirement maintenance

Agenda item:			5.4

Source:	Intel, Samsung

Document for:	Approval

# Way Forward

Issue 1-1-3 Whether UE need to track UL time/frequency if source RS in UL TCI state is not in the DL active TCI list

- To be confirmed:

  - [Do not define requirements for the case if source RS in UL TCI state is not in DL active TCI state list]

Issue 1-2-1 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

- Proposals

  - Proposal 1(Apple, Huawei, Samsung):

    - When PL-RS in UL TCI state switch is SSB in FR2, longer delay is expected.

    - If no consensus can be achieved in RAN4, no requirements are defined for this case.

  - Proposal 2(MTK,vivo):

    - Reuse the existing delay requirement of MAC CE based UL TCI state switch.

  - Proposal 3(ZTE):

    - Prefer to reuse the existing delay requirement of MAC CE based UL TCI state switch. However to move forward, a compromised solution is needed, e.g. allowing a clear but not too long additional latency.

  - Proposal 4(Nokia):

    - There is no need for beam sweeping for PL-RS measurements in FR2 if the PL-RS is SSB (assuming UE is having no more than 4 different PL-RS).

    - RAN4 does not discuss UE requirements for the scenario where the UE is configured with more than 4 different PL-RS for all active UL (or joint) TCI states.

    - There is no need for any additional measurements or beam sweeping at UL TCI state switch if the associated PL-RS is maintained by the UE.

    - The number of samples will not always be fixed as 5 samples when PL-RS is not maintained.

    - RAN4 to add [] around the number of samples needed if NM=1.

Issue 1-3-1 Definition of maintained PL-RS

- Agreement

  - Further discuss the detail wording.

Issue 1-4-1 Update condition of common TCI state update based on shared RS

- Agreement

  - Update the condition when DL/ UL unified TCI state switch requirements apply to common TCI – when all CCs in the list are configured with unifiedTCI-StateRef-r17 to the same serving cell and BWP

Issue 1-5-1 TCI state list update delay if all target TCI states are unknown

- Agreement

  - Remove the square bracket in previous agreement. If all target TCI states in the active TCI state list are unknown, no requirements are applicable.

Issue 1-6-1: When SSB for BFD/CBD/RLM is overlapped with SSB for L1-RSRP of another cell

- Agreement

  - Define Measurement restriction.

Issue 1-7-1 Whether need clarification or update in RAN4 spec when PDCCH/PDSCH and SSB associated  different PCI are overlapped on the same RE

- To be confirmed:

- [If the TCI state of the PDSCH /PDCCH is associated to the SSB of the cell with different PCI,

-	UE is not expected to receive PDSCH/PDCCH on the symbols corresponding to the SSB indexes configured for L1-RSRP measurement on the serving cell].