3GPP TSG-RAN WG4 Meeting # 106															R4-2303148

Athens, Greece, 27 February –03 March, 2023

Agenda item:			4.4

Source:	Apple

Title:	WF on MRTD/MTTD requirement applicability for EN-DC/NE-DC

Document for:	Approval

# Background

The MRTD/MTTD requuiremetn applicability was discussed based on [1-4] in RAN4#106 meeting. This contribution captures the WF for this issue.

# Way Forward

For TDD-TDD EN-DC with overlapping frequency, the MTTD/MRTD requirement for asynchronous operation in 7.5.2/7.6.2 is not applicable.

For Rel-15 UE supporting TDD-TDD EN-DC with overlapping frequency, only the MTTD/MRTD requirement for intra-band collocated case in 7.5.3/7.6.3 is applicable.

MTTD/MTRD requirements for synchronous operation in 7.5.2.1/7.5.5.1/7.6.2.1/7.6.5.1 are applicable for TDD inter-band EN-DC/NE-DC with overlapping frequency from R16.

Update the description for interBandMRDC-WithOverlapDL-Bands-r16 to differentiate FDD-FDD EN-DC/NE-DC combination and TDD-TDD EN-DC/NE-DC combination. Exact wording is FFS.

MRTD/MTTD Requirement applicability is clarified for Rel-15 UE supporting inter-band TDD-TDD EN-DC with overlapping frequency for TS 38.133 from Rel-15, and for Type-2 UE for TS 38.133 from Rel-16. CR will be presented for RAN4#106-bis meeting.

# Reference

[1]	R4-2300239, On MRTD/MTTD requirement applicability for EN-DC/NE-DC, Apple

[2]	R4-2302472, Clarification on MRTD/MTTD requirement in Rel-15, Apple

[3]	R4-2302473, Clarification on MRTD/MTTD requirement in Rel-16, Apple

[4]	R4-2300450, CR to TS38.133 on MTTD/MRTD requirements for non-collocated inter-band EN-DC/NE-DC with overlapping DL bands for Rel-16 Type-2 UE, Samsung