3GPP TSG-RAN WG4 Meeting #106	R4-2303486

Athens, GR, February 27th ‒ March 3rd, 2023

Title: 	WF on US 900 MHz bands

Agenda Item:	8.32.6

Source: 	Qualcomm Incorporated

Document for:	Approval

# Band plan

Way forward:

- Band number 106 and n106 are reserved for this band for LTE and NR, respectively

- Proposals from [2] are agreeable for the LTE band.

Proposal 1: To define the new LTE band as in table 2.1-1.

Proposal 2: For the new LTE band, it supports 1.4MHz, 3MHz and 5MHz channel bandwidth.

Proposal 3: For the new LTE band, to reuse the transmission bandwidth configuration of TS 36.101.

Proposal 4: for the new LTE band which support NB-IoT, to reuse the transmission bandwidth configuration of TS 36.101.

Proposal 5: For E-UTRA channel numbers, it is proposed to define as Table 2.3-1.

Proposal 6: For TX–RX frequency separation, it is proposed to define as Table 2.4-1.

In addition, the summary of the corrections on TS 36.101 are given in Table 2.5-1.

For proposal 2, the channel bandwidths for LTE (and also for NR) will be modified to include only 1.4 MHz and 3 MHz. See below.

# UE RF requirements

Way Forward:

- Channel Bandwidth for LTE and NR.  Propose to remove 5 MHz CBW requirement from both LTE and NR work items in upcoming plenary.  Focus will be on 3 MHz, 1.4 MHz and NB-IoT.  The current center channel for the 3 MHz CBW will be shifted to 899 MHz (UL) and 938 MHz (DL).

- Defer the discussion on 3 MHz NR until further progress is made in NR_FR1_lessthan_5MHz_BW.  Anyways, the work plan does not consider starting the NR band until Aug 2023.

- LTE reference sensitivity to reuse Band 8 value since the Band 8 duplexer is assumed to be reused.

- Out-of-band blocking frequency limits for this band are to reuse the Band 8 frequency limits.

- General or FCC SEM and additional spurious emissions assumed to be OK now with removal of 5 MHz CBW and center channel shift.

- With the removal of 5 MHz CBW from the WID and the move of the center channel for 3 MHz shifted to 899 MHz it is not expected that A-MPR (to be verified) will be necesssary and the use of the 3GPP general SEM or FCC SEM will provide enough protection.

- FFS to see if the -50 dBm/MHz coexistence requirement can be relaxed or removed. This includes imposing only the FCC emission requirement or general SEM, RB allocation restriction, or assuming improved UE performance.

# BS RF requirements

Way Forward:

- Proposed changes from [3] are agreeable for the LTE band.

Expected changes to 36.104 are itemized.

Clause 6.6.3.1	Minimum requirements for Wide Area BS (Category A):

Clause 6.6.4.3	Additional spurious emissions requirements

Clause 6.6.4.4	Co-location with other base stations

Clause 7.6.1	General blocking requirement

Clause 7.6.2	Co-location with other base stations

# Reference

- R4-2302812, “Topic summary for [106][119] LTE_NR_US_900MHz,” Moderator (Qualcomm Incorporated)

- R4-2301225, “Discussion on band definition for 900 MHz LTE Band,” ZTE Corporation

- R4-2301196, “BS requirements for 900 MHz LTE Band in the US,” Nokia, Nokia Shanghai Bell