3GPP TSG RAN WG4 Meeting #106		 R4-2303508

# 27 February - 3 March 2023, in Athens, Greece

Agenda Item:	9.22.2.1

Source: 	MediaTek Inc., [Qualcomm Incorporated]

Title: 	WF on ambiguity issue when two Tx chains are switched between different band pairs

Document for:	Discussion

1. Background

RAN4 Agreements captured in the LS R4-2220548 (RAN4 #105):

For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, and it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}, RAN4 agreed that:

- As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}.

Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively.

Pros: Ease of implementation

Cons: Worst transmission throughput performance

It is agreed by RAN4 on top of default behavior, figure out an approach for advanced UE be capable to transmit on the earlier done switching Tx chain if the ambiguity issue can be solved without or minimum additional signaling overhead.

If the switching configuration can be represented in order of switching bands for example, a band pair “Band B+A” is configured, this implicitly indicate Band B is at Tx chain #1, Band A is at Tx chain #2. Under such representation, if Tx switching is configured from “B+A” to “C+D”, it means exactly that in Tx chain #1, Band B uplink is switched to Band C, and in Tx chain #2, Band A is switched to Band D. The ambiguity is resolved without any additional signaling overhead with such representation. Blanking max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C} would not be required.

2. Agreement

- RAN4 maintains the baseline assumption in Issue 1-2-3 agreed in R4-2220546 during RAN4#105.

- Neither of the two Tx chains is expected to be used for transmission during the overlapping and non-overlapping switching periods in Rel-18.

- Enhancements for Rel-18 Tx switching

  - Resolving the switching pattern ambiguity issue if it is determined that it is possible that concurrent switching occurs with switching periods for two band pairs:

    - Example solution: The order of switching scheduling either via downlink control information (DCI) or RRC commands represents the mapping of Tx switching bands. An example is illustrated below, where, a scheduling of band A and band C, band B and band D is done in such manner that it implies switching from “A+B” to “C+D”, that in this way, switching pattern ambiguity is resolved without any additional signaling overhead.

    - By resolving the switching pattern ambiguity issue, the switching period can be the  switching capability of switched band pairs.

  - When two overlapping switching periods occur, union of switching periods is extended by the length of the shorter switching period among the switching band pairs based on ue capability

    - If UE capability is reported, extension is allowed

      - Details to be discussed in the next meeting

    - If not reported as a baseline, the union of switching periods applies

    - Impacts on network requires for further study

  - Note: This means new capability is created for this purpose

  - FFS When switching occurs between one band pair and UE has scheduled grants for a third band that is unaffected by the switching, switching period can be double the length of the declared value for that band pair

- RAN4 send an LS to RAN1/2 for the enhancements

Reference:

- R4-2220548 LS on Rel-18 UL Tx switching, RAN4#105

- R4-2302833	Topic summary for [106][140] NR_MC_enh_UERF, RAN4#106

- R4-2301722_Draft LS on Rel-18 UL Tx switching, RAN4#106

- R4-2301713 Discussion on multi-carrier enhancement for single-TAG Tx switching, RAN4#106