3GPP TSG-RAN4 Meeting #106	R4-2303509

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5912 | rev | 1 | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTE interband 2UL CA co-ex simplication R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has discussed to apply intersection rule for 2 UL interband CA UEtoUE co-ex table including LTE specification. Non-3GPP RATs are not to be changed as those do not generally follow intersection rule. Reason for this action is to reduce unnecessary testing of a UE as single carrier requirements guarantee performance also in 2 UL case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Following the intersection rule redundant requriements are removed - After removal of requirements unnecessary notes are removed - Error is specification is fixed Band 26 Frequency range - Notes that are void are removed from table. - CA_3A-11ACA_3-11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary requirements remain and need to be double tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of changes ************************************

#### 6.6.3.2A	Spurious emission band UE co-existence for CA

This clause specifies the requirements for inter-band carrier aggregation configurations with the uplink assigned to two E-UTRA bands for coexistence with protected bands. When both constituent bands have common coexistence band protection requirements as specified in clause 6.6.3.2, the requirements are also applied to the carrier aggregation configuration

As exceptions, the additional requirements in Table 6.6.3.2A-0 apply on each component carrier with all component carriers are active.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3.2A-0: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_1-5 |  |  |  |  |  |  |  |  |
|  | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_1-8 |  |  |  |  |  |  |  |  |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3, 11 |
| CA_1-11 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1-18 |  |  |  |  |  |  |  |  |
|  | Frequency range | 758 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1-19 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1-20 |  |  |  |  |  |  |  |  |
|  | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_1-21 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1-26 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  |  | 799 |  | - | 803 | -40 | 1 | 3 |
| CA_1-28 |  |  |  |  |  |  |  |  |
|  | Frequency range | 470 |  | - | 694 | -42 | 8 | 3, 22 |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 |  | - | 694 | -26.2 | 6 | 3 |
| CA_2-5 |  |  |  |  |  |  |  |  |
|  | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_2-13 |  |  |  |  |  |  |  |  |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_2-14 |  |  |  |  |  |  |  |  |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_3-5 |  |  |  |  |  |  |  |  |
|  | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_3-8 |  |  |  |  |  |  |  |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 11 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3,11,17 |
| CA_3-11 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-18 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-19 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-20 |  |  |  |  |  |  |  |  |
|  | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_3-21 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-26 |  |  |  |  |  |  |  |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  |  | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_3-28 |  |  |  |  |  |  |  |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_3-40 |  |  |  |  |  |  |  |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 |  |
| CA_3-41 |  |  |  |  |  |  |  |  |
|  | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 4, 18 |
| CA_3-42 |  |  |  |  |  |  |  |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_4-5 |  |  |  |  |  |  |  |  |
|  | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_4-13 |  |  |  |  |  |  |  |  |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_4-28 |  |  |  |  |  |  |  |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  |  | 803 | -50 | 1 |  |
| CA_5-7 |  |  |  |  |  |  |  |  |
|  | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_5-12 |  |  |  |  |  |  |  |  |
|  | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_5-17 |  |  |  |  |  |  |  |  |
|  | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_5-40 |  |  |  |  |  |  |  |  |
|  | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_7-26 |  |  |  |  |  |  |  |  |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 |  |
| CA_7-28 |  |  |  |  |  |  |  |  |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 11 |
| CA_11-18 |  |  |  |  |  |  |  |  |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_11-26 |  |  |  |  |  |  |  |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_13-66 |  |  |  |  |  |  |  |  |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_14-30 |  |  |  |  |  |  |  |  |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_14-66 |  |  |  |  |  |  |  |  |
|  | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_18-28 |  |  |  |  |  |  |  |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_19-21 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_19-42 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_21-28 |  |  |  |  |  |  |  |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_21-42 |  |  |  |  |  |  |  |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_26-46 |  |  |  |  |  |  |  |  |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_26-48 |  |  |  |  |  |  |  |  |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_28-41 |  |  |  |  |  |  |  |  |
|  | Frequency range | 470 | - |  | 694 | -42 | 8 | 3, 22 |
|  | Frequency range | 470 | - |  | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 662 | - |  | 694 | -26.2 | 6 | 3 |
|  | Frequency range | 758 | - |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 | - |  | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - |  | 1915.7 | -41 | 0.3 | 4, 5, 18 |
| CA_28-42 |  |  |  |  |  |  |  |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_40-42 |  |  |  |  |  |  |  |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 8 |
| CA_41-42 |  |  |  |  |  |  |  |  |
|  | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 4, 18 |
| NOTE 1: Void NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE10: Void NOTE 11: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 12: Void NOTE13: Void NOTE 14: Void NOTE 15: Void NOTE 16: Void NOTE 17: This requirement is applicable only when Band 3 transmission frequency is less than or equal to 1765 MHz. NOTE 18: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 19: Void NOTE 20: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 21: Void NOTE 22: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 23: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48.NOTE 24: Void NOTE 25: Void |  |  |  |  |  |  |  |  |

***************************** End of changes ************************************