3GPP TSG-RAN WG4 Meeting #106                                          R4-2303519

Athens, Greece, February 27 – March 3, 2023

Title:	[Draft] LS on the UE SRS IL imbalance issue

Response to:	-

Release:	Rel-18

Work Item:	NR_ENDC_ RF_FR1_enh2

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:	TSG RAN WG2

Contact person:

- Name:	Xiang Gao

- E-mail Address:	gaoxiang74@huawei.com

Attachments: None

- 1		Overall description

An issue that insertion loss (IL) for the diversity branch might be different from the main branch has been identified by RAN4, and the IL imbalances across branches are called SRS IL imbalance. For better explanation, the following example is provided.

Fig 1. Example of possible RF architecture between ‘t1r4’ (left) and ‘t1r8’ (right) AS-SRS capable UE

As depicted in the above figure, LPAF is the module includes LNA/PA/Filter/switch and is same for different antennas, DRX-M is the diversity receive module which includes the LNA and filters. Apart from the different RF switch (e.g. SPDT/SP4T) that could be applied for different branch, the trace loss, which is due to antenna location, can also contribute to the SRS insertion loss imbalance between the main branch (LPAF branch) and diversity branch (DRX-M branch).

As a result, RAN4 has defined a relaxation for 4Rx as a non-ideal factor in PCMAX_L, f, c, so that the transmission power of any SRS resource from diversity branch would be allowed for a certain reduction. Additionally, such relaxation could be larger in order to deal with 8Rx RF architecture that would increase more implementation complexity, but the discussion on the value is still ongoing. The following definition is excerpted from TS 38.101-1.

Considering that such imperfection could lead to inaccurate channel estimation at receiver, which could lead to incorrect PMI selection that would degrade overall system performance, RAN4 think it is necessary to address such IL imbalance issue. Consequently, RAN4 would like to ask RAN1 to evaluate possible solution for it.

Besides, whether the reporting is static and/or dynamic is up to RAN1 decision, however, it is recognized in RAN4 that UE may change the physical Tx antenna mapping of each antenna port in some conditions for example due to hand blocking issue.

For RAN1 information, some possible resolutions to mitigate the impact of the SRS IL imbalance are provided in Annex based on the current discussion in RAN4.

- 2		Actions

To RAN1

ACTION: 	RAN4 respectfully ask RAN1 to consider above issue with, but not limited to the three resolutions listed in the Annex for their future study.

- 3		Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #106bis-e	  April 17 – April 21, 2023		         Electronic meeting

TSG RAN WG4 Meeting #107	     May 22 – May 26, 2023		         Incheon, Korean

- 4		Annex for information

- Enable UE report on the actual IL imbalance for each diversity branch used for SRS so network can use this information for AS-SRS based channel estimation accuracy improvement where the granularity of such report can be per SRS resource and either static or dynamic.

- Utilize PCMAX, f, c via PHR type 3, where power imbalances across antenna ports are derived by comparing the maximum configured power of main branch to those of diversity branches, where such report is per transmission occasion per SRS (only a report of a single SRS resource is transmitted per transmission occasion).

- Define UE measurements of downlink channels which are reported in order to assist the network in determining the difference between the UE insertion losses for two given antenna ports, where the network also does its own measurements of SRS channels.

- RAN4 does not preclude other options.

The above alternatives are considered from Release 18 onwards and for 8Rx capable UE’s, and possible applicability UE’s supporting 2RX or 4RX is FFS.