# 3GPP TSG-RAN WG4 Meeting #106	R4-2303546

Athens, Greece, February 27 – March 3, 2023

Source: 	vivo, Huawei, HiSilicon, Skyworks, ZTE, Qualcomm, Xiaomi

Title: 	TP for TR 38.872 to complete the open issues for CA_n8-n20-n28

Agenda Item:	9.2.5

Document for:	Approval

Introduction

In this contribution, ∆TIB,c and ∆RIB,c values and 2UL IMD3 MSDs for CA_n8-n20-n28 are provided in this TP considering 2-antenna architecture and 3-antenna architecture. This TP captures all the evaluation results from companies to complete the open issues for CA_n8-n20-n28 in the study item phase.

# References

[1] R4-2218418, TR 38.872 v0.3.0, CATT

[2] R4-2302817, Topic summary for [106][124] FS_NR_700800900, Moderator(CATT)

[3] R4-2300654, 2UL IMD3 MSDs for CA_n8-n20-n28, Skyworks

[4] R4-2302099, Discussion on MSD evaluation and deltaTibRib for CA_n8-n20-n28, Huawei, HiSilicon

[5] R4-2302100, TP for TR 38.872 to capture MSD evaluation and deltaTibRib for CA_n8-n20-n28, Huawei, HiSilicon

# Text proposal

## <<Start of Change for TR 38.872>>

#### 5.3.2.4	∆TIB,c and ∆RIB,c values

For CA_n8-n20, the TIB,c and RIB,c values have been specified in TS 38.101-1.

## <<Next of Change>>

#### 5.3.3.4	∆TIB,c and ∆RIB,c values

For CA_n8-n28, the TIB,c and RIB,c values have been specified in TS 38.101-1.

## <<Next of Change>>

#### 5.3.5.3	∆TIB,c and ∆RIB,c values

For CA_n8-n20-n28, the TIB,c and RIB,c values for 2-antenna and 3-antenna architectures are captured in this part. However, only one set of values will be defined in the specification in the follow-up WI allowing all possible UE RF architectures.

For CA_n8-n20-n28, the TIB,c and RIB,c values are derived from the existing CA_n8-n28 and CA_n20-n28 based on 3-antennas UE architecture as the baseline .

Table 5.3.5.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.6 |
|  | n20 | 0.5 |
|  | n28 | 0.5 |

Table 5.3.5.3-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.2 |
|  | n20 | 0 |
|  | n28 | 0.1 |

For CA_n8-n20-n28, the ΔTIB,c and ΔRIB,c values are derived in Table 5.3.5.3-3 and Table 5.3.5.3-4 and enable a two-antenna architecture in the long-term.

Table 5.3.5.3-3: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.8 |
|  | n20 | 0.7 |
|  | n28 | 0.7 |

Table 5.3.5.3-4: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.3 |
|  | n20 | 0.2 |
|  | n28 | 0.2 |

## <<Next of Change>>

#### 5.3.5.5	REFSENS evaluation

The MSD test configurations can be evaluated as below for band n8 MSD due to IMD3 of Tx band n20 + band n28 and band n28 MSD due to IMD3 of Tx band n8 + band n20.

Table 5.3.5.5-1: REFSENS exceptions due to IMD interference for CA_n8-n20-n28

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n20-n28 | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | FDD | N/A |
|  | n8 | 908.5 | 5 | 25 | 953.5 | 23.5 | FDD | IMD3 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 726.5 | 5 | 25 | 781.5 | 23.0 | FDD | IMD3 |

CA_n8-n20-n28 2UL IMD3 MSDs are derived in Table 5.3.5.5-2 and enable a two-antenna architecture in the long-term.

Table 5.3.5.5-2: 3DL/2UL IMD3 MSDs for CA_n8-n20-n28

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n8-n20-n28 | n8 | N/A | 5 | 25 | 951.5 | [25] | FDD | IMD3 |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 717.5 | 5 | 25 | 772.5 | N/A | FDD | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | 25 | 781.5 | [25] | FDD | IMD3 |

## <<End of Change>>