3GPP TSG-RAN WG4 Meeting #106	R4-2303552

Athens, GR, February 27th ‒ March 3rd, 2023

Title: 	Ad-hoc minutes for RF requirements for 5G broadcast

Agenda Item:	10.3.5

Source: 	Qualcomm Incorporated

Document for:	Approval

# Introduction

This document contains meeting minutes for an ad-hoc meeting held on the evening of March 2, 2023.

# Band plan

From [1] two band plan options not mutually exclusive were identified

- Define a single band covering the entire UHF frequency range 470 – [698/702] MHz

- The requirements are relaxed for this band (TBD which requirements and how much) and/or

- This band is assumed to be implemented with more than one filter in the UE (TBD how many filters and the characteristics of those filters)

- Define smaller bands

- At least one of the bands overlaps with Band n105 DL to reuse n105 Rx filter

- Specify the following bands 470-542 MHz, 540-606 MHz and 602-702 MHz.

- Other

Agreement: (Previous agreement as captured in RAN4 chairman’s meeting minutes)

- Define the band plan based on available data for BS and UE.

  - Full UHF band, i.e., 470 – 698MHz,

    - with reduced UE blocking (for information: no or limited protection from IMT adjacent bands).

  - Narrow band, i.e., 612 – 652 MHz, which is the same as the downlink operating band of n105 (for information:  protection from IMT bands).

  - If data for other frequency ranges becomes available before the conclusion of the work item, it can be considered either as a separate band or as a modification to initially defined bands.

AH Chair:  The agreement was copied directly from RAN4 chairman’s minutes

Ericsson:  Concerned about blocking performance but understand the available data.  Therefore would like to add the clarification.

R&S:  What does “protection from IMT bands” mean?

Ericsson:  UE blocking

R&S:  What is the consequence of this statement?

Ericsson:  It’s just for information

SWR:  Reluctant to add this clarification.  Won’t know what it means in the future.

AH Chair:  Would Ericsson be able to accept not adding the clarification?

Ericsson:  No.

Huawei:  Tend to agree with Ericsson

AH Chair:  This is just for the band plan.  The requirements will be developed for each band later.

Ericsson:  Still have concern with stating “with reduced UE blocking” only without further clarification

Apple:  Does reduced blocking indicate in-band blocking, out-of-band blocking, or both?

Nokia:  What does in-band blocking have to do with the band plan?

Apple:  At least out-of-band, but not sure about in-band.

AH Chair:  Further discussion needed if there will be modification of the note.  Otherwise, the agreement captured in the RAN4 chairman’s meeting minutes still stands.

# Reference sensitivity

Tentative way Forward:

- The values of -99.2, -98.5, and -97.9 dBm are agreed for 6, 7, and 8 MHz PMCH channel assuming 10 MHz UE receive channel filter.

- FFS on whether all bandwidths are needed to be tested.

Apple: The refsens depends on the UE channel filter bandwidth, not the signal bandwidth itself

Qualcomm: 10 MHz of noise, 6 MHz of signal, but the FFT only demods the useful portion

Apple: We will have 10 MHz of noise.  Refsens calculations refer to the noise bandwidth. Do you want to change the test?

Qualcomm: We need 3 new reference channels corresponding to 6, 7, and 8 MHz.  Yes, we have 10 MHz of noise, but the FFT will discard the outer tones.

MTK: Even before the FFT, we have a sharper digital filter.

Samsung: Same view as Apple. Depends on channel bandwidth. Refsens corresponds to SNR = -1 within channel bandwidth.  kTB noise cannot be eliminated.

Apple: Analog and digital filter is the main effect.

Qualcomm: Only the noise in the tone that you are demodulating is relevant. SNR=-1 dB.  Assume we only test the refsens only 1 RB in 10 MHz?  What noise power applies?

Apple: We don’t believe the FFT can provide the rejection of a digital filter.  The main impact is the analog and digital filter.

SWR: It cannot be the case that it depends on the digital filter.  It depends on the noise in the effective bandwidth and nothing else.

Apple: The digital filter is programmed for the channel bandwidth. It is not a fixed filter.

R&S: There are different understanding between companies, so agreement doesn’t seem possible.

Apple: Our proposal is 10 MHz refsens value with 10 MHz PDSCH

Nokia: PDSCH may make sense if the UE also supports LTE.  But if it is only a broadcast receiver without LTE, it would not make sense to define with 10 MHz PDSCH.

Qualcomm: If we have 10 MHz PDSCH, then the UE would implement it only for this test because in practice, it would never be used for broadcast.

Apple: According to the WID, reuse existing requirements for 10 MHz as much as possible

R&S: Study the options and find a compromise

To resolve

Which bandwidth to use?  6, 7, 8, or 10?

What channel?  PDSCH or PMCH?

# ACS

Tentative way Forward:

- Channel raster for ACS is 6, 7, or 8 MHz

- ACS is specified as [-16] dB for 6 MHz channel with the assumption of 10 MHz UE channel filter and coordinated network deployment.

Apple: Agree that blocker inside the UE filter will lead to degradation.  Should we just use a single value, or separate values for 6, 7, and 8 MHz?  Concern about the workload if multiple offsets are specified.

Qualcomm: We are willing to do the work

Apple: Would like to see values for all bandwidths

Qualcomm: ACS values for 6, 7, and 8 MHz were provided in our paper.

SWR:  The -16 dB value is for 6 MHz channel and 15 kHz SCS?  The -16 dB would not be deployed.

Qualcomm: Yes. Our expectation is ACS will be better for wider channel and for narrower SCS. This was the worst case.

SWR:  Can we indicate a range [-16 to -33]?

- Blocker placement for ACS is 6, 7, or 8 MHz

- ACS is specified as [-16 to -33] dB for 6, 7, and 8 MHz channel with the assumption of 10 MHz UE channel filter and coordinated network deployment.

Apple: Would like to check.  We would like to consider that ACS might be worse than -16.

MTK: We would like to check also.

Nokia: Should use SCS used for broadcast. 1.25 kHz.

Chair: Should we specify to reuse LTE 10 MHz as much as possible?  Or specify to more closely align with the 5G broadcast?

SWR: Align to the 5G broadcast

AH Chair:  Agreement from AH meeting

- Blocker placement for ACS is 6, 7, or 8 MHz

- ACS is specified as [-16 to -33] dB for 6, 7, and 8 MHz channel with the assumption of 10 MHz UE channel filter and coordinated network deployment.

# Reference

- R4-2220501, “WF on UE RF for 5G Broadcast,” Qualcomm Incorporated