3GPP TSG-RAN WG4 Meeting #106	R4-2303571

# Athens, Greece, 27 February– 3 March, 2023

Source:	Huawei, HiSilicon

Title:	TP for 37.718-00-00 to introduce SUL configuration CA_n8A_SUL_n78A-n81A

Agenda item:	8.13.2

Document for:	Approval

1 Background

This contribution provides text proposal on the NR SUL band combination CA_n8A_SUL_n78A-n81A.

2 Text Proposal

---Start of changes---

## 5.X	CA_n8_SUL_n78-n81

### 5.X.1	Operating bands

Table 5.X.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n8_SUL_n78-n81 | n8, n78, n81 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.X.2	Channel bandwidths per operating band

Table 5.X.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A_SUL_n78A-n81A | SUL_n78A-n81A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n81 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

### 5.X.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.X.4	Spurious emission band UE co-existence

There is only single UL in uplink so the requirement for each band in clause 6.5.3.2 from 38.101-1 is applicable.

### 5.X.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.X.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n8 | n811 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |  |
| n78 | n81 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. This restriction of REFSENS configurations applies also for these carriers when applicable SUL configuration is part of a higher order configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Since the 4th harmonic interference of SUL band n81 may fall into the Rx band n78, the MSD due to harmonic interference should be took care of. The REFSENS requirements can refer to the MSD due to harmonic interference between SUL band n81 and n78 which has been specified in Table 7.3C.2-2, if harmonic interference need to be considered.

### 5.X.6	∆TIB and ∆RIB values

For CA_n8_SUL_n78-n81, the TIB,c and RIB,c values are given in the tables below referring to CA_n8-n78.

Table 5.X.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n8_SUL_n78-n81 | 0.6 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 5.X.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n8_SUL_n78-n81 | 0.2 | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

---End of changes---

Reference

[1]	RP-223537, “New WID: Rel-18 Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)”, Huawei, HiSilicon